3GPP TSG-RAN WG2 Meeting #127	 R2-2406325

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.6

Source: 	CATT

Title: 	Conclusion on remaining issues on LTE TN to NR NTN IDLE mode Mobility

Document for: 	Discussion and Decision

Introduction

This contribution addresses the remaining issues for LTE TN to NR NTN IDLE mode mobility as recorded in [0].

Discussion

- [Issue 1]	Down selection of signalling options for NR satellite assistance information

It was agreed to discuss the signaling design for NR satellite assistance information from the following 3 signaling options [1].

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

In this meeting we provide the separate draft CRs in [2][3][4] for the three signaling options. RAN2 should discuss and select one of the options to adopt for the provision of the NR satellite assistance information, and take the corresponding draft CR as the baseline for further CR discussion.

During the running CR drafting, we find that if option 1 is adopted, perhaps some eMTC specific impacts are needed to define the applicability of the new SIBXY (e.g. in procedure in 5.2.1.3). This seems to be not fully aligned with the following agreement that we made at the last meeting: [1]

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

So this may need to be taken into account by companies when we make the down selection among options.

Observation 1: Signaling Option 1 seems to need some changes specific to eMTC in order to specify the applicability of the new SIBXY introduced.

Proposal 1: RAN2 selects one of the 3 signaling options taking into account the corresponding draft CRs for each option provided in [2][3][4].

- [Issue 2] Whether ntn-PolarizationDL is mandatory or optional

The field of ntn-PolarizationalDL in NTN-Config defined in 38.331 is optional as follows [5]:

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                  EpochTime-r17                                                            OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}              OPTIONAL,  -- Cond SIB19

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

kmac-r17                       INTEGER(1..512)                                                          OPTIONAL,  -- Need R

ta-Info-r17                    TA-Info-r17                                                              OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ephemerisInfo-r17              EphemerisInfo-r17                                                        OPTIONAL,  -- Need R

ta-Report-r17                  ENUMERATED {enabled}                                                     OPTIONAL,  -- Need R

...

}

It indicates polarization information for downlink transmission on the service link. If it is not present, UE may need more time to search the SSB of the satellite, but it does not mean the UE cannot detect the SSB. So we think the ntn-PolarizationalDL defined in 36.331 can also be optional as how it is defined in 38.331.

Proposal 2: The ntn-PolarizationalDL is optional.

- [Issue 3] Whether the SatelliteId-r18 is reused as NR satellite ID

It was agreed to use satellite ID to identify NR satellite assistance information at the last meeting [1]. In Rel-18 IoT NTN, the IE SatelliteId-r18 is defined and used by SIB31 and SIB33 for the identification of satellite using EUTRA. Whether the existing IE SatelliteId-r18 can also be reused to identify NR satellite or it is needed to define NR specific satellite ID (e.g. SatelliteId-NR-r19) should be discussed.

Currently, the range of the SatelliteId-r18 is 0-255, that is, the maximum number of the satellites the IE could identify is 256 [6].

SatelliteId-r18 ::= INTEGER (0..255)

As for whether the SatelliteId-r18 can be reused for NR satellite, one issue is whether the range of the SatelliteId-r18 is sufficient. If it only takes the serving/neighbor cell into consideration, we think the SatelliteId-r18 is enough to identify any satellite, no matter it uses NR or EUTRA access technology. If the SatelliteId-r18 is not only used to identify the satellites providing neighbor cell/serving cell coverage, but also used to uniquely identify one satellite in any orbit, whether the value ranges of the current SatelliteId-r18 is enough for both NR satellites and IoT NTN satellites is questionable.

RAN2 should discuss whether to define NR specific satellite ID or reuse current satellite ID SatelliteId-r18 for NR satellite identification.

Proposal 3: RAN2 down-selects following alternatives to identify the NR satellite assistance information:

- Alternative 1: Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite;

- Alternative 2: Define NR specific satellite ID, e.g. SatelliteId-NR-r19, to identify NR satellite specifically.

- [Issue 4] What is the maximum number of NR satellites configurable

In the signaling Option 1 [2] and Option 2 [3], separate list is introduced for NR satellite assistance information, apart from the IoT NTN satellite assistance information. So the maximum number of the NR satellite should be defined. According to the current spec, the maximum number of neighbor satellite for IoT NTN is 4. Considering the TN cell coverage is much smaller than NTN cell coverage, the number of satellites which could provide coverage for one TN cell is not likely to be more than the number of satellites which could provide coverage for one NTN cell; so the number of NR satellite does not necessarily exceed 4. So we suggest, if signaling option1 or option 2 is adopted, the maximum number of NR satellites is 4 as defined for IoT NTN satellites using EUTRA.

Proposal 4: If signaling Option1 [2] or Option 2 [3] is adopted, maxSat-r17 (4) is reused for the maximum number of NR satellites.

- [Issue 5] Confirmation of Working Assumption for SMTC

In the previous meeting, the following working assumption for SMTC configuration was made: [7]

Working Assumption:

- We don’t introduce multiple SMTCs in LTE

As discussed in previous meeting, multiple SMTCs are not needed, considering the small coverage of TN cell and considering that the SMTC is set based on the assumption of the SSB propagation delay between UE and gNB equals to 0 ms (which is similar as TN cell), so multiple SMTCs are not needed. It seems no reason to revise this work assumption, so it should be confirmed as formal agreement.

Proposal 5: RAN2 confirms the working assumption "We don’t introduce multiple SMTCs in LTE".

- [Issue 6] Confirmation of Working Assumption on cell (re)selection procedure

The following working assumption needs to be confirmed [1]:

Working Assumption:

NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

Currently for the cell reselection procedure, no matter it is between TN cells, between NTN cells or between TN cell and NTN cell, only RRM measurement is used for cell reselection evaluation. The same mechanism should be inherited for cell reselection from LTE TN to NR NTN. No strong reason to enhance this mechanism. From this point of view, no spec impact on TS 36.304 is foreseen. This working assumption can then be confirmed as well.

Proposal 6: RAN2 confirms below working assumption and confirms no Spec impact to TS 36.304:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

Conclusion

In this contribution, the open issues on idle mode mobility from EUTRA TN to NR NTN are discussed with the following observations and proposals:

Observation 1: Signaling Option 1 seems to need some changes specific to eMTC in order to specify the applicability of the new SIBXY introduced.

Proposal 1: RAN2 selects one of the 3 signaling options taking into account the corresponding draft CRs for each option provided in [2][3][4].

Proposal 2: The ntn-PolarizationalDL is optional.

Proposal 3: RAN2 down-selects following alternatives to identify the NR satellite assistance information:

- Alternative 1: Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite;

- Alternative 2: Define NR specific satellite ID, e.g. SatelliteId-NR-r19, to identify NR satellite specifically.

Proposal 4: If signaling Option1 [2] or Option 2 [3] is adopted, maxSat-r17 (4) is reused for the maximum number of NR satellites.

Proposal 5: RAN2 confirms the working assumption "We don’t introduce multiple SMTCs in LTE".

Proposal 6: RAN2 confirms below working assumption and confirms no Spec impact to TS 36.304:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

Reference

- R2-2406318	Open issue list for LTE_TN_NR_NTN_mob WI

- R2-2405703	Report from Break-out session on NR-NTN and IoT-NTN

- R2-2406319, Introduction of LTE TN to NR NTN IDLE mode mobility (Option 1)

- R2-2406320, Introduction of LTE TN to NR NTN IDLE mode mobility (Option 2)

- R2-2406321, Introduction of LTE TN to NR NTN IDLE mode mobility (Option 3)

- TS 38.331, V18.2.0

- TS 36.331, V18.2.0

- R2-2403733	Report from Break-out session on NR-NTN and IoT-NTN