3GPP TSG-RAN WG2 Meeting #127	R2-2406326

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on support of store and forward operation

Document for: 	Discussion and Decision

Introduction

In RAN2#126 [1], the following agreements were achieved:

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this contribution, the support of store and forward operation will be further discussed.

Discussion

2.1	 Output from SA2

Based on the TR 23700-29 [2] from SA2, two kinds of network architectures are agreed for supporting Store and Forward operation, which are split MME architecture and full CN on-board architecture.

## 8.2 Conclusion for KI #2: Support of Store and Forward Satellite operation [...] The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles: [...] The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: [...]

- For split MME architecture

Partial MME is on-board. Under this architecture, attach/TAU procedure cannot be completed during the coverage of one satellite during one round of RRC connection. The MME can optionally provide a list of satellite IDs over which the UE may resume the Attach/TAU procedure by the RRC connections established with a series of subsequent satellites, after the procedure was initiated.

- For full CN on-board architecture

The whole CN including MME, SGW, PGW, HSS, E-SMLC, SMSC, etc. are on board for each satellite. The UE attaches, transfers data and detaches from each satellite as required. Optionally the MME provides the UE with a list of satellites IDs, during attach/TAU. The UE uses the satellites in the list for MO/MT data/signalling transfer with the CN.

For both of these two architectures, MME can provide a list of Satellite IDs to UE for monitoring or accessing. The signalling format of the Satellite ID aligns with the satellite ID indicated in the system information.

Observation 1: For both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs for UE to monitor and access, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

Additionally, SA2 agreed that the network shall be able to inform UE(s) of whether S&F operation is applied. How the NW informs UE depends on RAN.

Observation 2: For both split MME architecture and full CN on-board architecture, SA2 agreed that the NW shall be able to inform UE of whether S&F operation is applied. How to inform UE depends on RAN.

Based on the output of SA2, for both split MME architecture and full CN on-board architecture, there are some potential impacts on RAN2, including what information the eNB provides in SIB with respect to the indicated satellite IDs by MME, how to inform UE of whether S&F operation is applied and what the corresponding UE behaviours should be after receiving these information. RAN2 need further investigate the impacts on RAN2 in these aspects.

2.2	 Impacts on RAN

Issue#1: Informing UE of whether S&F operation is applied

In RAN2#126, it was agreed that S&F indication can be provided by SIB. In detail, RAN2 needs to consider how to indicate UEs whether S&F is applied by the NW and the intended UE behaviour. Besides, the NW working with S&F operation should be able to prevent UE that does not support S&F (including pre-Rel-19 NTN capable UE, TN only capable UE and Rel-19 UE not supporting S&F feature) trying to access the cell. To achieve this purpose, legacy barring bits including cellBarred and cellBarred-NTN can be replied on via appropriate NW configuration. Additionally, a new barring bit, e.g. sf-cellBarred, can be introduced for S&F operation; the presence/absence of sf-cellBarred can inform Rel-19 UEs supporting S&F operations of whether S&F operation is applied. Also, the access control of the Rel-19 UEs supporting S&F operations is also performed by this new barring bit, when the NTN is operating in S&F mode.

Proposal 1: Introduce a new barring bit to inform UE of whether the S&F operation is applied and whether the satellite is barred for access of the UEs supporting S&F operation.

Based on that, there are following two options on how to configure these barring bits in a cell applying S&F operation and how UEs with different capabilities understand these barring bits:

- Option 1: cellBarred and cellBarred-NTN are set as "barred", sf-cellBarred is set as "barred" or "notBarred";

- Option 2: cellBarred is set as "barred", cellBarred-NTN is absent, sf-cellBarred is set as "barred" or "notBarred";

Accordingly, the behaviours of UEs with different capabilities are shown as in the following table.

|  | Option 1: cellBarred is set as barred cellBarred-NTN is set as barred sf-cellBarred is set as barred or not barred | Option 2: cellBarred is set as barred cellBarred-NTN is absent sf-cellBarred is set as barred or not barred |
| --- | --- | --- |
| Legacy NTN UE | UE ignores cellBarred, and considers this cell is an NTN cell which is barred according to cellBarred-NTN. | UE considers this cell is a TN cell which is barred according to cellBarred. |
| UE support S&F operation | UE ignores cellBarred and cellBarred-NTN. UE considers this cell is in S&F operation which is barred or notBarred according to sf-cellBarred. | UE ignores cellBarred. UE considers this cell is in S&F operation which is barred or notBarred according to sf-cellBarred. |

Under both of these two options, comparing with legacy NTN accessing, the cellBarred-NTN is used to bar the connectivity to NTN not applying S&F operation.

Proposal 2a: When a cell is applying S&F operation, it includes the S&F specific barring bit (e.g. sf-CellBarredNTN) and UEs applying S&F operation determines whether the cell is barred or not depending only on this barring bit (and ignoring existing cellBarred/cellBarred-NTN).

Proposal 2b: When a cell is applying S&F operation, the cellBarred-NTN is used to bar the connectivity to NTN from UEs not applying S&F operation.

Some companies argued at the last meeting that an S&F satellite may turn into a non-S&F satellite with legacy R17/18 IoT NTN operation, when it gets feeder link connection back in some occasions (e.g. flying back from ocean to land). To cover also this case, the NW can simply set this new sf-CellBarredNTN bit to absent, enabling all NTN UEs to rely on the existing cellBarred/cellBarred-NTN to perform legacy IoT NTN operations as in the legacy.

Proposal 2c: When a cell is not applying S&F operation (e.g. turning from S&F satellite to non-S&F satellite), NW via implementation sets the S&F specific barring bit (e.g. sf-CellBarredNTN) to absent (no Spec impact).

Issue#2: How to use assistance information for the indicated satellite ID from MME

Considering the use case of the S&F, it is highly likely that the satellites indicated by MME are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage). Obviously if the UE could start/stop idle mode tasks related to S&F upon the indicated satellite coming and leaving, it will bring UE power saving gain. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by MME, the UE need not perform idle mode tasks (e.g. cell (re)selection, paging monitoring, etc.) related to S&F operation. Only when it is in coverage of a target satellite indicated by MME, need the UE perform idle mode tasks related to S&F operation. How the UE determines whether it is in coverage or out of coverage of the indicated satellite(s) using satellite assistance information can be left up to UE implementation.

Proposal 3a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" status of a target satellite indicated by MME is up to UE implementation (via using satellite assistance info).

Proposal 3b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

The MME only provides Satellite IDs to UE for monitoring and accessing without the related satellite assistance information which should still be acquired from SIB broadcast by eNB. RAN2 needs to discuss what information the eNB should provide to UE for the indicated Satellite IDs.

Obviously, it would be helpful if eNB provides some information for UE to determine the serving time of these indicated satellites, so that the UE could determine when to start and/or stop searching these satellites to avoid unnecessary power consumption. In Rel-17, SIB32 was introduced for the prediction of discontinuous coverage, in which ephemeris info and footprint info are provided. As mentioned above, as the satellites indicated by the MME to provide the subsequent service is probably not the next satellite which provides the same coverage as the current satellite, the satellite assistance information needed for S&F operation is logically similar with that for discontinuous coverage, so that similar configuration can be used.

SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } }

However, it is better to introduce a separate list specifically for S&F use, instead of reusing the existing list in SIB32 directly. That is because discontinuous coverage and S&F are corresponding to different functions, serving different purposes. For discontinuous coverage, the NW indicates the next satellite which may provide coverage for same area covered by the current cell. It does not distinguish which CN the next satellite belongs to. But for S&F, the satellite indicated by MME is the satellite which may provide service for the specific UE and is designated by the CN, and it may not be the next satellite which will provide coverage for the UE.

Besides, the UE behaviour for the operation towards these two kinds of satellites is different either. Whilst in coverage of any satellites in SIB32, the UE shall perform idle mode tasks related to NTN. However, for the satellites indicated by MME for the S&F purpose, the UE needs to search for the specific satellites. Only in coverage of these satellites, does the UE need to perform related tasks. If SIB32 is reused for S&F use, besides S&F operation, the UE may also perform discontinuous coverage mechanism, i.e., perform idle mode tasks related to NTN for all the satellites broadcast in SIB32, even if the satellite is not belongs to the Satellite ID list provided by MME. This will bring unnecessary power consumption.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

Furthermore, the maximum number of satellites for discontinuous coverage in SIB32 is 4. If it is directly reused to provide IDs of several satellites not limited to the next hop, this maximum number may be too limited.

Hence, it is better to introduce a separate list to indicate the assistance information of the satellite(s) indicated by MME for S&F purpose. But the format of SatelliteInfo-r17 in SIB32 can be taken as the baseline for the signalling design.

Proposal 4a: Introduce a separate list to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the serving time of the satellites indicated by MME.

Proposal 4b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

Issue#3: How can the UE select the cells belonging to the satellites indicated by MME

For the satellites indicated by MME, there may be some stored DL signalling or data for the UE.

- For split MME architecture, the UE should camp on the cells served by these satellites to perform paging monitoring and initiate RRC connection setup/resume procedure etc.

- For full CN on-board architecture, the UE should perform attach procedure on the satellites indicated by MME, so that the satellite knows whether the UE is under coverage and determine whether to transmit DL signalling/data towards UE.

For both of these two architectures, a common issue is how the UE selects the cells belonging to the indicated satellites by the MME. Even when the UE initiates cell (re)selection measurement based on the above satellite assistance information, it is also possible that satellites other than the indicated satellites that provide coverage for the UE are detected. In this case, if the existing cell (re)selection mechanism is directly reused, UE may select to one cell served by a satellite not indicated by the MME to camp on and thus miss the MT data or signalling from the indicated satellite.

Observation 3: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected), in order to receive possible paging and/or initiate access to receive MT data and signalling.

RAN2 should consider some enhancements on cell (re-)selection with the consideration of the MME-indicated satellites introduced by SA2. For example, UE considers the frequencies served by the indicated satellites as highest priority. As another example, the UE should select the cell which belongs to the indicated satellite by MME to camp on, if detected.

Proposal 5: RAN2 needs to enhance cell (re)selection mechanism to ensure UE to (re-)select a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the cell which belongs to the indicated satellite to camp on, etc.).

Issue#4: RRC state of UE applying S&F operation

In RAN, we need to discuss the RRC state of UE during the end to end procedures (e.g. attach procedure) based on the S&F operation.

- For split MME architecture

During the attach procedure, the UE needs multiple rounds of connectivity establishment (with one or multiple satellites) towards the CN on ground. Since SA2 already agreed that some information (e.g., reject cause, wait timer and satellite IDs) will be indicated to UE by MME for continuing the intermittent attach procedure later, RAN could release UE into RRC_IDLE without additional indication. When performing next round of connection for attach, the RRC connection will be triggered by NAS layer of UE.

According to SA2's progress, both CIoT CP solution and CIoT UP solution can be supported for split MME architecture. After the attach procedure is finished, end to end connection is established. If CIoT CP connection is established, the UE could access a satellite and perform UL and DL transmission, after the transmission is finished, the eNB could release UE in RRC_IDLE. If CIoT UP connection is established, the eNB could release UE in RRC_IDLE with suspend, so that the UE could keep the AS context. In summary, for both of these two kinds of solutions, eNB can follow legacy mechanisms to release the UE to RRC_IDLE, when UL and DL transmission is finished.

Proposal 6: For the split MME architecture, during the attach procedure or data transmission procedure (via CIoT CP solution or CIoT UP solution), eNB can release the UE to RRC_IDLE as in legacy, when UL and DL transmission with this satellite is finished. No impact on RRC State.

- For full CN on-board architecture

Under this architecture, the UE attaches, transfers data and detaches from each satellite as required. During the connection of one satellite, CIoT CP solution, CIoT UP solution and normal connection can be supported theoretically, and there is no difference with legacy architecture. Hence, the eNB could follow legacy mechanisms to control the RRC state of UE.

Proposal 7: For the full CN on-board architecture, eNB controls the RRC state of UE as in legacy. No impact on RRC State.

Conclusion

In this contribution, some potential issues on supporting S&F operation are discussed with corresponding observations and proposals listed as follows:

Observation 1: For both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs for UE to monitor and access, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

Observation 2: For both split MME architecture and full CN on-board architecture, SA2 agreed that the NW shall be able to inform UE of whether S&F operation is applied. How to inform UE depends on RAN.

Observation 3: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected), in order to receive possible paging and/or initiate access to receive MT data and signalling.

Proposal 1: Introduce a new barring bit to inform UE of whether the S&F operation is applied and whether the satellite is barred for access of the UEs supporting S&F operation.

Proposal 2a: When a cell is applying S&F operation, it includes the S&F specific barring bit (e.g. sf-CellBarredNTN) and UEs applying S&F operation determines whether the cell is barred or not depending only on this barring bit (and ignoring existing cellBarred/cellBarred-NTN).

Proposal 2b: When a cell is applying S&F operation, the cellBarred-NTN is used to bar the connectivity to NTN from UEs not applying S&F operation.

Proposal 2c: When a cell is not applying S&F operation (e.g. turning from S&F satellite to non-S&F satellite), NW via implementation sets the S&F specific barring bit (e.g. sf-CellBarredNTN) to absent (no Spec impact).

Proposal 3a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" status of a target satellite indicated by MME is up to UE implementation (via using satellite assistance info).

Proposal 3b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

Proposal 4a: Introduce a separate list to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the serving time of the satellites indicated by MME.

Proposal 4b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

Proposal 5: RAN2 needs to enhance cell (re)selection mechanism to ensure UE to (re-)select a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the cell which belongs to the indicated satellite to camp on, etc.).

Proposal 6: For the split MME architecture, during the attach procedure or data transmission procedure (via CIoT CP solution or CIoT UP solution), eNB can release the UE to RRC_IDLE as in legacy, when UL and DL transmission with this satellite is finished. No impact on RRC State.

Proposal 7: For the full CN on-board architecture, eNB controls the RRC state of UE as in legacy. No impact on RRC State.

Reference

- RAN2 #126 meeting minutes

- 3GPP TR 23700-29: "Study on integration of satellite components in the 5G architecture ".