3GPP TSG-RAN WG2 Meeting #127	R2-2406327

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on the feasibility of RAN2 scope for UL capacity enhancements

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting [1], agreements were made for UL capacity enhancements as follows:

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

LS on “uplink synchronization for contention based msg3 transmission” [2] was also sent to RAN1/RAN4 for response on required timing accuracy for Msg3 transmission without Msg1/Msg2.

In this document, we discuss the feasibility of the RAN2 scope for the UL capacity enhancements in IoT NTN. Our observations as well as proposals are provided.

Discussion

For the feasibility on the TA related issue, since RAN2 has sent LS to RAN1 and RAN4, RAN2 needs to wait for the reply without progressing on any related issue.

Proposal 0: RAN2 waits for RAN1/RAN4 response before carrying out any further discussions on TA related issue for CB-Msg3 transmission.

Actually, at the last meeting, there was another issue raised concerning the feasibility to introduce this CB-Msg3 transmission, i.e. how gNB handles the transmission collisions by different UEs on the same PUSCH resource. People in the last meeting seemed to assume that the there is still likeliness for the gNB to distinguish and thus decode the transmissions from different UEs successfully, even if the multiple transmissions are performed on the same PUSCH resources. However, there is no clear clarification on which technique can the gNB applies to perform such collision handling on the PUSCH, making this assumption a bit skeptical. Note that such collision on PUSCH is quite different from Msg1 transmissions where different UEs can be distinguished by different preambles used, even if the same PRACH occasion is selected.

Fundamentally, there is anyway necessarily one dimension that needs to be relied on to distinguish the multiple transmissions. If now the CB-Msg3 transmission assumes the possibility of more than one transmission from different UEs using the same time-frequency PUSCH occasion, there has to be some other ways for the gNB to perform the collision handling and distinguish the transmissions from different UEs, in order to achieve successful decoding. Otherwise, the transmissions fail, resources are wasted and the UL capacity is degraded in the other way around. This collision issue gets much severe, when a large number of UEs are performing such CB-Msg3 transmission at the same time, which is however actually a typical case in NTN with broad cell coverage.

Observation 0: RAN2 needs to address the issue on how gNB handles collisions of multiple UEs' CB-Msg3 transmissions on the same PUSCH resources, in order for successful decoding of each UE's transmission. Otherwise, the severe collision degrades resource efficiency, resulting in the feasibility of such CB Msg3 transmission skeptical, especially on whether such CB-Msg3 transmission can really improve UL capability.

From our perspective, in general such collision handling may be achieved in code domain or in time domain. We list the potential candidates and provide some analyses based on our understanding.

Code-domain collision handling

Candidate code-domain collision handling can rely on, from our perspective, techniques like OCC, 2-step RACH and DMRS.

- OCC

OCC is widely used in LTE and NR, for example on PUCCH. In order to increase the PUSCH capacity, RAN1 has studied OCC for NPUSCH in Rel-19 [3][4]. The following agreements were made:

RAN1#116bis Agreement OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

RAN1#117 Agreement For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Agreement The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

It can be seen that for PUSCH, multiplexing more than 4 UEs will not be considered by RAN1 for NPUSCH format 1. Furthermore, the OCC transmission requires symbol/slot alignment and the PUSCH transmission from different UEs should be configured with the same length according to the OCC characteristics. Considering EDT may be initiated by the UEs with different data volume, this will bring huge impacts and restriction on the resource allocation, especially when the resource is configured via broadcast signaling. RAN2 can check with RAN1 on the feasibility of OCC, considering that whether OCC can work and how OCC works highly depend on RAN1.

Observation 1: OCC allows multiple UEs to transmit at the same time to improve the uplink capability for Msg3-EDT transmission without msg1/RAR, but it has rigorous requirement on symbol/slot alignment which involves RAN1 justification. Also, multiplexing more than 4 UEs are not supported for NPUSCH format 1, as determined for this release by RAN1.

- 2-Step RACH

For 4-step RACH access procedure, ZC sequence is used to enable many UEs simultaneously transmitting at slightly different timing offsets. But, 4 steps are required bringing latency on transmission. In order to reduce the overall latency caused by 4-step RACH, 2-step RACH was introduced in Rel-16 NR. It allows PUSCH transmission at the very first step reducing the number of signaling steps for RACH procedure, thus able to realize the purpose of Msg2 overhead saving to some extent.

In case the MsgA preamble transmission from multiple users (UEs) trying to perform the initial access is mapped to the same PUSCH physical resources, the associated data parts overlap and may result in unsuccessful decoding. However, if PUSCH transmission fails, the UE can fallback to 4-step RACH.

Therefore, not only TA issue is solved in 2-step RACH but also contention resolution procedure among multiple UEs can simply follow the legacy mechanism as already specified in NR.

However, it is questionable whether 2-step RACH is in the WI scope, and it has never been introduced into LTE Spec.

Observation 2: 2-step RACH may realize Msg2 overhead saving purpose, and may be able to resolve the contention resolution/TA issues as well. However, it has never been introduced in LTE Specs, and whether it is in/out of WI is questionable.

- DM-RS

Inspiring from CG type 1, when the multiple UEs are configured with different DM-RS, the network may decode the transmissions on the same PUSCH from different UEs successfully if the DM-RS is decoded successfully. However, such UE differentiation based on DM-RS on the same PUSCH resource typically need be applied together with MU-MIMO, where the spatial characteristics for different UEs' radio links are required for MU-MIMO handling. This further requires channel status of different UEs well tracked at the gNB side, which is hardly realized in the CB-Msg3 transmission that mainly aims at RRC_IDLE UEs.

Observation 3: Using DM-RS may be one way to differentiate transmissions from different UEs on the same PUSCH resource. But this needs to be done together with MU-MIMO which requires effective tracking of different UEs' channel status at the gNB, and is thus difficult to be realized for IDLE UEs.

Time domain collision handling

- (CR)DSA

It was proposed in [5] to rely on a contention resolution diversity slotted ALOHA ((CR)DSA) mechanism to handle the collision of UEs transmitted on the same PUSCH resource. The basic concept is to rely on different resource patterns to avoid subsequent collisions on the repetition/retransmissions by different UEs. So conceptually, it is a time-domain method to handle the collisions.

Actually a similar way was specified in LTE sidelink (mode-4) and NR sidelink (mode-2), where the UE can randomly select PSSCH resources, and use the randomized resources selected to avoid collisions with other UEs. However, this method on (CR)DSA may need further discussions and bring non-trivial Spec impact to Uu, for example: pattern design, trigger of resource (re)selection, how to select resources, whether to sense the channel, etc. Such discussions involve not only RAN2 but also RAN1. Also, the interference cancelation techniques, as mentioned by [5], could be very complicated, with the feasibility and benefits it can bring both unclear.

Also, this (CR)DSA mechanism may come at the sacrifice of resource efficiency, as the collision on the same resources may not be able to be effectively resolved. It can be foreseen that the collision probability can be increased dramatically, when the UE number is large. Therefore, whether this mechanism can really realize the UL capacity purpose needs to be justified.

Observation 4: (CR)DSA relies on the repetition/retransmission with different resource patterns in the time domain to handle collisions among different UEs. However it may involve non-trivial discussions/Spec impacts on RAN1/RAN2 and the benefit to UL capability improvement may need to be justified first.

Summary

Based on the analysis above, we propose that RAN2 to first justify the feasibility of CB-Msg3 transmission by down-selecting the collision handling method and taking into account whether the candidate solution can really bring UL capability improvement.

Proposal 1: RAN2 discusses the feasibility of CB-Msg3 transmission and justifies how the gNB handles the transmission collision on the same PUSCH resource by different UEs, by taking into account the following candidates and the UL capability gain they can bring:

- Code-domain collision handling, e.g. via OCC, 2-Step RACH, DMRS.

- Time domain collision handling, e.g. (CR)DSA.

- 3.	Conclusion

In this document, we analyse the potential impacts for UL capacity enhancements in IoT NTN, and find the following observations and proposals:

Observation 0: RAN2 needs to address the issue on how gNB handles collisions of multiple UEs' CB-Msg3 transmissions on the same PUSCH resources, in order for successful decoding of each UE's transmission. Otherwise, the severe collision degrades resource efficiency, resulting in the feasibility of such CB Msg3 transmission skeptical, especially on whether such CB-Msg3 transmission can really improve UL capability.

Observation 1: OCC allows multiple UEs to transmit at the same time to improve the uplink capability for Msg3-EDT transmission without msg1/RAR, but it has rigorous requirement on symbol/slot alignment which involves RAN1 justification. Also, multiplexing more than 4 UEs are not supported for NPUSCH format 1, as determined for this release by RAN1.

Observation 2: 2-step RACH may realize Msg2 overhead saving purpose, and may be able to resolve the contention resolution/TA issues as well. However, it has never been introduced in LTE Specs, and whether it is in/out of WI is questionable.

Observation 3: Using DM-RS may be one way to differentiate transmissions from different UEs on the same PUSCH resource. But this needs to be done together with MU-MIMO which requires effective tracking of different UEs' channel status at the gNB, and is thus difficult to be realized for IDLE UEs.

Observation 4: (CR)DSA relies on the repetition/retransmission with different resource patterns in the time domain to handle collisions among different UEs. However it may involve non-trivial discussions/Spec impacts on RAN1/RAN2 and the benefit to UL capability improvement may need to be justified first.

Proposal 0: RAN2 waits for RAN1/RAN4 response before carrying out any further discussions on TA related issue for CB-Msg3 transmission.

Proposal 1: RAN2 discusses the feasibility of CB-Msg3 transmission and justifies how the gNB handles the transmission collision on the same PUSCH resource by different UEs, by taking into account the following candidates and the UL capability gain they can bring:

- Code-domain collision handling, e.g. via OCC, 2-Step RACH, DMRS.

- Time domain collision handling, e.g. (CR)DSA.

- 4.	Reference

- RAN2#126 chairman notes;

- R2-2405769, LS on uplink synchronization for contention based msg3 transmission, ZTE, LS out, Rel-19, IoT_NTN_Ph3-Core, To:RAN1,RAN4;

- R1-2403821, RAN1#116bis chairman notes;

- RAN1#117 chairman notes;

- R2-2405202, Discussion on Msg3-EDT enhancements, ESA.