3GPP TSG-RAN WG2 Meeting #127	R2-2406331

Maastricht, Netherlands, Aug 19th – 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | Draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 38.321 for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue 1 Regarding LTM with MIMO two TA, in 5.2 it has captured that, 1> when an LTM Cell Switch Command MAC CE is received and the Timing Advance Command is not set as FFF: 2> apply the Timing Advance Command for the PTAG indicated by the LTM Cell Switch Command MAC CE; 2> start or restart the timeAlignmentTimer associated with the PTAG indicated by LTM Cell Switch Command MAC CE.. The wording “for/with the PTAG indicated by the LTM Cell Switch Command MAC CE” is not clear, due to the fact that the PTAG is not indicated in the LTM Cell Switch Command MAC CE directly. However how to apply the TA value with its corresponding PTAG is specified in 6.1.3.75 clearly: If tag-Id-ptr is configured for the TCI state indicated by the TCI state ID field in the LTM target cell and tag-Id-ptr is set to value n1, this field indicates the TA for the TAG indicated by the tag2-Id of the LTM target cell; otherwise, this field indicates the TA for the TAG indicated by the tag-id of the LTM target cell.. Thus, the current procedure description in 5.2 on how to apply the TA value with its corrsponding PTAG could refer to the clause of 6.1.3.75 simply. Issue 2 In 5.18.35, upon receiving the LTM Cell Switch Command MAC CE, after MAC reset, UE always uses the the SSB associated to the TCI state for configured uplink grant selection. But the configured uplink grant may not be present as it is optionally configured by the NW. Therefore, UE should first check whether configured uplink grant is configured or not. Issue 3 Similar as legacy, the configured unified TCI states for LTM candidate cell should be deactivated upon configuration or re-configuration or after reconfiguration with sync. The description is missed in 5.18.36. Besides, according to RAN1 LS R2-2402117, After reception of LTM cell switch command, UE deactivates all activated LTM TCI states other than indicated TCI state.. Although it had been captured in TS 38.300, it’s better to also reflect the handling of activated TCI state upon reception of LTM cell switch command in 5.18.36 of TS 38.321. |
| Summary of change: |  | Change 1 Capture in 5.2 that the application of the TA value with its corrsponding PTAG refers to the clause of 6.1.3.75. Change 2 In 5.18.35, add the if condition “3> if RACH-less cell switch is ongoing and the configured uplink grant for the initial uplink transmission towards the candidate cell has been provided:” for UE to verify whether to use the SSB indicated by the TCI state to determine the CG resource for the first PUSCH transmission. Change 3 Capture the following description in 5.18.36, “The configured TCI states are initially deactivated upon (re-)configuration by upper layers. All TCI states are deactivated after the reconfiguration with sync except the reconfiguration with sync is triggered by LTM. All the activated TCI states except those received in the cell switch command are deactivated upon LTM cell switch execution.” Impact analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: LTM Inter-operability: Issue 1 There is no inter-operatility issue foreseen. Issue 2 There is no inter-operability issue foreseen. Issue 3 If UE implements this but NW not, there is no inter-operability issue foreseen; If NW implements this but UE not, UE may keep some TCI state activated, which is not the NW expectation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The procedure for UE to apply the Timing Advance Command in the LTM Cell Switch Command MAC CE is confusing when the candidate cell is configured with two-TA; - UE will perform the CG resource determination in vain if the NW does not provide the CG resource configuration for the first PUSCH transmission of RACH-less cell switch; - Some situations that UE should deactivate the previously activated TCI state are missed; |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.18.35, 5.18.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG;

-	inactivePosSRS-TimeAlignmentTimer which controls how long the MAC entity considers the Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned;

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned;

-	inactivePosSRS-ValidityAreaTAT which controls how long the MAC entity considers Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned when SRS positioning validity area is configured.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell configured with two TAGs or in a MSGB for an SpCell configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for the TAG indicated in the received Random Access Response message or MSGB;

3>	start or restart the timeAlignmentTimer associated with TAG indicated in the received Random Access Response message or MSGB.

2>	else if the timeAlignmentTimer associated with the TAG indicated in the received Random Access Response message or MSGB is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	stop the timeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell not configured with two TAGs or in a MSGB for an SpCell not configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

5>	set the NTA value to the value before applying the received Timing Advance Command as in TS 38.211 [8].

3>	when the Contention Resolution is considered successful for Random Access procedure while the CG-SDT procedure is ongoing:

4>	stop timeAlignmentTimer associated with this TAG;

4>	start or restart the cg-SDT-TimeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered successful for Random Access procedure while SRS transmission in RRC_INACTIVE is ongoing:

4>	if SRS positioning validity area is configured:

5>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

4>	else:

5>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell configured with two TAGs:

2>	apply the Timing Advance Command for the PTAG indicated in the Absolute Timing Advance Command MAC CE;

2>	start or restart the timeAlignmentTimer associated with this PTAG.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell not configured with two TAGs:

2>	apply the Timing Advance Command for PTAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the inactivePosSRS-ValidityAreaTAT associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with PTAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with PTAG.

1>	when the MAC entity is configured with rach-LessHO:

2>	set the NTA value (as defined in TS 38.211 [8]) to the value indicated by targetNTA in rach-LessHO for PTAG;

2>	start the timeAlignmentTimer associated with PTAG.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer:

2>	stop the inactivePosSRS-TimeAlignmentTimer.

1>	when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer:

2>	start or restart the inactivePosSRS-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for starting the cg-SDT-TimeAlignmentTimer:

2>	start the cg-SDT-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for stopping the cg-SDT-TimeAlignmentTimer:

2>	consider the cg-SDT-TimeAlignmentTimer as expired.

1>	when the indication is received from upper layer for starting the inactivePosSRS-ValidityAreaTAT:

2>	start or restart the inactivePosSRS-ValidityAreaTAT.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-ValidityAreaTAT:

2>	stop the inactivePosSRS-ValidityAreaTAT.

1>	when instruction from the upper layer has been received for starting the TimeAlignmentTimer associated with PTAG:

2>	start the TimeAlignmentTimer associated with the indicated PTAG.

1>	when an LTM Cell Switch Command MAC CE is received and the Timing Advance Command is not set as FFF:

2>	apply the Timing Advance Command for the PTAG as in clause 6.1.3.75;

2>	start or restart the timeAlignmentTimer associated with the PTAG as in clause 6.1.3.75.

1>	when an LTM Cell Switch Command MAC CE is received, and the Timing Advance Command is set as FFF, and the UE has successfully measured the Timing Advance as in clause 5.18.35:

2>	apply the measured Timing Advance for the PTAG;

2>	start or restart the timeAlignmentTimer associated with the PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or

2>	if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else:

3>	if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or

3>	if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running:

4>	flush all HARQ buffers for all such SCells;

4>	notify RRC to release PUCCH, if configured for all such SCells;

4>	notify RRC to release SRS, if configured for all such SCells;

4>	clear any configured downlink assignments and configured uplink grants for all such SCells;

4>	clear any PUSCH resource for semi-persistent CSI reporting for all such SCells;

4>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

3>	else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells:

4>	clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer;

4>	clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

4>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

1>	when the inactivePosSRS-TimeAlignmentTimer expires:

2>	notify RRC to release Positioning SRS for RRC_INACTIVE configuration(s).

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell not configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

When the MAC entity stops uplink transmissions associated to a STAG for an SCell configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the STAG as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer(s) associated with all TAG(s) to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing. Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or inactivePosSRS-ValidityAreaTAT is not running. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

Next Change

### 5.18.35	LTM Cell Switch Command

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing.

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	if RACH-less cell switch is ongoing and the cg-LTM-Configuration is included in the candidate configuration indicated by the Target Configuration ID:

4>	consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell (as in clause 5.8.2);

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

Next Change

### 5.18.36	Candidate Cell TCI States Activation/Deactivation

The network may activate and deactivate the TCI states of LTM candidate cell(s) configured in CandidateTCI-State and CandidateTCI-UL-State by sending the Candidate Cell TCI States Activation/Deactivation MAC CE described in clause 6.1.3.76. The network deactivates the TCI state(s) for one LTM candidate cell by not including the corresponding TCI state ID field(s) in the Candidate Cell TCI States Activation/Deactivation MAC CE. The configured TCI states are initially deactivated upon (re-)configuration by upper layers. All TCI states are deactivated after the reconfiguration with sync except the reconfiguration with sync is triggered by LTM. All the activated TCI states except those received in the cell switch command are deactivated upon LTM cell switch execution.

The MAC entity shall:

1>	if the MAC entity receives a Candidate Cell TCI States Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Candidate Cell TCI States Activation/Deactivation MAC CE.

End of Change