3GPP TSG RAN WG2 Meeting #127	R2-2406335

Maastricht, Netherlands  19-23 August 2024

Agenda Item:	8.1.2.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	LCM for UE-sided model for Beam Management use case

Document for:	Discussion & Decision

# Introduction

The WID [1] , lists, among others, the following as objective for RAN2:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we focus on aspects of monitoring and management of AI/ML models forming part of functionality.

# Functionality identification

A UE may have one or more AI/ML models for an active functionality, with one AI/ML model designated for inference as the active model. Besides, a fallback mechanism can be associated with an active functionality. In RAN2#126, the following was agreed “the UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability.”

An AI/ML model may need to be switched or deactivated based on the outcome of monitoring, since in some situations the performance of the active AI/ML model at the UE may not be sufficient. This may require an inactive model to be used for inference (or even fetched to the UE when not available) or a fallback solution to be triggered. To avoid latency or failures, a UE shall be configured with a fallback solution. The fallback solution may be activation of a legacy mechanism.

Observation 1: An AI/ML model at UE may provide errorneous inference results, and may require fallback operation.

Proposal 1: An activated functionality shall consist of one or more AI/ML models and shall be a subset of configured functionalities, and there is a fallback mechanism for a configured functionality.

Proposal 2: At most one model may be used for inference for a functionality at a given time, a fallback mechanism shall be activated when the performance of an active AI/ML models falls below a threshold value.

# Model switching:

Instead of running several models in parallel to determine if an inactive model (or functionality) would perform better, a UE may utilize model monitoring and management algorithms to predict the expected performance of inactive models/functionalities before switching. But performance is not the only decision criterion, since different AI/ML models/functionalities may come with different costs in terms of:

      - Computation, which may affect the battery life

      - Signalling, including the reference signals that need to be processed

      - Activation time, which may be critical for latency critical applications

      - Latency, for example, in transferring a model not already available at the UE

      - Costs of monitoring, for example, different monitoring configuration may be needed for different functionalities/models.

A balance between performance requirements and cost of activation/switching needs to be ensured. In certain situations, it may be beneficial to fallback to legacy mechanisms instead of keeping an AI/ML assisted functionality.

Observation 2: Different AI/ML models/functionalities may come with different performance benefits and costs in terms of computation, signalling, activation time, latency, cost of monitoring, etc..

Proposal 3: A UE shall indicate a metric to the network using UE assistance information (UAI), wherein the metric takes into account the performance benefit and/cost of deactivating a model/functionality and activating a new model/functionality.

# Diagnosis of AI/ML model failure

One or more monitoring configurations shall be executed, either in series or in response to the outcome of one monitoring analytic. A key part of monitoring is to evaluate if a model is not performing adequately due to  the environment being permanently changed.

Temporary (such as constructions for events) or permanent environmental alterations may render the AI/ML model or functionality available at the UE to perform suboptimally. Thus, the data observed during training may be different from the data observed during inference. If it is environment specific, then a group of UEs will see similar performance instead of one particular UE observing degradation. If the network observes similar performance from a group of UEs, then such information can be used to adapt the AI/ML model applicable to an area.

Observation 3: Alterations in environment (temporary or permanent) can alter the validity of AI/ML model used by the UE.

Proposal 4: The gNB shall collect performance metrics from several UEs and use such information to determine whether the environment has changed permanently and therefore AI/ML models are no longer performing as expected.

FFS: Which KPIs and which reporting mechanism shall be used.

# Conclusions

In this contribution, we make the following observations:

Observation 1: An AI/ML model at UE may provide errorneous inference results, and may require fallback operation.

Observation 2: Different AI/ML models/functionalities may come with different costs and benefits in terms of computation, signalling, activation time, latency, cost of monitoring and so on.

Observation 3: Alterations in environment (temporary or permanent) can alter the validity of AI/ML model used by the UE.

Based on the above observations, we make the following proposals:

Proposal 1: An activated functionality shall consist of one or more AI/ML models and shall be a subset of configured functionalities, and there is a fallback mechanism for a configured functionality.

Proposal 2: At most one model may be used for inference for a functionality at a given time, a fallback mechanism shall be activated when the performance of an active AI/ML models falls below a threshold value.

Proposal 3: A UE shall indicate a metric to the network using UE assistance information (UAI), wherein the metric takes into account the benefit and/cost of deactivating a model/functionality and activating a new model/functionality.

Proposal 4: The gNB shall collect performance metrics from several UEs and use such information to determine whether the environment has changed permanently and therefore AI/ML models are no longer performing as expected.

FFS: Which KPIs and which reporting mechanism shall be used.

# References

| [1] | RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," 3GPP, Online, 2023-12. |
| --- | --- |