3GPP TSG-RAN2 Meeting #127	R2-2406336

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	5.1.3.1

Source: 	MediaTek Inc.

Title:  	Preconditions for MCG reconfiguration with sync

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss potential ambiguity in TS 38.331 (Rel-15 and onwards) regarding preconditions for MCG reconfiguration with sync procedure.

# Discussion

## Preconditions for MCG reconfiguration with sync

For MCG reconfiguration with sync procedure, preconditions are specified in clause 5.3.5.2 of TS 38.331 [1]:

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

The discussion in this paper is about interpretation of the "SRB2 with at least one DRB are setup" part of the precondition marked in yellow highlight above, especially the "are setup" part.

There are two possible interpretations of the "are setup":

- Interpretation 1: SRB2 and at least one DRB must exist in the UE configuration before the network sends RRCReconfiguration message including reconfigurationWithSync for MCG to the UE.

- Interpretation 2: The network can setup SRB2 and at least one DRB to the UE configuration with the same RRCReconfiguration message which includes reconfigurationWithSync for MCG.

The "are setup" part of the "the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended" can be understood in two different ways.

## LTE Background

NR inherits most RRC designs from LTE, including handover procedure principles. RAN2 has revisited the signalling only connection issue and had following agreement at RAN2#103bis [2]:

Agreements

1: 	It should be supported to set up security (SMC) without setting up SRB2/DRB.

2: 	No change to the current approach that handover is only possible after SRB2/DRB are established

RAN2 has agreed that handover (i.e., MCG reconfiguration with sync) is possible only after SRB2 and DRB are established.

## T304 Expiry

If the MCG reconfiguration with sync procedure fails (T304 expires), the UE follows clause 5.3.5.8.3 of TS 38.331 [1]:

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	revert back to the UE configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

2>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

If MCG reconfiguration with sync procedure fails, the UE is required to revert back to the configuration used in the source PCell and initiate RRC connection re-establishment procedure.

When the UE initiates RRC connection re-establishment procedure for MCG T304 expiry, the UE configuration does not contain any SRB/DRB additions configured by the RRCReconfiguration message which included reconfigurationWithSync for MCG.

For the RRC connection re-establishment procedure, clause 5.3.7.1 of TS 38.331 [1] specifies:

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

The network applies the procedure e.g as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RBs;

-	to fallback to establish a new RRC connection.

If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'.

The UE can initiate RRC connection re-establishment procedure only if the UE configuration contains SRB2 and at least one DRB.

Based on above, the UE can't follow the requirement specified in clause 5.3.5.8.3 to initiate RRC connection re-establishment procedure, if the UE configuration before the MCG reconfiguration with sync procedure does not contain SRB2 and at least one DRB.

The UE behaviour upon T304 expiry is unspecified, if the network sends RRCReconfiguration message including reconfigurationWithSync for MCG to the UE when the current UE configuration does not include SRB2 and at least one DRB.

## Conclusion

Given the earlier RAN2 agreement (chapter 2.2) and unspecified UE behaviour in the scenario where the network sends RRCReconfiguration with reconfigurationWithSync for MCG when the current UE configuration does not contain SRB2 and at least one DRB (chapter 2.3), we think the interpretation 1 is correct and consequently the interpretation 2 is incorrect. We kindly request RAN2 to confirm this understanding.

RAN2 to confirm that the interpretation 1 (i.e., SRB2 and at least one DRB must exist in the UE configuration before the network sends RRCReconfiguration message including reconfigurationWithSync for MCG to the UE) is the correct interpretation of 38.331 clause 5.3.5.2.

# Summary

RAN2 is kindly requested to confirm the understanding:

Proposal 1	RAN2 to confirm that the interpretation 1 (i.e., SRB2 and at least one DRB must exist in the UE configuration before the network sends RRCReconfiguration message including reconfigurationWithSync for MCG to the UE) is the correct interpretation of 38.331 clause 5.3.5.2.

# References

- TS 38.331, V15.26.0

- Report of RAN2#103bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_103bis/Report/R2-1816201.zip