3GPP TSG-RAN WG2 Meeting #127	R2-2406337

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:	7.4.2

Source: 	MediaTek Inc.

Title: 	Rel-18 LTM CP remaining issues

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the remaining control plane issues for Rel-18 LTM.

# 2. Discussion

## 2.1 TCI state configuration and activation

In RAN2#126 meeting, the TCI state issue was discussed, and the following agreements were made:

- RAN2 assumes that TCI configuration under LTM TCI info could be different to TCI configurations in candidate configuration. - RAN2 to adopt Approach 2 from R2-2404115, i.e., introduce new RRC parameters under LTM-TCI-Info.

The intention of these agreement is to say that the TCI states in the LTM candidate configuration (ltm-CandidateConfig) could be different from LTM TCI info (ltm-TCI-Info) which is used in early DL sync and LTM cell switch execution stages.  During the meeting, RAN2 understands that the TCI states in the LTM candidate configuration will not be activated by the UE upon LTM cell switch execution, but will only be activated by subsequent TCI activation using MAC-CE/DCI.

However, there is a remaining issue here. According to TS 38.214, if RRC configuration indicates multiple TCI states, those TCI states will not be applied or activated until the UE receives MAC-CE/DCI for TCI activation. However, if the RRC configuration indicates only one TCI state, that TCI state is activated without further MAC-CE/DCI for TCI activation.

TS 38.214 V18.3.0 If a UE receives a higher layer configuration of dl-OrJointTCI-StateList where only one TCI-State can be used as an indicated TCI state, the UE obtains the QCL assumptions from that TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

Observation 1: According to 38.214 (legacy rule), if only one TCI state is indicated in RRC configuration, it is activated without the need for further MAC-CE/DCI for TCI activation.

In the context of LTM, this discrepancy may lead to misalignment between the UE and the network. If the network configures only one TCI state in the LTM candidate configuration, different from the LTM TCI info (ltm-TCI-Info), then the UE does not know which TCI state is to be activated when receiving the LTM Cell Switch Command MAC CE. This is because there is no further activation indication to the TCI state in the LTM candidate configuration and it should be activated as legacy rule.

Therefore, we propose to clarify that the network should ensure the consistency of the TCI states configuration in this scenario when only one TCI state is present in the LTM candidate configuration:

Proposal 1: RAN2 to clarify that if there is only one TCI state in ltm-CandidateConfig for the candidate cell indicated by LTM Cell Switch Command MAC CE, the network ensures this TCI state is identical with the TCI state indicated by LTM Cell Switch Command MAC CE.

## 2.2 SpCell reporting

In current LTM report config, the number of report cells and the RS's per cell is indicated as the following structure:

LTM-ReportContent-r18 ::=     		SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}             OPTIONAL -- Need R

}

The value of cells and beams are further indicated by UE capabilities R1 45-1/45-1a.

The spCellInclusion flag can be optionally configured to indicate whether the UE shall include a L1 measurement report associated to the current SpCell. In the field description, it is said that this field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration.

LTM-ReportContent is included in LTM-CSI-ReportConfig, but it is not clear whether SpCell resources are mandatorily included in LTM-CSI-ResourceConfig while spCellInclusion is True. If LTM-CSI-ResourceConfig does not include SpCell resources while spCellInclusion is True, it is not clear whether the UE needs to report extra beams for the SpCell in addition to the non-SpCell beams configured by LTM-ReportContent or not.

For example:

UE’s current SpCell is cell A; LTM-CSI-ResourceConfig contains SSBs of neighbor cells B,C, and D;

In LTM-ReportContent, nrOfReportedCells = 1, nrOfReportedRS-PerCell = 1, and spCellInclusion = True.

Question: Does the UE need to report SSB for the SpCell in this case?	What is the total number of reported beams?

Observation 2: It is not clear whether the UE needs to report extra beams for SpCell in addition to the non-SpCell beams, if LTM-CSI-ResourceConfig does not include SpCell resources while spCellInclusion is set to True.

In order not to raise further complicated discussion, one simple way is to add clarification that spcellInclusion cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig.

Proposal 2: In the field description of spcellInclusion, clarify that it cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig.

A TP and related inter-operability analysis can be found below.

---- Begin TP ----

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and a CSI resource of the current SpCell is configured in LTM-CSI-ResourceConfig.

---- End TP ----

Inter-operability analysis:

- If the network implements the change and the UE does not, there is no interoperability issue.

- If the UE implements the change the network does not, the UE and the network might have different understanding on whether the UE should report SSB of the current SpCell or not.

## 2.3 unifiedTCI-StateType

In LTM-TCI-Info, the field unifiedTCI-StateType is optional Need R field. This means that the UE LTM configuration does not necessarily contain unifiedTCI-StateType for an LTM candidate.

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18       OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId                  OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18       OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17             OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource            OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId          OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet      OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId    OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17    OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17 OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                                                       OPTIONAL,    -- Need R

...

}

Observation 3: The UE LTM configuration does not necessarily contain unifiedTCI-StateType for an LTM candidate.

However, according to the field description, the UE needs the unifiedTCI-StateType in order to know how to process ltm-DL-OrJointTCI-StateToAddModList and ltm-UL-TCI-StateToAddModList.

LTM-TCI-Info field descriptions

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation.

Observation 4: The UE behaviour is unspecified if unifiedTCI-StateType is absent but TCI states are configured for an LTM candidate in LTM-TCI-Info.

Since the purpose of the LTM-TCI-Info is to provide TCI state information of an LTM candidate, it seems the unifiedTCI-StateType is actually mandatory information in LTM-TCI-Info, i.e., there seems to be no need for the network to release unifiedTCI-StateType in LTM-TCI-Info. However, as a change of optional field to mandatory field by removing the OPTIONAL keyword would be an NBC ASN.1 change, we instead propose to update the field description of unifiedTCI-StateType to specify a value (value separate) which the UE applies in case unifiedTCI-StateType is absent.

Proposal 3: In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

A TP and related inter-operability analysis can be found below.

---- Begin TP ----

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18       OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId                  OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18       OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17             OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource            OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId          OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet      OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId    OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17    OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17 OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                                                       OPTIONAL,    -- Need S

...

}

LTM-TCI-Info field descriptions

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation. If the field is absent, the UE applies value separate.

---- End TP ----

Inter-operability analysis:

- If the network implements the change and the UE does not, the UE and the network might have different understanding of the value of unifiedTCI-StateType.

- If the UE implements the change the network does not, the UE and the network might have different understanding of the value of unifiedTCI-StateType.

## 2.4 UE-based TA measurements upon SpCell change

### 2.4.1 Overview

We discuss four different issues related to handling of the UE-based TA measurements upon SpCell change. The issues in chapters 2.4.2, 2.4.3, and 2.4.4 are related to the procedural text in TS 38.331 clause 5.3.5.18.6, which is copied below and marked with color coding further referred to within 2.4.2, 2.4.3, and 2.4.4. The issue in chapter 2.4.5 is not related to clause 5.3.5.18.6. After description of each issue, we provide a joint TP for the issues discussed in chapters 2.4.2, 2.4.3, and 2.4.4. For the issue in chapter 2.4.5, we do not provide a TP.

TS 38.331, clause 5.3.5.18.6 'LTM cell switch execution':

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

1>	else if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-UE-MeasuredTA-ID:

### 2.4.2 UE-based TA measurements are not stopped for LTM candidate which becomes the current SpCell by LTM cell switch procedure

The UE-based TA measurements is a method for early UL sync, which is in general relevant procedure for LTM candidates other than a candidate which is the current SpCell. The early UL sync is not necessary for the LTM candidate which is the current SpCell, because the TA for the current SpCell is already known and maintained by the UE/gNB.

Observation 5: The UE-based TA measurements are not applicable to an LTM candidate which is the current SpCell.

However, the current procedural text in TS 38.331 clause 5.3.5.18.6 does not result the UE to stop the UE-based TA measurements for the LTM candidate which becomes the current SpCell by LTM cell switch procedure. In the procedural text, there is no else-branch for the condition in yellow. Since the condition in yellow evaluates as true, if the UE-based TA measurements were not configured for the LTM candidate which becomes the current SpCell, an else branch is required where the UE-based TA measurements are stopped.

Observation 6: The UE-based TA measurements are not stopped for the LTM candidate which becomes the current SpCell by LTM cell switch procedure.

Proposal 4: In clause 5.3.5.18.6, correct the procedural text so that the UE stops the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

### 2.4.3 UE-based TA measurements are started for LTM candidate which becomes the current SpCell by LTM cell switch procedure

If the condition in yellow in the procedural text evaluates as true, the UE erroneously considers the UE-based TA measurements as configured for the LTM candidate which becomes the current SpCell, because the condition in green evaluates always as true for the LTM candidate. The condition in green evaluates always as true, because the UE variable is updated (purple text) based on the LTM candidate before the evaluation.

Observation 7: The UE-based TA measurements are started for the LTM candidate which becomes the current SpCell by LTM cell switch procedure.

However, proper correction for this issue is not to move the purple UE variable updating to occur after the green condition has been evaluated, since it would cause the condition to be evaluated wrongly for the other LTM candidates. Instead, one way to correct the issue is to rule out the LTM candidate which becomes the current SpCell from the green condition evaluation.

Proposal 5: In clause 5.3.5.18.6, correct the procedural text so that the UE does not start the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

### 2.4.4 Unnecessary procedural text

The procedural text in turquoise to stop the UE-based TA measurements is fully unnecessary, because the UE-based TA measurements were never ongoing for the LTM candidate which becomes the serving cell, if the LTM candidate does not have field ltm-UE-MeasuredTA-ID.

Proposal 6: In clause 5.3.5.18.6, remove the unnecessary procedural text to stop the UE-based TA measurements when the LTM candidate which becomes the current SpCell does not have field ltm-UE-MeasuredTA-ID.

### 2.4.5 Handling of UE-based TA measurements is fully missing for SpCell change by L3 mobility procedure

An LTM candidate can become the current SpCell also by legacy L3 mobility procedure (handover/PSCell change) while LTM is configured. When this occurs, the UE variable VarLTM-ServingCellUE-MeasuredTA-ID should be updated similarly and the condition to start/stop the UE-based TA measurements for the LTM candidates should be evaluated similarly as in LTM cell switch procedure. This procedural text is fully missing in 38.331.

Observation 8: Handling of the UE-based TA measurements is fully missing in TS 38.331 for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

A TP is not provided for this issue, but RAN2 is kindly requested to discuss if and how to capture this in 38.331.

Proposal 7: RAN2 to discuss if and how to capture handling of the UE-based TA measurements for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

### 2.4.6 TP for issues discussed in 2.4.2, 2.4.3, and 2.4.4

A TP and related inter-operability analysis for proposals 4, 5, and 6 can be found below.

---- Begin TP ----

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config except the LTM-Candidate IE indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

2>	else:

3>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

---- End TP ----

Inter-operability analysis:

- If the network implements the change and the UE does not, there is no interoperability issue since the UE-based TA measurements is a procedure which the UE locally executes.

- If the UE implements the change the network does not, there is no interoperability issue since the UE-based TA measurements is a procedure which the UE locally executes.

# 3. Conclusion

In this paper, we provide following observations and proposals:

Observation 1: According to to 38.214 (legacy rule), if only one TCI state is indicated in RRC configuration, it is activated without the need for further MAC-CE/DCI for TCI activation.

Observation 2: It is not clear whether the UE needs to report extra beams for SpCell in addition to the non-SpCell beams, if LTM-CSI-ResourceConfig does not include SpCell resources while spCellInclusion is set to True.

Observation 3: The UE LTM configuration does not necessarily contain unifiedTCI-StateType for an LTM candidate.

Observation 4: The UE behaviour is unspecified if unifiedTCI-StateType is absent but TCI states are configured for an LTM candidate with LTM-TCI-Info.

Observation 5: The UE-based TA measurements are not applicable to an LTM candidate which is the current SpCell.

Observation 6: The UE-based TA measurements are not stopped for the LTM candidate which becomes the current SpCell by LTM cell switch procedure.

Observation 7: The UE-based TA measurements are started for the LTM candidate which becomes the current SpCell by LTM cell switch procedure.

Observation 8: Handling of the UE-based TA measurements is fully missing in TS 38.331 for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

Proposal 1: RAN2 to clarify that if there is only one TCI state in ltm-CandidateConfig for the candidate cell indicated by LTM Cell Switch Command MAC CE, the network ensures this TCI state is identical with the TCI state indicated by LTM Cell Switch Command MAC CE.

Proposal 2: In the field description of spcellInclusion, clarify that it cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig.

Proposal 3: In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

Proposal 4: In clause 5.3.5.18.6, correct the procedural text so that the UE stops the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

Proposal 5: In clause 5.3.5.18.6, correct the procedural text so that the UE does not start the UE-based TA measurements for the LTM candidate which becomes the current SpCell.

Proposal 6: In clause 5.3.5.18.6, remove the unnecessary procedural text to stop the UE-based TA measurements when the LTM candidate which becomes the current SpCell does not have field ltm-UE-MeasuredTA-ID.

Proposal 7: RAN2 to discuss if and how to capture handling of the UE-based TA measurements for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured.

In addition, we provide TPs for proposals 2, 3, 4, 5, and 6.

# 4. References

[1]  R2_126 feMob mIAB Johan Notes EOM

[2]  3GPP TS 38.331 V18.2.0

[3]  3GPP TS 38.214 V18.3.0