3GPP TSG-RAN WG2 Meeting #127	R2-2406340

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	8.1.4

Source:	NEC

Title:	Remaining issues for Data Collection for UE side Model training

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting, the table on the analysis of different data collection options for UE-side model training was agreed. And there was a post meeting email discussion to discuss how to capture the corresponding TP into TR38.843.

This paper intends to discuss the remaining issues (i.e. FFS) within that table to finalize the discussion for the data collection of UE side model training.

# Discussion

## CP/UP tunnel for Data Transfer

Within the table (as shown in Annex) that compares different options for data collection, it is still FFS on the UP tunnel for data transfer for both option 2 (server inside the CN) and option 3 (server inside the OAM domain).

For option 2, it was agreed that the CP tunnel can be used to support data transfer for the case where the data volume remains within the NAS signalling capacity. Logically the UP channel is terminated between the UE and the CN, so then UP tunnel based data transfer may be possible. Meanwhile, we did not see any relevant discussions at SA2 in the context of Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (FS_AIML_CN) as led by SA2 for this issue at its previous meetings. If RAN2 aims to resolve the FFS on this matter, it is suggested to send an liaison to SA2 to make a query.

Proposal-1: Send an LS to SA2 to check the feasibility of UP channel based data collection of UE sided model training for option 2.

For option 3, it was agreed that the CP tunnel can be used to support data transfer for the case where the data volume remains within the RRC signalling capacity. However, we do not think there is a UP channel existing between the UE and the OAM domain server. The UP channel between the UE and the gNB was not specified so far. Hence, we do not think there is a direct option to support UP channel based data transfer for option 3. It is proposed to simply remove the FFS for option-3 and stick to the RRC layer CP signalling based data transfer.

Proposal-2: For option-3 (server inside the OAM domain), remove the FFS on UP channel based data transfer.

## Protocol layer for Data Transfer

Within the table (as shown in Annex) that compares different options for data collection, it is still FFS on the Protocol layer for data transfer when UP tunnel is adopted for data transfer for both option 2 (server inside the CN) and option 3 (server inside the OAM domain), as this is a relevant issue for the discussion in section 2.1.

For option 2, in case of UP channel based approach, the UE can collect the training data and transfers it to the CN via data radio bearer, and then the CN may forward the data to the a specific server within the 3GPP network domain. In our understanding, the AIML training data can be seen as a type of normal data, so then there is no point to specify a protocol layer to host such data transfer. However, as proposed in Proposal-1, we anyway will send a LS to SA2, RAN2 can also enquire SA2 for their understanding on this.

Proposal-3: Confirm with SA2 if the AIML training data can be seen as normal data if transferred over UP channel between the UE and CN.

For option 3, we do not think it is practical to support UP channel based data transfer between the UE and OAM domain server. Or otherwise, it could be handled by network implementations. Hence we do not see a need to specify a protocol layer for UP channel based data transfer for option-3. The FFS for option-3 can be simply removed.

Proposal-4: The FFS on protocol layer for UP channel based data transfer for option-3 can be simply removed, as the UP channel based data transfer between the UE and OAM domain server is not possible.

## Controllability on data transfer

Within the table (as shown in Annex), for both option 2 (server inside the CN) and option 3 (server inside the OAM domain), there is full controllability at the network. In this case, the MNO has the capability to manage data transfer to the server for UE-sided model data collection, which includes initiating, terminating, and fully managing data transfer. However for option 1b (OTT server within 3GPP network), it is still FFS on the level of controllability. Our understanding is that option 1b is based on the implementation of EVEX model as discussed at SA2. The EVEX server can control the data transfer according to its management of the corresponding PDU session. Meanwhile, as the EVEX server is still part of 3GPP network, basically, the 3GPP network can still have full controllability on the initiating, terminating, and fully managing of the data transfer for UE-sided model data collection. From this perspective, there is no difference between option 1b and option 2(server inside the CN).

Proposal-5: In option 1b, there is full controllability at the network to manage data transfer for UE-sided model data collection.

## Recommendations for RAN#105

In the current study of the data collection for training UE-sided model, a number of options were identified and the corresponding analysis was performed by RAN2. We do not believe it is practical to specify the data collection mechanisms based on multiple options identified simultaneously at Rel-19 for training the UE sided model, since otherwise the specification work would be too complicated. With this consideration in mind, we think it is the right time to down-select the options and eventually RAN2 should just specify the data collection mechanism with one of the options. RAN2 can send the identified options and its corresponding analysis to RAN plenary for down-selection.

Proposal-6: RAN2 to send the identified options and its corresponding analysis to RAN plenary to down-select only one of the options to support the data collection for the purpose of training UE-sided model.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Send an LS to SA2 to check the feasibility of UP channel based data collection of UE sided model training for option 2.

Proposal-2: For option-3 (server inside the OAM domain), remove the FFS on UP channel based data transfer.

Proposal-3: Confirm with SA2 if the AIML training data can be seen as normal data if transferred over UP channel between the UE and CN.

Proposal-4: The FFS on protocol layer for UP channel based data transfer for option-3 can be simply removed, as the UP channel based data transfer between the UE and OAM domain server is not possible..

Proposal-5: In option 1b, there is full controllability at the network to manage data transfer for UE-sided model data collection.

Proposal-6: RAN2 to send the identified options and its corresponding analysis to RAN plenary to down-select only one of the options to support the data collection for the purpose of training UE-sided model.

# Reference

- RAN2#126 meeting minutes

- [POST126][034][AI/ML PHY] TP for data collection

# Annex: Analysis of different data collection options for UE-side model training (TP to TR38.843)

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN (e.g., LMF) | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | NAS procedure FFS: impact to other layers | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |