3GPP TSG-RAN WG2 Meeting #127	R2-2406341

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	8.2.4

Source:	NEC

Title:	Random Access for Ambient IoT device

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting, the following agreements were reached for the random access procedure for Ambient IoT device:

Agreement for 4 step RA

- A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

- A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

Agreement for 2 step RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement for CFRA

- From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

This paper intends to discuss the further details of the procedures required for random access procedure for Ambient IoT device.

# Discussion

## Drawbacks of Slotted ALOHA access for RFID

For RFID, to start an inventory round for the tags in the field, the reader needs to send a Query command as the first step. The anti-collision dynamic framed slotted ALOHA algorithm is realized by specifying a value ranging from 0-15 to the 4-bit Q field in the Query command. Upon receiving the Query command, the matching tags pick a random number in [0, 2exp(Q)-1] to load into the slot counter.

If a tag, in response to the Query command, loads its slot counter with zero initially or after counting down the original random number selected, it responds with a 16-bit random number (RN16), which contains a random number of e.g. 16 bits for contention resolution, in the corresponding “RACH-slot” to the reader. The tag collision occurs when multiple tags in the reader field select the same slot and thus respond to the Query command simultaneously. A form of collision control can be realized by specifying a large value for Q to reduce the probability of collisions, in the extreme case when Q equals 16 the probability for an N tag collision is (1/65536) exp (N-1).

Considering the similarity between Ambient IoT system and RFID, it may be worthwhile to review the basic  access procedure adopted by RFID tag (ISO 18000-6c).

On one hand, RFID MSG-1 design enables quite low probability that multiple devices select the same 16-bit random number for a given Query. On the other hand, it is not designed to be robust against miss-detection/false-alarm due to noise/interference. Note that the miss-detection still occurs even when 1 bit out from 16 bits random number is erroneously detected.

However, the selection of the value of Q of the slotted-ALOHA access mechanism is very tricky.

- Too small Q(e.g. 1) will largely increase the contention among the devices. When a small Q is selected and indicated by the reader, multiple devices will generate the same random value and transmit their random numbers in the same access slot. When multiple random numbers are transmitted to the reader simultaneously at the same time slot, the transmission collisions will happen and the reader cannot distinguish the signal due to interference. This will lead to the waste of system resource and also the device’s energy.

- Too large Q (e.g. 16) will reduce the contention but the random occasion may be not sufficiently used within one Access Round, since the counting may take a long time to count down to zero before the Device can initiate its access attempt, which causes the access latency. With large Q, it may introduce many blank access slots which may not be not chosen by the device to transmit their random numbers. In the meantime, when the device’s countdown of the random number is not reached, the devices that have not finished the access procedure will continuously monitor the command from the reader (e.g. QueryRep) in order to identify an accurate the start of an access slot and maintain its clock tracking. This will also lead to the waste of system resource and also the device’s energy.

In addition, the Q value based approach represents the random number range with a power factor of 2, which puts some unnecessary restrictions for the slot counting.

Observation-1: the value of Q of the slotted-ALOHA access mechanism has many restrictions.

It should be noted that the AIoT system may host a huge number of devices and a robust access mechanism should be defined to handle the devices with big volume and to handle the potential contention. Then we think RAN2 needs to study an alternative access mechanism which have a better performance than the Q value based slotted-ALOHA access mechanism.

Proposal-1: RAN2 to study an alternative access mechanism which should have a better performance than the Q value based slotted-ALOHA access mechanism.

## RACH resource selection for Ambient IoT device

In order to select a time slot to respond to the reader via random access procedure, the AIoT device needs to know the exact time slots that can be used to initiate its random access. In addition, according to the discussion at RAN1, the FDM approach was also discussed on top of the TDM based access. FDM based approach may be used to afford large number of AIoT device access, which allows different devices to access the reader via a different frequency bandwidth at the same time slot. Based on this background, we discuss below what type of parameters the reader should announce before the AIoT device can initiate its random access.

The initial triggering message sent by the reader should include the time slots allocated for RA (e.g. M slots) within one frame or one access round and also the set of frequency location for each time slot. Assuming intended devices information (i.e., filter criteria from CN) is provided in the initial trigger message, each  device matching the filter criteria may generate a random number according to the number of the available time slots (e.g. M) in the initial triggering message for random access. The random number should be in the range [0, (M-1)]. It should be noted that here we do not intend to use Q value based slotted ALOHA due to its drawbacks discussed in the previous section.

Proposal-2: Each AIoT device matching the filter criteria generates a random number according to the number of the available time slots in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Based on the random number generated, the AIoT device may start “RACH-slot” counting down since receiving “initial triggering message” command.

For RFID, the slotted ALOHA access mechanism highly relies on the run of “QUERYREP” command, which allows the tag to count down when receiving such command at every slot. If AIoT device uses the similar approach, the AIoT device may need to count down whenever receiving a “Repeat” command from the reader following the initial triggering message. The “Repeat” command may be used by the device to maintain the synchronization or clock tracking. It should be noted that the run of “Repeat” command is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device. RAN2/RAN1 needs to study if we adopt the similar approach for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

Observation-2: the run of “QUERYREP” command (for RFID) is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device.

Proposal-3: RAN2/RAN1 needs to study if we adopt the similar approach (as “QUERYREP” command for RFID) for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

In any case, the abovementioned mechanism can only handle the RA slot selection but not the frequency location selection. So then in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Proposal-4: in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Meanwhile, it is very important to note that when there are frequency domain resources for each RA slot, RAN2 may need to study additional mechanisms or enhancements for AIoT device access, since anyway the slotted ALOHA mechanism can not be simply reused any more.

As discussed above, an RO in A-IoT can be defined as a combination of a time slot and a frequency location for each time slot (if FDM access is supported). RAN2#126 agreed that “AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).”, therefore it is nature to assume that the total available ROs (for example, M slot and N frequency locations) is indicated by AIoT paging message.

Currently, both 2-step and 3-steps RA are supported in A-IoT, on the association between available ROs and 2-step and/or 3-steps RA, we have following options:

Option 1: available ROs are associated to 2-step RA only;

Option 2: available ROs are associated to 3-step RA only;

Option 3: available ROs are associated to both 2-setp and 3-steps

On how to indicate the association between available ROs and 2-step and/or 3-steps RAs, we see following two alternatives:

Alternative 1: AIoT paging message explicitly indicates association between available ROs and 2-setp and/or 3-steps ROs;

Alternative 2: Pre-define association between available ROs and 2-step and/or 3-steps RA in the specification.

Since Alternative 1 is much more flexible, we propose to support Alternative 1 in A-IoT system.

Proposal-5: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

## Contention free access for Ambient IoT device

As agreed from last RAN2 meeting, RAN2 will study both single and multi-device case based contention-free access procedure from reader perspective.

Single device based contention-free access procedure

In case of single device based contention-free access procedure, we think that the reader may use either contention based resources or dedicated access resources to support a single AIoT device access. For both cases, the reader may include, in the initial triggering message, a single device ID, or a targeted device group which contains only a single device.

For single device access, in terms of contention based resource allocation framework, the reader should indicate, in the initial triggering message, that the device is expected to perform contention free access with a contention-free resource, this means the AIoT device should not run slot-ALOHA based procedure to randomly select the access resource for its access or it just selects Zero as the random number for slot counting during slot-ALOHA based procedure. Within the contention based access framework, the initial triggering message may carry a single random access resource(e.g. with a single slot with a single frequency location) for the device’s access. If a single random access resource is carried by the initial triggering message, the single device should select that resource for its access. Alternatively, the initial triggering message may carry multiple random access resources, since it is within contention based resource allocation framework, usually there are multiple random access resources. If multiple random access resource are carried by the initial triggering message, the AIoT device may select the earliest available slot for its access. Our preference is to just provide a single RA occasion/resource for the single device to carry contention free based access.

For single device access, in terms of dedicated contention free based resource allocation framework, it is similar to the multiple device access case as discussed below.

Multiple device based contention-free access procedure

In case of multiple device based contention-free access procedure, we think that the reader should allocate dedicated access resources to support AIoT device’s access and direct multiple AIoT devices to perform contention free access procedure. The reader can include a set of device identities and further indicate each random access resource for each device, with slot number and frequency location number for the current frame or access around.

As discussed during the TP discussion ([POST126][021][AIoT] TP), during contention free access, the device may enable one step based access and response, since no contention resolution is needed. In the message sent to the reader from the device, the device may carry its Device ID for message verification and also the signalling response  (e.g. following a command from the higher layer) to the reader. Meanwhile, RAN2 should also study the possibility for the device to carry some of the available data within the D2R message for the one step access based on the contention free resource. Although it was discussed during TP discussion before RAN2#127, RAN2 can confirm this understanding to avoid any ambiguity.

Based on the discussion above, we have the following proposals:

Proposal-6: The intention to enable contention free access should be indicated within the initial triggering message from the reader to the AIoT device.

Proposal-7: For single device access, the contention based resource allocation framework may be used to support contention free access with only one single AIoT device selected for access.

Proposal-8: For single device access, with the contention based resource allocation framework, the resource for contention free access may only include one single RA occasion/resource.

Proposal-9: For multiple device contention free access, one to one resource mapping should be indicated within the initial triggering message from the reader to the AIoT device.

Proposal-10: RAN2 to confirm that the device can carry the available data within the D2R message for the one step access based on the contention free resource.

## Contention Failure handling Ambient IoT device Access

In NR, there is mature mechanism specified for handling the contention failure for UE random access due to the fact that the contention failure may occur often. Basically the UE re-access can be based on backoff approach. Even though the contention failure for Ambient IoT device Access maybe not that often through proper parameter setting, but as discussed in section 2.1, there are some side effects if a too large Q value is announced by the reader. Meanwhile, the channel error probability for A-IoT transmissions would be higher than of typical NR transmission due to device’s limited capabilities. So then Contention Failure handling is still needed for Ambient IoT device Access.

Similar to the case of NR, the reader may not be able to successfully decode the initial transmission from Ambient IoT device due to D2R link interference, or the reader can only decode the transmission from only one device but not the other device’s. Then there will be some Ambient IoT devices experiencing contention failure. The question is how the Ambient IoT device handle such Contention Failure.

Following the similar discussion in RAN1, a timer of time window may be defined for the Ambient IoT devices to determine if it need to keep monitoring the response from the reader after its initial message transmission over D2R link.

With the slotted ALOHA based mechanism, the reader may provide the access resources (e.g., access slots) to the Ambient IoT devices for a batch of access occasions, which can be called one Access Round. Within one access round, the Ambient IoT device may experience contention failure during its access attempt. The question is how the Ambient IoT device try another access occasion after its contention failure. It should be based on the current access round or after the current access round (e.g. next access round).

Proposal-11: RAN2 should study the possibility for the device to try to access via another access occasion within the current access round or after the current access round after initial access contention failure.

In addition, we did not see the need to apply the backoff mechanism of NR to the Ambient IoT device.

One possible solution is to introduce the second trigger message within an access round triggered by the initial trigger message. Such a second trigger message only allows the following devices to respond:

- matching the filter criteria provided in the initial trigger message and

- experiencing contention failure during its access attempt responding to the initial trigger message

Proposal-12: Introduce second trigger message following the initial trigger message within the same access round.

To determine whether the device is allowed to respond to the second trigger message, the device needs to acknowledge its status, i.e., whether it experienced contention failure during access attempt responding to the initial trigger message. Such an status acknowledge can be supported upon receiving subsequent R2D transmission of “ACK/NACK” indication after D2R transmission (Msg.1 and Msg.3). Since Msg.2 R2D transmission can be treated as a subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.1, we propose to support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

Proposal-13: Support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

# Conclusion and Proposal

We have the following observations and proposals:

Observation-1: the value of Q of the slotted-ALOHA access mechanism has many restrictions.

Observation-2: the run of “QUERYREP” command (for RFID) is a big signaling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device.

Proposal-1: RAN2 to study an alternative access mechanism which should have a better performance than the Q value based slotted-ALOHA access mechanism.

Proposal-2: Each AIoT device matching the filter criteria generates a random number according to the number of the available time slots in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Proposal-3: RAN2/RAN1 needs to study if we adopt the similar approach (as “QUERYREP” command for RFID) for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

Proposal-4: in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Proposal-5: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

Proposal-6: The intention to enable contention free access should be indicated within the initial triggering message from the reader to the AIoT device.

Proposal-7: For single device access, the contention based resource allocation framework may be used to support contention free access with only one single AIoT device selected for access.

Proposal-8: For single device access, with the contention based resource allocation framework, the resource for contention free access may only include one single RA occasion/resource.

Proposal-9: For multiple device contention free access, one to one resource mapping should be indicated within the initial triggering message from the reader to the AIoT device.

Proposal-10: RAN2 to confirm that the device can carry the available data within the D2R message for the one step access based on the contention free resource.

Proposal-11: RAN2 should study the possibility for the device to try to access via another access occasion within the current access round or after the current access round after initial access contention failure.

Proposal-12: Introduce second trigger message following the initial trigger message within the same access round.

Proposal-13: Support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

# Reference

- RAN2#126 meeting minutes

- TS38.300