3GPP TSG-RAN WG2 Meeting #127	R2-2406342

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	8.11.2

Source:	NEC

Title:	Random Access for SBFD Operation

Document for:	Discussion, Decision

# Introduction

In the last RAN1 meeting, the following agreements were reached for the random access procedure for SBFD operation:

Part of Agreements made by RAN1#117 for Rel-19 SBFD

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

- For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

This paper intends to discuss the RAN2 impact of the random access procedures for SBFD operation.

# Discussion

## RACH configuration

According to the discussion of RAN1, two options were agreed for the RACH configurations for SBFD operation:

- RACH configuration Option 1 (i.e., use one single RACH configuration with possible enhancement) is agreed by RAN1 to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option.

- RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) was agreed by RAN1 to support random access operation for SBFD-aware UEs in RRC CONNECTED state.

For RACH configuration Option 1, RAN1 will consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols following the agreements reached during RAN1#116bis.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

  - FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

It should be highlighted that, the currently RAN1 agreement for both option 1 and option 2 of RACH configuration are applicable to SBFD-aware UEs in RRC CONNECTED state. Meanwhile, as can be seen, RAN1 also agreed to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode and WID revision [3] was approved at RAN#104 accordingly. Then our first proposal is to extend the applicability of the agreed RACH configuration options to SBFD-aware UEs in RRC_IDLE/INACTIVE mode as well.

Proposal-1: Extend the applicability of the agreed RACH configuration options (i.e., both option-1 and option-2) to SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

Then one thing to discuss is how to make such RACH configurations for SBFD-aware UEs (both RRC CONNECTED UEs and UEs in RRC_IDLE/INACTIVE). For legacy UEs in NR, the general RACH configuration is provided in SIB1. The legacy UEs can read the RACH configuration via system information acquisition during RRC_IDLE/INACTIVE state or via dedicated RRC message in RRC_ CONNECTED mode. Currently, the configuration of additional RACH resources is available for some specific purpose, for example, for the support of the access for IAB-MT, which was introduced during Rel-16.

Hence, for the support of the option-2 based RACH configuration for SBFD-aware UEs, the additional RACH configuration should be provided in system information. Following the legacy design, the straightforward way is to provide the additional RACH configuration (i.e., for option-2) via SIB1. Considering that having an additional set of parameters may cause the overhead for SIB1, the alternative way is to specify a new SIB to host this additional RACH configuration.

For the dedicated RRC message (e.g. RRCReconfiguration) based approach, it is still for further study, since the current SBFD configurations allow cross-slot random access in Option 2 according to RAN1 discussion, the UE basically needs to acquire all of the additional RACH configurations before its access. With this said, the UE may not be able to acquire such information before it is connected.

Proposal-2: The additional RACH configuration (i.e., for option-2) is provided in SIB1 or a new SIB to support the SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

Proposal-3: FFS if the additional RACH configuration (i.e., for option-2) can be provided via RRCReconfiguration message to support the SBFD-aware UEs in RRC CONNECTED mode.

In addition, when there is additional RACH configuration for SBFD-aware UEs, it would be beneficial to allow the deactivation and activation of all or a subset of such ROs. Usually less number of ROs configured in the system will improve the system throughput, since the additional resources can be used schedule the data transmission for the connected UEs. RAN2 can discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured. RRC message or MAC CE based approach may be discussed in this context. Such message may also indicate the time duration for which the deactivation is applicable for. For example, this can be in form of DRX cycle configured by gNB where within the DRX-on duration (when gNB is in active state), gNB enables all ROs and for DRX-off duration (when gNB is in low power state), gNB can configure a set of ROs to be deactivated.

Proposal-4: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

## RA-RNTI Calculation Confusion

Specific to the option 2 based additional RACH configuration for the SBFD-aware UEs. One distinguishing issue to how to avoid RA-RNTI confusion due to two sets of RACH resources.

It is well known that the RA-RNTI is used by UE to receive Random Access Response (RAR) once PRACH is transmitted. Different RACH occasions within a system frame are associated with different RA-RNTI values to ensure that RAR for a given PRACH transmission can be properly addressed to only the UE which has used that given RACH occasion. If RA-RNTI values are same for 2 RACH occasions, then there is a chance of confusion where 2 UEs which have used different RACH occasions act on the same RAR message.

To ensure abovementioned uniqueness, according to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

When additional RACH resources are configured to UE (especially when FDMed with legacy RACH resources or additional RACH resources starting from non-SBFD symbol) then there is a chance that RA-RNTI confusion may occur as one legacy RACH resource and an additional RACH resource have get the same RA-RNTI value.

Additional RACH resources starting from non-SBFD symbol are still under RAN1 discussion. Meanwhile, the following RAN1#117 agreement implies that additional RACH resources staring from non-SBFD symbol and ending in SBFD symbols is not supported. :

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

On the other hand, based on the following agreements made by RAN1#116bis meeting, if Alt2-4 is agreed at RAN1#117, then the additional ROs in non-SBFD symbols configured by additional RACH configuration will be supported for SBFD-aware UEs:

Agreement (@RAN1#116bis)

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives:

Alt 2-3:

The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

Alt 2-4:

The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

When additional RACH resources do not share the same time domain resource as a legacy RACH resource then the RA-RNTI confusion may not occur. Hence, one simple solution is to ensure that additional RACH resources are always TDMed with legacy RACH resources.

However the abovementioned TDMed configuration may put a hard restriction to the UE side behavior, provided that the UE cannot expect to receive a configuration of additional RACH resource where the starting symbol of the RO of additional RACH resources is not the same as the starting symbol of any RO of legacy RACH resource set. Therefore, in some cases, it may be beneficial to perform FDM of additional RACH resources and legacy RACH resource to compress the PRACH occasion occurrences in time domain.

Proposal-5: Both TDM and FDM based configuration between additional RACH resource set for SBFD and legacy RACH resource set are allowed.

To resolve the potential RA-RNTI confusion, the solutions should be studied by RAN2. During RA-RNTI calculation, the formula may be updated to also consider whether the RACH resource belongs to legacy RACH resources or additional RACH resources. For example, in the formula, the factor of RACH configuration can be added following UL Carrier ID as below:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2 × RACH_config

Here, RACH_config is 0 for the case of legacy RACH resource and has a non-zero value for additional RACH resource set for SBFD.

Proposal-6: RAN2 to discuss the method to resolve the potential RA-RNTI calculation confusion when additional RACH resource set for SBFD is configured for the SBFD-aware UEs.

An alternative way to resolve the potential RA-RNTI confusion can be to allocate different radio resources for the reception of RAR for the UEs. For example, a different PDCCH search space can be used for receiving RAR corresponding to additional RACH resource set, which is subject to RAN1 discussion.

## RACH resource selection and switch

Based on the legacy NR mechanism, the UE always selects the RACH resource set corresponding to the feature (e.g. IAB, Redcap, etc) applicable to the random access procedure. However, for SBFD operation,  one of the main motivation of introducing additional RACH resources is to reduce RACH latency, and it should be possible for the UE to select the earliest RACH occasion among the additional RACH resources and legacy RACH resources. In addition, for SBFD operation, it may not always be a good approach for the UE to select the SBFD based RACH resources as they are expected to have lower coverage than legacy RACH resources. Hence, it should be possible for UE to select legacy RACH resources based on its radio conditions.

The network may be able to command the UE to perform random access via PDCCH order, and the UE can select the RACH resource between legacy and additional RACH resources following the recommendation from the network within such PDCCH order. However, the detailed PDCCH order design in this case is subject to RAN1 discussion.

However, there are also cases where UE is required to select the RACH resources autonomously among the RACH resources configured to it, for example, when the UE loses the synchronization during RRC CONNECTED mode. It would be helpful to specify a criteria for the UE to choose the suitable RACH resource.

During the random access procedure, the UE may sweep the beams and measure their SSBs/CSI RSs in order to determine the best-associated beam with the best RSRP performance. In the meantime, in order to avoid random access delay, it may be desirable for the SBFD-aware UEs to always select the first available RO among all of the RACH resources valid to the UE (including both legacy RACH resources and additional RACH resources). In this manner, the UE is expected to always select the best suitable RO set or RO type (legacy RO or additional RO) to initiate random access.

Proposal-7: RAN2 to discuss the mechanism for the SBFD-aware UEs to select the RACH resource between legacy and additional RACH resources, e.g. the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.”

There is one more issue for the RACH resource decision. In general, the RSRP threshold for SSB/CSI-RS selection can be the same for both legacy RACH resources and additional RACH resources.

However, in practice, the network may configure the UE with a different threshold for SSB/CSI-RS signal strength (e.g. RSRP) or threshold for the path loss that UE experiences for SSB/CSI-RS for RACH resources occurring during SBFD symbols.

Proposal-8: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

In order to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, RACH resource selection for SI request and RA-SDT should also be discussed. The same discussion for proposal 7 can be applied for RACH resource selection for SI request and RA-SDT. Whether the network can configure separated criteria for SI request and RA-SDT is FFS.

Proposal-9: RAN2 to discuss the mechanism for the SBFD-aware UEs to select the RACH resource for SI request and RA-SDT between legacy and additional RACH resources, e.g. the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.”

Additional RACH resources are expected to be configured in SBFD symbols and hence they are likely to experience higher interference due to self-interference of gNB or inter-gNB CLI. Hence, it is important to also consider that UE can switch to legacy RACH resources if serving cell’s signal strength is not good. So, we may need a way for SBFD-aware UEs to switch between additional RACH resources and legacy RACH resources based on the serving cell signal strength.

Proposal-10: RAN2 to discuss if the UE can switch its usage of RO between additional RACH resources and legacy RACH resources based on the serving cell signal strength.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Extend the applicability of the agreed RACH configuration options (i.e., both option-1 and option-2) to SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

Proposal-2: The additional RACH configuration (i.e., for option-2) is provided in SIB1 or a new SIB to support the SBFD-aware UEs in RRC_IDLE/INACTIVE mode.

Proposal-3: FFS if the additional RACH configuration (i.e., for option-2) can be provided via RRCReconfiguration message to support the SBFD-aware UEs in RRC CONNECTED mode.

Proposal-4: RAN2 to discuss the method on how to deactivate and activate additional RACH Occasions for SBFD-aware UEs after it is configured.

Proposal-5: Both TDM and FDM based configuration between additional RACH resource set for SBFD and legacy RACH resource set are allowed.

Proposal-6: RAN2 to discuss the method to resolve the potential RA-RNTI calculation confusion when additional RACH resource set for SBFD is configured for the SBFD-aware UEs.

Proposal-7: RAN2 to discuss the mechanism for the SBFD-aware UEs to select the RACH resource between legacy and additional RACH resources, e.g. the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.”

Proposal-8: RAN2 to discuss if RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

Proposal-9: RAN2 to discuss the mechanism for the SBFD-aware UEs to select the RACH resource for SI request and RA-SDT between legacy and additional RACH resources, e.g. the UE always selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE.”

Proposal-10: RAN2 to discuss if the UE can switch its usage of RO between additional RACH resources and legacy RACH resources based on the serving cell signal strength.

# Reference

- RAN1#117 meeting minutes

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- TS38.321