3GPP TSG-RAN WG2 Meeting #127	R2-2406343

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	8.3.4.1

Source:	NEC

Title:	Simulation assumption for RLF/HOF prediction

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting (i.e. RAN2#126), the following agreement was reached during the discussion for RLF predictions use case, for the study on AI/ML for mobility.

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

This paper intends to discuss the simulation assumptions and evaluation methodology for RLF predictions.

# Discussion

## RLF/HOF Modelling

As described in the WID, simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839 and the detail discussion can leave to RAN2.

Following the guidance of the WID, basically, the start point of the simulation assumption and methodology for AI based mobility is already available, which is in TR36.839. It should be noted that TR36.839 was developed when the HetNet Mobility was studied in 3GPP for HetNet scenarios at Release-11 for LTE.

In TR36.839, the Handover Failure Modelling was introduced and the following definitions were captured:

- Definition of Handover states

- RLF modelling and definition of RLF states

- Handover/PDCCH failure modelling

The parameters for determine the RLFs and the PDCCH failures include:

- Qout

- Qin

- T310

- N310

- T311

- N311

We think that the same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

Proposal-1: The same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

## Simulation assumptions consideration for RLF failure prediction

The simulation assumptions discussed in [POST125bis][021][AI/ML mobility ] Simulation assumptions and methodology and [POST126][031][AIMob] Simulations mainly applies to RRM measurement prediction use case. However, we believe some of the simulation assumption should be common to other use cases (e.g., RLF failure prediction use case). Hence we think the agreements reached for the simulation assumptions methodology for RRM measurement prediction use case should be used as the baseline for RLF/HOF failure prediction use case.

In the meantime, there may be some specific issues to clarify for RLF failure prediction use case. At first, the current discussion in RRM measurement prediction use case assumes a simple UE trajectory. For RLF failure, we may adopt a bit more complicated deployment scenario (with more cells) and design a UE trajectory to easily occur RLF. From the UE speed perspective, medium to high UE speed may be prioritized for the simulation to provide a more likely RLF environment.

Proposal-2: Design specific deployment scenario, UE trajectory and UE speed for RLF failure prediction use case.

## RLF/HOF scenarios for evaluation

When evaluating the RLF/HOF predictions, we suggest to classify different handover scenarios. The performance of the RLF/HOF predictions for these scenarios may be observed separately. During the discussion of mobility robustness optimization, the following handover failure cases were investigated (see also the detailed analysis of these scenarios in our companion paper R2-2405075, submitted to agenda item 8.3.4.2):

- Too late HO

  - an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- Too early HO

  - an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- HO to wrong cell

  - an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

The above listed handover failure cases are combinations of handover failures and RLF, so then it would be beneficial to evaluate all these types of HO failures as target scenarios for RLF/HOF prediction study.

Proposal-3: RAN2 to evaluate the following types of HO failures as target scenarios for RLF/HOF prediction study:

- Too late HO

- Too early HO

- HO to wrong cell

## RLF prediction result

In the last meeting, it was agreed that the RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. And it is FFS on expected RLF time and indirect case. However based on the RLF prediction, more prediction result should be possible.

As discussed in section 2.3, there are multiple type of handover failure cases, which are non suitable handovers from the serving cell to target cell(s) for the UE. Logically, there may be RLF in the middle of Handover process or after the completion of the handover. It would be beneficial for the UE to predict such occurrence of non suitable handover as one AIML output. Meanwhile, we think correspondingly, the UE should predict a suggested handover as a replacement of the non suitable handover, which can be seen an  alternative to the above non suitable handover.

Proposal-4: RLF prediction result should include the occurrence of non suitable handover and the replacement of that non suitable handover.

In addition, the UE is likely to just experience failed link (e.g. following RLC failure) and lose the connection with the network. As a result of the AIML inference, the UE should be able to suggest one candidate cell for the UE to switch to avoid such connection loss.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: The same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

Proposal-2: Design specific deployment scenario, UE trajectory and UE speed for RLF failure prediction use case.

Proposal-3: RAN2 to evaluate the following types of HO failures as target scenarios for RLF/HOF prediction study:

- Too late HO

- Too early HO

- HO to wrong cell

Proposal-4: RLF prediction result should include the occurrence of non suitable handover and the replacement of that non suitable handover.

# Reference

- RP-234055 (AI)Machine Learning (ML) for mobility

- TR36.839

- TR38.838