3GPP TSG-RAN WG2 Meeting #127						R2-2406346

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.3

Source:	Samsung

Title:	On-demand SIB1

Document for:	Discussion and decision

# Introduction

RAN2 discussed procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode in RAN2#126 and following agreements were made:

Further clarification

1. Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

2. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

3. Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

4. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

5. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

6. If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

7. After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

8. RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

9. NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

10. How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

11. If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

12. After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

14. As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

15. Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

16. RAN2 to wait for RAN1’s progress whether to support scenario 3.

# Discussion

2.1 Scenarios

RAN2 has agreed to at least study the scenario (shown in figure 1) where, UE obtains the SIB1 request configuration of Cell B from Cell A, UE sends SIB1 request to Cell B based on the SIB1 request configuration and acquires SIB1 of Cell B from Cell B upon sending the SIB1 request.

Figure 1

Instead of sending the SIB1 request to Cell B, SIB1 of Cell B can be directly obtained from Cell A as shown in Figure 2. Cell A can provide SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A). UE can request for SIB X using the legacy SI request mechanism. This does not require introduction of new SIB1 request configuration and SIB1 request.

Figure 2

This was discussed in RAN2#126 and postponed. Some questions/concerns were raised during the discussion. These are further addressed below for better understanding of the proposal:

Question/comment 1: This scenario requires coordination between two cells in order to guarantee the latest SIB1.

- Yes, the coordination is needed between the cells. Note that co-ordination is also needed for agreed scenario 1a as SIB1 request configuration includes cell specific parameters which can be updated.

Question/comment 2: Once the UE gets SIB1 from cell A and camps on NES cell, if the UE requires OD SIB1 again for the camped NES cell, will the UE back to cell A and request it again?

- No.

  - Once camped on the NES cell, the SIB1 of that NES cell will be maintained by SIB change notification as in legacy. If SIB1 is changed, UE will receive SI change notification and will reacquire SIB1 without any SIB1 request. The assumption is that upon sending SI change notification, network will broadcast SIB1.

  - In the scenario where SIB1 change does not trigger SI change notification, e.g. change in broadcaststatus bit for on demand OSI, UE may have to reacquire SIB1 (Note that in legacy, for on demand OSI, UE needs to check status of broadcast status bit in SIB1 acquired in current modification period before requesting OSI). In this case UE will send SIB1 request to NES Cell using the SIB1 request configuration signalled by the NES Cell.

Question/comment 3: UE gets SIB1 of NES cell from Cell A while UE is camped on Cell A. Before the UE camps on NES cell, acquired SIB1 may become obsolete if SIB1 of NES cell is updated.

- UE requests SIB1 of NES Cell from Cell A after NES cell becomes target candidate cell. Once SIB1 is acquired, UE performs cell reselection. So acquired SIB1 is not obsolete as it is acquired just before cell reselection.

Proposal 1: RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

2.2. Further details of Scenario 1a (i.e. scenario agreed in RAN2#125bis)

Timing for SIB1 request configuration acquisition

While the UE is camped on Cell A, at some point in time, UE needs to acquire SIB1 request configuration for Cell B from Cell A.

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires after it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

Option 2 seems better in terms of latency to reselect/camp on Cell B.

Proposal 2: RAN2 to discuss and agree on one of the following for acquiring SIB1 request configuration of Cell B from Cell A:

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires after it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

Dynamic switching between periodic broadcast and on demand

UE has acquired the SIB1 request configuration of Cell B from Cell A. Cell B is the highest ranked cell for cell reselection. Before acquiring SIB1 of Cell B for reselection, does UE need to check if SIB1 is currently being broadcasted or provided on demand in Cell B. Note that it is possible that even though SIB1 request configuration of Cell B is received, Cell B may be currently broadcasting SIB1. In legacy, a cell supporting on demand SI request can dynamically switch between periodic broadcast and on demand SI transmission. UE checks the broadcast status bit to determine whether to transmit SI request or not.

Proposal 3: RAN2 to discuss whether a Cell for which SIB1 request configuration is available, can periodically broadcast SIB1 or not.

Proposal 4: If UE has SIB1 request configuration of a Cell, RAN2 to discuss whether UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

UL carrier for SIB1 request

Cell which supports on demand SIB1 can be configured with both SUL and NUL. So SIB1 request needs to be supported for both SUL and NUL. UE should select the UL carrier before transmitting SIB1 request.

Proposal 5: SIB 1 request is supported for both SUL and NUL.

SIB1 request configuration

In release 18, RACH configuration for early TA acquisition of neighboring cells was introduced. This can be used as baseline for RACH configuration for SIB1 request procedure. Early TA acquisition is based on PDCCH order and SSB/UL carrier is indicated in PDCCH order, rsrp-ThresholdSSB and rsrp-ThresholdSSB-SUL are not included in RACH configuration for early TA acquisition. However, these are needed for SIB1 request configuration for UE to select SSB/Ul carrier. SIB1 request configuration should atleast include the following:

RACH parameters:

- FrequencyInfoUL

- rach-ConfigGeneric

- bwp-GenericParameters

- ssb-PerRACH-Occasion

- prach-RootSequenceIndex

- prach-SubcarrierSpacing

- n-TimingAdvanceOffset

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- ra-SearchSpace

SIB1 request resources (similar to legacy SI request)

  - sib1-RequestPeriod

  - ra-PreambleStartIndex

  - ra-AssociationPeriodIndex

  - ra-ssb-OccasionMaskIndex

Proposal 6: SIB1 request configuration include SIB1 request resources (similar to legacy SI request): sib1-RequestPeriod, ra-PreambleStartIndex, ra-AssociationPeriodIndex and ra-ssb-OccasionMaskIndex.

Msg1 repetitions for SIB1 request

Msg1 repetitions is supported for UL coverage in R18 for random access (including Msg1 based SI request). RACH based SIB1 request is agreed for on-demand SIB1. So, it is reasonable to support Msg1 repetitions for SIB1 request.

Proposal 7: Msg1 repetitions is supported for SIB1 request.

SIB1 acquisition after transmitting SIB1 request

Inorder to acquire SIB1, UE needs to know the search space 0, coreset 0 and Kssb. In a cell not broadcasting SIB1, Kssb in MIB indicates that SIB1 is not broadcasted in cell and search space 0 and coreset 0 are not signalled in MIB. One of the following options can be considered to obtain Kssb, search space 0, coreset 0 to acquire SIB1.

Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

Option 3: Upon transmitting SIB1 request ack, network updates MIB and include Kssb, search space 0, coreset 0 for SIB1 acquisition. UE reacquires MIB.

Option 3 is not efficient and may delay SIB1 acquisition.

Proposal 8: RAN2 to discuss and agree on one of the following options for acquiring SIB1

  - Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

  - Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

SIB1 Validity

If SIB1 is provided on-demand in a cell, UE acquires SIB1 upon transmitting SIB1 request. If SIB1 is periodically broadcasted in a cell, UE acquires SIB1 periodically broadcast transmission without transmitting SIB1 request. Once the UE acquires SIB1 in a cell (with or without SIB1 request), UE reacquires SIB1 in that cell according to conditions specified in current specification. No additional (new) condition/limitation/validity checks are needed.

As per current spec

- UE acquires SIB1 when it camps to cell (cell selection or cell reselection)

- UE will reacquire SIB1 upon receiving SI change notification. If SIB1 is changed, UE will receive SI change notification and will reacquire SIB1 without any SIB1 request. The assumption is that upon sending SI change notification, network will broadcast SIB1.

- If on-demand OSI is supported in the cell, UE will reacquire SIB1 (to check broadcaststatus bit in SIB1) in current modification period irrespective of whether UE has previously acquired SIB1 in cell or not.

Proposal 9: No additional (new) condition/limitation/validity checks are needed for reacquiring SIB1 in camped cell. UE reacquires SIB1 in the camped cell according to conditions specified in current specification.

SIB1 request configuration transmission by NES cell

In RAN2#126, RAN2 postponed the discussion on whether UE can acquire WUS configuration from the NES cell, after the UE camps on NES cell.

When camping to an NES cell, UE acquire SIB1 of the cell. Later, UE may have to reacquire SIB1. The previously acquired SIB1 request configuration for the NES cell from another cell, may be not valid at the time UE needs to reacquire SIB1. Once UE camps to NES cell, it may obtain/maintain the latest SIB1 request configuration of that cell from SIB of that cell.

Proposal 10: After the UE camps on NES cell, UE can acquire SIB1 request configuration from the NES cell.

2.3 Handling legacy UEs

On-demand SIB1 affects legacy UEs. Legacy UEs (i.e. UEs not supporting on-demand SIB1) cannot camp on cell providing SIB1 on-demand (i.e. not broadcasting SIB1). Legacy UEs can bar the cell based on a) cellBarred bit set to barred in MIB or b) based on no SIB1 indication via ssb-SubcarrierOffset in MIB or c) based on failure to acquire SIB1.

    - a) will require additional barring bit to bar the on-demand SIB1 capable UE

    - b) will require additional configuration to signal RMSI (SIB1) occasions to receive SIB1 upon sending on demand SIB1 request. For example, search space zero and coreset zero would need to be signalled in SIB1 request configuration.

    - c) delays the barring in case SIB1 is not broadcasted.

Network can also include cells supporting on demand SIB1 to list of excluded cells. This could be more beneficial then barring as legacy UEs can avoid PBCH/MIB decoding and measurements of cells supporting on demand SIB1.

Proposal 11: RAN2 to discuss how to handle Legacy UEs (i.e. UEs not supporting on-demand SIB1).

  - Option 1: Legacy UEs bar the on-demand SIB1 cell based on cellBarred bit set to barred in MIB. NES UE ignore the cellBarred bit. Additional barring bit in SIB1 to bar the on-demand SIB1 capable UE

  - Option 2: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

  - Option 3: Network includes cells supporting on demand SIB1 to list of excluded cells

# Conclusion

Proposal 1: RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

Proposal 2: RAN2 to discuss and agree on one of the following for acquiring SIB1 request configuration of Cell B from Cell A:

- Option 1: UE acquires when UE intends to perform cell reselection to Cell B

- Option 2: UE acquires when it camps to Cell A and reacquires the configuration, if updated (based on SI change notification).

Proposal 3: RAN2 to discuss whether a Cell for which SIB1 request configuration is available, can periodically broadcast SIB1 or not.

Proposal 4: If UE has SIB1 request configuration of a Cell, RAN2 to discuss whether UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Proposal 5: SIB 1 request is supported for both SUL and NUL.

Proposal 6: SIB1 request configuration include SIB1 request resources (similar to legacy SI request): sib1-RequestPeriod, ra-PreambleStartIndex, ra-AssociationPeriodIndex and ra-ssb-OccasionMaskIndex.

Proposal 7: SIB1 request configuration include SIB1 request resources (similar to legacy SI request): sib1-RequestPeriod, ra-PreambleStartIndex, ra-AssociationPeriodIndex and ra-ssb-OccasionMaskIndex.

Proposal 8: RAN2 to discuss and agree on one of the following options for acquiring SIB1

  - Option 1: Kssb, search space 0, coreset 0 is signalled in SIB1 request configuration

  - Option 2: Kssb, search space 0, coreset 0 is signalled in SIB1 request ack i.e. RAR

Proposal 9: No additional (new) condition/limitation/validity checks are needed for reacquiring SIB1 in camped cell. UE reacquires SIB1 in the camped cell according to conditions specified in current specification.

Proposal 10: After the UE camps on NES cell, UE can acquire SIB1 request configuration from the NES cell.

Proposal 11: RAN2 to discuss how to handle Legacy UEs (i.e. UEs not supporting on-demand SIB1).

  - Option 1: Legacy UEs bar the on-demand SIB1 cell based on cellBarred bit set to barred in MIB. NES UE ignore the cellBarred bit. Additional barring bit in SIB1 to bar the on-demand SIB1 capable UE

  - Option 2: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

  - Option 3: Network includes cells supporting on demand SIB1 to list of excluded cells

# References