3GPP TSG-RAN WG2 Meeting #127						R2-2406347

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.2

Source:	Samsung

Title:	On-demand SSB SCell Operation

Document for:	Discussion and Decision

# Introduction

The Rel-18 work item on network energy savings for NR specified techniques primarily for RRC Connected UEs such as SSB-less SCell operation, cell DTX/DRX mechanism, mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, CHO procedure enhancement(s) etc.

Rel-19 work item aims to specify further network energy savings techniques such as on-demand SSB for SCell operation. WID of Rel-19 network energy saving specifies the following objective for on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

This contribution discusses the details on supporting on-demand SSB on SCell.

# Discussion

In the legacy NR operation, a UE can be configured with SSB based RRM measurement to discover SCell(s). UE reports the RRM measurement results to the gNB on the quality of the measured SSB. The gNB can further provide the SCell configuration by RRC to the UE. At the time of configuration, SCell can be initially activated or deactivated. If the SCell is deactivated, the gNB can activate it by SCell activation command by MAC CE. UE uses SCell’s SSB (or TRS if configured) for AGC and synchronization purpose to be ready for transmitting or receiving data on the activated SCell. After the SCell is activated, the UE can still use the SSB to get re-synchronized with the SCell, by its implementation.

An example of the legacy NR operation of a SCell is shown in Figure 1, wherein the SSB is transmitted by the SCell and is periodic. In case of SSB-less SCell, operation is same except that SSB measurements of a reference serving cell (SpCell or another SCell) are used for the SSB-less SCell.

Figure 1 Illustration of legacy NR procedure for the SCell transmitting SSB

The periodic transmission of SSB for the SCell operation affects the network energy consumption. For network energy savings following are agreed by RAN1 for SSB transmission on SCell:

  - Case #1: No always-on SSB on the cell. Upon on demand SSB trigger, SSBs are periodically transmitted.

  - Case #2: SSBs are periodically transmitted in SSB burst/window. On demand SSB can be additionally provided during the interval between SSB burst/window as and when needed. Note that periodic transmission can be at a longer periodicity to reduce energy consumption but affects performance ((re)sync/AGC delay, etc). Additional on demand SSB can improve performance.

Scenarios for on demand SSB

RAN1 has agreed that on demand SSB can be triggered for the following scenarios:

- Scenario 2: SCell is configured to a UE but before the UE receives SCell activation command

- Scenario 2A: When UE receives SCell activation command

Following scenarios for triggering on demand SSB are FFSs in RAN1

- Scenario 3A: After UE receives SCell activation command until SCell activation is completed

- Scenario 3B: When or after SCell activation is completed and SCell is activated

RAN1 has also agreed that for a cell supporting on-demand SSB SCell operation, L1 and/or L3 measurement based on on-demand SSB is supported for the cell. It’s not clear whether on-demand SSB is used for RRM measurements for only case #1 or both case #1 and case #2.

For case #1 there is no periodic SSB, so on-demand SSB can be used for RRM measurements after network starts to transmit these SSBs. For case #2 periodic SSB is always transmitted and there seems no need to use on-demand SSB for RRM measurements in case #2. If on-demand SSB can be used for RRM measurements, one of a) and b) can be considered, a) UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE use periodic SSB; b) UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Currently, for L3 measurement (i.e. RRM), SCell can be configured with a measurement object for always on periodic SSB. For supporting L3 measurement (i.e. RRM) based on on-demand SSB, a new measurement object for on demand SSB can be configured for SCell or measurement object including on-demand SSB information and/or including always on periodic SSB information can be configured. If measurement object for SCell include on demand SSB information or is associated with on demand SSB information, UE perform L3 measurement (i.e. RRM) using on-demand SSB when these are transmitted.

Proposal 1: RAN2 to discuss whether on-demand SSB measurements for L3 measurement (i.e. RRM) are used for

  - Option 1: Case #1 only

  - Option 2: both Case #1 and Case #2

Signalling for on demand SSB:

RAN1#117 Agreements

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Based on the above RAN1 agreements following options are possible:

  - Option 1:

    - RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

    - RRC message indicates on-demand SSB transmsission

    - This option can be used for scenario 2. It can also be used for scenario 2A (e.g. when SCell is activated by RRC)

  - Option 2:

    - RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

    - MAC CE indicates on-demand SSB transmsission from certain time after the reception of this MAC CE.

      - 2A: MAC CE which supports both on-demand SSB indication and SCell activation command

      - 2B: MAC CE which supports only on-demand SSB indication

    - This option can be used for scenario 2. It can also be used for scenario 2A (e.g. when SCell is activated by MAC CE)

Option 1 needs to be supported in spec. It is needed at atleast for the case SCell is activated by RRC. For option 2, question is whether to support 2A or 2B or both in specification.

- Option 1 + option 2A

  - MAC CE cannot be used for triggering on demand SSB transmsission in scenario 2. It can only be used for scenario 2A.

  - MAC CE for SCell activation is impacted.

- Option 1 + option 2B

  - Same MAC CE can be used for triggering on demand SSB transmsission in scenario 2 and Scenario 2A.

  - No impact to MAC CE for SCell activation.

- Option 1 + option 2A + option 2B

  - Different MAC CE is used for triggering on demand SSB transmsission in scenario 2 and Scenario 2A

  - MAC CE for SCell activation is impacted.

In our view, option 2B is simple and does not impact legacy SCell activation MAC CE.

Proposal 2: RAN2 to agree the following for RRC based on demand SSB transmission

- RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

- RRC message indicates on-demand SSB transmsission

Proposal 3: RAN2 to agree the following for MAC CE based on demand SSB transmission

- RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

- New MAC CE indicates on-demand SSB transmsission from certain time after the reception of this MAC CE. Legacy MAC CE is used for SCell activation.

# Conclusion

Proposal 1: RAN2 to discuss whether on-demand SSB measurements for L3 measurement (i.e. RRM) are used for

  - Option 1: Case #1 only

  - Option 2: both Case #1 and Case #2

Proposal 2: RAN2 to agree the following for RRC based on demand SSB transmission

  - RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

  - RRC message indicates on-demand SSB transmsission

Proposal 3: RAN2 to agree the following for MAC CE based on demand SSB transmission

  - RRC message includes on-demand SSB configuration (Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst and Periodicity of the on-demand SSB)

  - New MAC CE indicates on-demand SSB transmsission from certain time after the reception of this MAC CE. Legacy MAC CE is used for SCell activation.

# References