3GPP TSG-RAN WG2 Meeting #127	R2-2406352

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda item:		8.8.4 Support of broadcast service

Title:		Signalling for the support of MBS broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this paper, we would like to provide our views on the highlighted aspects on support of broadcast service in NTN.

# 2. Discussion

ETRI believes that significant progress was made in supporting MBS broadcast services in NTN during the 125bis and 126 meetings of RAN WG2. The remaining issues are as follows:

- How to define the details of MBS broadcast intended service areas (e.g. Circles, geographical area information)

- How to provide the details of MBS broadcast intended service areas to UE with the following possibilities: SIB20/SIB21/MBSBroadcastConfiguration.

- How to associate the service area information with MBS sessions.

- How to define UE behaviour when the UE receives signalling for the MBS broadcast intended service area configuration.

- Whether or not there are any impacts on the legacy MBS service continuity mechanism when the MBS broadcast intended service area concept is introduced.

2.1	Detailed formats for MBS broadcast intended service area

During the 125bis meeting of RAN WG2, it was agreed that at least the following geographical area formats for modelling MBS broadcast service areas in NTN can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Regarding this, RAN2 had already engaged in similar discussions when a new system information block (SIB25) was introduced for signalling TN coverage in Release 18 NTN. A circle-based format can reduce signalling overhead, while a geographical area information-based format can express more accurate service areas. Based on their respective pros and cons in terms of accuracy and signalling overhead, it was concluded in Release 18 NTN that SIB25 provides the corresponding geographical area information via a list of (possibly overlapping) areas, where each area is defined using centre location coordinates and radius. Therefore, ETRI believes that a similar approach using a circle-based format can easily be applied to detailed formats for MBS broadcast intended service areas without duplicating discussions, which would be time-consuming and hinder efficient progress. The approach of employing a list of (possibly overlapping) circles as a geographical area format for MBS broadcast intended service areas offers an efficient and flexible means to represent various shapes of intended service areas by NW implementations.

To express a more accurate service area, it is necessary that each circle in the list for an MBS broadcast intended service area can be designated as either a permitted area or a prohibited area by implementation.

Observation 1: The same circle-based format in SIB25 from Rel-18 NTN can be applied for MBS broadcast intended service areas.

Proposal 1: For MBS broadcast intended service areas, RAN2 to define a list of (possibly overlapping) circular areas, where each circle is defined by its centre location coordinates and radius. Each circle to be indicated as either a permitted area or a prohibited area

Figure 1. Configuration example of geographical area information

2.2	Association between MBS broadcast intended service areas and MBS sessions

In our view, the MBS broadcast intended service area can be used for the following purposes:

- To support location-dependent broadcast services within a portion of NTN cell(s).

- To avoid unnecessary UE behaviour, including the reception of MBS-related SIs and/or MBS sessions outside the MBS broadcast intended service area.

Regarding the first purpose, the location-dependent MBS service enables the distribution of different content data to different MBS service areas. For the support of location-dependent broadcast services, similar to the legacy MBS mechanism, the MBS broadcast intended service area ID, which consists of a reference location and radius within a portion of NTN cell(s), can be used in combination with the MBS session ID to uniquely identify the service area-specific part of the MBS service within 5GC. Therefore, ETRI believes that for association with MBS sessions, MBS broadcast intended service areas should be associated with MBS session IDs with the following possibilities:

- Option 1: one MBS broadcast intended service area ID mapped to several corresponding MBS session IDs

- Option 2: several corresponding intended service area IDs mapped to one MBS session ID.

Considering flexibility for the configuration of independent intended service area per each MBS session, in our view, Option 2 is preferred.

Observation 2: The MBS broadcast intended service area can be used for the following purposes:

- To support location dependent broadcast services within a portion of NTN cell(s).

- To avoid unnecessary UE behaviour including the reception of MBS related SIs and/or MBS sessions outside an intended service area.

Observation 3: To support location-dependent broadcast services, similarly to the legacy MBS mechanism, the MBS intended service area ID can be used in combination with the MBS session ID to uniquely identify the service area-specific part of the MBS service within 5GC.

Proposal 2: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.

Therefore, to indicate the details of the MBS broadcast intended service area associated with the MBS session ID to the UE, the following information needs to be provided via one or more SIBs:

- The list of MBS session IDs

- The list of corresponding MBS broadcast intended service area IDs per each MBS session ID

- The service area details per each MBS intended service area ID (reference point and radius)

Proposal 3: A UE can be configured with one or more SIB messages including the following information:

- The list of MBS session IDs

- The list of corresponding intended service area IDs per each MBS session ID

- The service area details per each MBS intended service area ID (reference point and radius)

Figure 2. Association between MBS broadcast intended service area and MBS session

2.3	Candidate SIB containing the details of MBS broadcast intended service areas

During the 126th meeting of RAN WG2, it was agreed that for MBS broadcast, the following possibilities for including the service area information should be considered: SIB20, SIB21, or MBSBroadcastConfiguration, containing the following information:

- SIB20: Information required to acquire the MCCH/MTCH configuration for MBS broadcast

- SIB21: The mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI.

- MBSBroadcastConfiguration: Control information applicable for MBS broadcast services transmitted via broadcast MRB

When the above information is provided via SIB20 or SIB21, UE behaviour can be optimized to avoid unnecessary actions. For example, if the information is provided via SIB20 and the UE is located outside the MBS broadcast intended service area, the UE can avoid receiving unnecessary MBSBroadcastConfiguration messages. Conversely, when the information is provided via the MBSBroadcastConfiguration message, signalling overhead for MBS broadcast intended service areas can be reduced because the MBS session ID information in the legacy MBSBroadcastConfiguration message can be reused to indicate the MBS broadcast intended service area. It is noted that legacy SIB20 and SIB21 do not include any MBS session ID information. Additionally, considering the legacy purposes of each SIB, ETRI believes that providing MBS intended service area information via the MBSBroadcastConfiguration message would be more appropriate, as the intended service area can be regarded as part of the control information for providing area-specific MBS broadcast services for each MBS session.

Observation 4: When the information indicating the details of the MBS broadcast intended service area is provided via SIB20 or SIB21, the UE may avoid unnecessary behaviour. Conversely, if the information is provided via the MBSBroadcastConfiguration message, the MBS session ID information in the legacy MBSBroadcastConfiguration message can be reused to indicate the MBS broadcast intended service area.

Proposal 4: UE can be configured with MBSBroadcastingConfiguration message including at least the following information.

- The list of MBS session IDs

- The list of corresponding intended service area IDs per each MBS session ID

By the way, it may be challenging to include all information indicating the MBS broadcast intended service area within the MBSBroadcastConfiguration message due to its inherent size limitations. In this case, alternative approaches can be considered, such as defining a new SIB or modifying the existing USD in the following ways:

- Modification of MBSBroadcastConfiguration to include:

  - The list of MBS session IDs

  - The list of corresponding intended service area IDs per each MBS session ID

- Definition of a new SIB or modification of USD to include:

  - The list of intended service area IDs

  - The service area details per each MBS intended service area ID (reference point and radius)

Proposal 5: UE can be configured with SIB-X (MBSBroadcastingConfiguration/USD/a new SIB) including the following information

- The service area details per each MBS intended service area ID (reference point and radius)

2.4	UE behaviour

Some location-dependent MBS services may need to be accessed only within allowed areas due to various MBS service frameworks (e.g., regulatory, billing) across different cities and/or countries. In TN systems, MBS service accessibility can be controlled by identifying an accessible cell list or tracking area lists without imposing restrictions on UE behaviour regarding the reception of the service. However, since location-dependent MBS service areas are within a portion of NTN cells, the legacy mechanism may not work effectively in NTN networks unless enhancements such as network-verified UE location and the assignment of service area-specific Group IDs are introduced. Therefore, ETRI believes that an effective way to manage service accessibility based on the geographical location of the UE is to define the following new UE behaviour for the reception of MBS services:

- Upon receiving the MBS broadcast service intended service area information, the UE should be restricted from accessing the respective services if it is outside the intended service area, regardless of whether it possesses a valid identifier for receiving those services.

Figure 4. UE behaviour for support of broadcast service in NTN

Observation 5: Legacy MBS service accessibility control via accessible cell list or tracking area list does not fit in NTN, since the service areas may exist within only a portion of NTN cell(s).

Proposal 6: RAN to define the following new UE behaviour for the reception of MBS services in NTN when the UE is located outside the intended service area:

- Upon receiving the geographical area information, the UE should be restricted from accessing the respective services if it is outside the designated service area, regardless of whether it possesses a valid identifier for receiving those services.

2.5	Service Continuity

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

- 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

- 2) Either:

  - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

  - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

  - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

Considering that SIB21 contains the mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI, this legacy mechanism for MBS service continuity assumes that the MBS service area is mapped onto one or more cells. However, in NTN systems, this may not work effectively because the UE may not consider that frequency as the highest priority if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session that the UE is receiving or interested in, even if the above conditions 1 and 2 are met.

Therefore, ETRI believes that enhancements to the legacy mechanism for MBS service continuity are necessary, including the following:

- To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs, and include the following information in SIB21:

  - Carrier frequency list (already existing in SIB21)

  - MBS intended service area ID list per each carrier frequency

- To update existing conditions for UE to perform MBS broadcast service cell reselection based on MBS broadcast intended service area information as followings.

  - If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the three following conditions are fulfilled:

    - 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

    - 2) Either:

      - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

      - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

      - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

    - 3) one or more MBS broadcast intended service area ID(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS broadcast intended service area ID(s) is also indicated for this MBS broadcast service session.

Observation 6: In NTN systems, the legacy mechanism for MBS service continuity may not work effectively because the UE may not prioritize a frequency if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session the UE is receiving or interested in, even if the two conditions of the legacy mechanism are fulfilled.

Proposal 7: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs, and include the following information in SIB21: MBS intended service area ID list per each carrier frequency

- To update the legacy MBS service continuity mechanism for UE to perform MBS broadcast service cell reselection based on MBS broadcast intended service area information.

# 3. Conclusions

Observation 1: The same circle-based format in SIB25 from Rel-18 NTN can be applied for MBS broadcast intended service areas.

Observation 2: The MBS broadcast intended service area can be used for the following purposes:

- To support location dependent broadcast services within a portion of NTN cell(s).

- To avoid unnecessary UE behaviour including the reception of MBS related SIs and/or MBS sessions outside an intended service area.

Observation 3: To support location-dependent broadcast services, similarly to the legacy MBS mechanism, the MBS intended service area ID can be used in combination with the MBS session ID to uniquely identify the service area-specific part of the MBS service within 5GC.

Observation 4: When the information indicating the details of the MBS broadcast intended service area is provided via SIB20 or SIB21, the UE may avoid unnecessary behaviour. Conversely, if the information is provided via the MBSBroadcastConfiguration message, the MBS session ID information in the legacy MBSBroadcastConfiguration message can be reused to indicate the MBS broadcast intended service area.

Observation 5: Legacy MBS service accessibility control via accessible cell list or tracking area list does not fit in NTN, since the service areas may exist within only a portion of NTN cell(s).

Observation 6: In NTN systems, the legacy mechanism for MBS service continuity may not work effectively because the UE may not prioritize a frequency if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session the UE is receiving or interested in, even if the two conditions of the legacy mechanism are fulfilled.

Proposal 1: For MBS broadcast intended service areas, RAN2 to define a list of (possibly overlapping) circular areas, where each circle is defined by its centre location coordinates and radius. Each circle to be indicated as either a permitted area or a prohibited area

Proposal 2: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.

Proposal 3: A UE can be configured with one or more SI messages including the following information:

- The list of MBS session IDs

- The list of corresponding intended service area IDs per each MBS session ID

- The service area details per each MBS intended service area ID (reference point and radius)

Proposal 4: UE can be configured with MBSBroadcastingConfiguration message including at least the following information.

- The list of MBS session IDs

- The list of corresponding intended service area IDs per each MBS session ID

Proposal 5: UE can be configured with SIB-X or USD including the following information

- The service area details per each MBS intended service area ID (reference point and radius)

Proposal 6: RAN to define the following new UE behaviour for the reception of MBS services in NTN when the UE is located outside the intended service area:

- Upon receiving the geographical area information, the UE should be restricted from accessing the respective services if it is outside the designated service area, regardless of whether it possesses a valid identifier for receiving those services.

Proposal 7: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs, and include the following information in SIB21: MBS intended service area ID list per each carrier frequency

- To update the legacy MBS service continuity mechanism for UE to perform MBS broadcast service cell reselection based on MBS broadcast intended service area information.