3GPP TSG-RAN WG2 Meeting #127	R2-2406355

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:	7.4.2

Source: 	MediaTek

Title: 	Leftover LTM UE capabilities issues

Document for:	Discussion and Decision

1. Introduction

In the last meeting RAN2 #126, RAN2 discussed the UE capabilities in RAN1/4 feature lists with granularity of per band pair per band combination, including the following capability:

- R1 45-5a RACH-based early TA acquisition with simultaneous transmission

- R4 39-4 Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

- R4 39-4a Interruption due to RF retuning for PDCCH- ordered RACH

- R4 39-5 RF/BB preparation time for PDCCH-order RACH

During the meeting, RAN2 agreed that the band pair should be the UE’s current band and the target band for RACH, which can be any support band without the current band combination.

RAN2 assumes that the target band for RACH transmission is any supported band within or outside the band combination. This can be revisited if RAN1 or RAN4 indicates otherwise in the future

Meanwhile, the signalling design is agreed to be per feature set, to avoid the massive signalling overhead for the combination for the bands in band entry and any possible target bands.

RAN2 pursues signalling solution where the target bands for RACH transmission are signalled per feature set, and further discuss how the target bands are indicated, by pointing to appliedFreqBandList.

In the latest TS38.331 V18.2.0, these capabilities can be found as below:

R1 45-5a:

FeatureSetUplink-v1800 ::= 							SEQUENCE {

…

-- R1 45-5a: RACH-based early TA acquisition with simultaneous transmission

rach-EarlyTA-BandList-r18    		 SEQUENCE (SIZE (1..maxBandsMRDC)) OF BOOLEAN      OPTIONAL,

R4 39-4, 39-4a, 39-5:

FeatureSetDownlink-v1800 ::= 							SEQUENCE {

…

pdcch-RACH-DL-InfoList-r18 				SEQUENCE (SIZE (1..maxBandsMRDC)) OF PDCCH-RACH-DL-Info-r18

OPTIONAL,

…

PDCCH-RACH-DL-Info-r18 ::=             CHOICE {

notSupported                          NULL,

supported                             SEQUENCE {

-- R4 39-4: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-AffectedBands-r18          ENUMERATED {noIntrruption, interruption},

-- R4 39-4a: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-SwitchingTimeList-r18      ENUMERATED {ms0, ms0dot25, ms0dot5 , ms1, ms2}                OPTIONAL,

-- R4 39-5: the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained

-- in any of UE's configured UL BWP(s) of active serving cells

pdcch-RACH-PrepTime-r18               ENUMERATED {ms1, ms3, ms5, ms10}                              OPTIONAL

}

}

The current signalling structure appears incomplete and still unclear how to indicate the target band of the band pairs under BC. Therefore, the signalling design for these UE capabilities are further discussed in this paper.

Meanwhile, some other issues are found in the latest LTM UE capability specification, and TPs are provided for corrections in this paper.

2. Discussion

2.1 Per band pair per band combination granularity

The feature set structure is agreed for these capabilities with per band pair per BC. One simple way is to add the corresponding signallings into FeatureSets in UECapabilityInformations. Given that feature set is already indicated as per band per band combination, the signalling in FeatureSets only need to cover the target band of the band pair (rather than the entire band pair of this BC). These target bands can then be individually paired with the band from the feature set to form different band pairs within this combination of the feature set. An example figure can be found as below:

Figure 1 Example for feature set to indicate UE capability with granularity of per band pair per band combination

Therefore, the target band list with the corresponding components for each target band for those capabilities should be included into FeatureSetDownlink/ FeatureSetUplink. For the Uplink feature R1 45-5a, this can be achieved using the following signalling structure:

FeatureSetUplink-v18xx ::= 							SEQUENCE {

…

-- R1 45-5a: RACH-based early TA acquisition with simultaneous transmission

rach-EarlyTA-BandList-r18 					SEQUENCE (SIZE (1.. maxBands)) OF	RACH-EarlyTA-TargetBand  OPTIONAL,

…

RACH-EarlyTA-TargetBand ::=                      SEQUENCE {

TargetbandNR               			  FreqBandIndicatorNR

}

For the other three R4 features R4 39-4, 39-4a, 39-5, the current signalling implements them as a unified structure (i.e., PDCCH-RACH-DL-Info-r18) in featureSetDownlink. However, this structure limits these three capabilities to share the same BC and serving band. This not only fails to reduce signalling overhead but may also cause additional signalling overhead due to the different BC and band pairs of the three features supported by the UE.

Therefore, we prefer separate structures for these three features, as in case of R1 45-5a. An example can be found below:

FeatureSetDownlink-v18xx ::=                         SEQUENCE {

…

-- R4 39-4: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-AffectedTargetBandsList             SEQUENCE (SIZE (1..maxBands)) OF PDCCH-RACH-AffectedTargetBand OPTIONAL,

-- R4 39-4a: Interruption due to RF retuning for PDCCH- ordered RACH

pdcch-RACH-TargetBandsRFSwitchingTimeList      SEQUENCE (SIZE (1..maxBands)) OF PDCCH-RACH-TargetBandSwitchingTime OPTIONAL,

-- R4 39-5: the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained

-- in any of UE’s configured UL BWP(s) of active serving cells

pdcch-RACH-TargetBandPrepTimeList              SEQUENCE (SIZE (1..maxBands)) OF PDCCH-RACH-TargetBandPrepTime      OPTIONAL

…

}

…

PDCCH-RACH-AffectedTargetBand ::=      			SEQUENCE {

TargetbandNR               			  FreqBandIndicatorNR

}

PDCCH-RACH-TargetBandSwitchingTime ::=      	SEQUENCE {

TargetbandNR               			FreqBandIndicatorNR

RACH-SwitchingTime					ENUMERATED {ms0, ms0dot25, ms0dot5 , ms1, ms2, notSupported}

}

PDCCH-RACH-TargetBandPrepTime ::=  				SEQUENCE {

TargetbandNR               			  FreqBandIndicatorNR

RACH-PrepTime							ENUMERATED {ms1, ms3, ms5, ms10, notSupported}

}

With the discussion above, we propose:

Proposal 1: For the LTM capabilities with the granularity of per band pair per band combination:

- Add the target band list into FeatureSetsDownlink/Uplink for each capability.

- Include the corresponding components for each target band within the target band list.

The ASN.1 structures as described above can be considered as a starting point.

The maximum sequence size for these components was previously denoted by maxSimultaneousBands, representing the maximum number of bands that could be aggregated simultaneously. However, with the shift to only present target bands within the current feature set, the scope of these target bands should align with the NR band range, i.e., maxBands=1024.

Proposal 2: maxBands is used for the maximum size of the target band list.

In the example above, the target band is indicated by FreqBandIndicatorNR, could present all band within the NR frequency band with the range of 1 to 1024. The size of target band list will still be huge if UE supports most of the target band within the current feature set.

In order to reduce the signalling overhead, the range of the target band could be further reduced by frequencyBandListFilter in UECapabilityEnquiry by network. The UE filters the target band list according to the candidate bands indicated in frequencyBandListFilter

Proposal 3: The target band lists of these capabilities are filtered in accordance with the potential candidate bands indicated in frequencyBandListFilter by network.

2.2 Clarification on R4 39-6

In the latest LTM UE capabilities, there is a capability ltm-FastProcessingConfig-r18 (R4 39-6) that provides UE granularity with FR1/2 differentiation. In legacy UE capability design, such capabilities are set to per band granularity with the description stating that the capability value remains consistent across all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands. This applies to the ltm-FastProcessingConfig-r18 capability as well.

However, this capability also includes parameters for the maximum number of stored (candidate and serving) cells and stored candidate configurations that the UE can support for fast processing. Given the per-band granularity, there is a potential for misunderstanding that these values are for the current band but not cross-band. (Even the value is consistent for all bands)

Therefore, we suggest providing further clarification in the description of this capability:

Proposal 4: Clarify that the capability value of ltm-FastProcessingConfig-r18 indicates the maximum number across all the supported bands, rather than the current band.

A TP can be found as below:

---- Begin TP ----

| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs that UE can support fast processing. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The capability value represents the maximum number across all the supported bands. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in section 6.3 in TS 38.133 [5]. | Band | No | N/A | No |
| --- | --- | --- | --- | --- |

---- End TP ----

Inter-operability analysis:

- If the network implements the change and the UE does not, the UE and the network might have different understanding of the value of maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18 .

- If the UE implements the change the network does not, the UE and the network might have different understanding of the value of maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

2.3 Correction on R4 39-7

In the latest RAN4 feature list, the feature group indexed as R4 39-7 is defined as "Faster UE processing time during LTM cell switch," which includes components for the reduced time values in FR1 to FR1, FR2 to FR2, and FR1/FR1 to FR2/FR1. This is designated as an optional feature group with capability signalling.

However, in the latest TS 38.331, this capability is implemented with a unified optional marker for these three components, which means that the feature in all three cases must be either supported or not supported.

Observation: R4 39-7 is implemented with a unified optional marker for three components, which is not aligned with the intention of this feature.

Therefore, we propose:

Proposal 5: Add separate optional markers for the three components of the feature R4 39-7 (ltm-FastUE-Processing).

A TP can be found as below:

---- Begin TP ----

MeasAndMobParametersCommon ::=          SEQUENCE {

…

-- R4 39-7: Faster UE processing time during cell switch

ltm-FastUE-Processing-r18                   SEQUENCE {

fr1-r18                                    ENUMERATED {ms10, ms15}		OPTIONAL,

fr2-r18                                    ENUMERATED {ms10, ms15}		OPTIONAL,

fr1-AndFR2-r18                             ENUMERATED {ms20, ms30}			OPTIONAL

}                                                                                OPTIONAL,

---- End TP ----

We would like to highlight that this is an NBC change. Given that the LTM UE capability is still incomplete in the latest version and many capabilities are still missing, the NBC change at this stage should be acceptable.

Inter-operability analysis:

- If the network implements the change and the UE does not, the network does not understand ltm-FastUE-Processing-r18 which the UE sends.

- If the UE implements the change and the network does not, the network might not understand the value indicated in ltm-FastUE-Processing-r18.

3. Conclusion

Per band pair per band combination granularity

Proposal 1: For the LTM capabilities with the granularity of per band pair per band combination:

- Add the target band list into FeatureSetsDownlink/Uplink for each capability.

- Include the corresponding components for each target band within the target band list.

The ASN.1 structures as described above can be considered as a starting point.

Proposal 2: maxBands is used for the maximum size of the target band list.

Proposal 3: The target band lists of these capabilities are filtered in accordance with the potential candidate bands indicated in frequencyBandListFilter by network.

Clarification on R4 39-6

Proposal 4: Clarify that the capability value of ltm-FastProcessingConfig-r18 indicates the maximum number across all the supported bands, rather than the current band.

Correction on R4 39-7

Proposal 5: Add separate OPTIONAL markers for the three components of the feature R4 39-7 (ltm-FastUE-Processing).

4. Reference

[1]. R2_126 feMob mIAB Johan Notes EOM

[2]. 3GPP TS 38.331 V18.2.0

[3]. R2-2404705 Band-pair signalling for Early TA acquisition UE capabilities, Qualcomm Incorporated

[4]. R2-2405245 Mobility UE capabilities with Per band pair per band combination granularity, Huawei, HiSilicon

[5]. R2-2405954 [AT126][502][R18MobE] UE cap Early TA acquisition (Qualcomm)

[6]. R1-2403703 Updated RAN1 UE features list for Rel-18 NR after RAN1_116bis

[7]. R4-2406680 Rel-18 RAN4 UE feature list for NR (version 4)