3GPP TSG-RAN WG2 Meeting #127	R2- 2406356

Maastricht, Netherlands, Aug 19th - Aug 23th, 2024

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Further discussion on Inter-CU LTM

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 discussed several aspects to support inter-CU LTM, including scenarios, RRC configuration and signalling procedures at different LTM stages.

This contribution further discusses RRC configuration and signalling aspects to support inter-CU LTM design. Given that SA3 are not yet in a position to provide feedback regarding security updates, security update methods are not covered in this paper.

Discussion

RRC configuration

Reference configuration

In the last meeting, RAN2 discussed the reference configuration issue on whether to introduce multiple reference configurations across different CU or to keep single reference configuration as Rel-18 LTM.

- Single reference configuration has simpler signalling structure, but it needs more signalling overhead when the candidate cells from very different CUs.

- On the other hand, multiple configurations per CU may reduce signalling overhead in inter-CU scenarios, but the effectiveness depends on network deployment. If more similarities can be found among configurations across CUs, forcing each CU to have a separate reference config also cause extra signalling overhead.

Consider that it is hard to say which solution is better in different scenarios, one simple way is to allow multiple configurations and let network implementation to config one or multiple reference configuration in inter-CU scenario (i.e., reference config per CU is not mandatory).

Proposal 1: Multiple reference configurations are introduced for inter-CU scenario. It is up to network implementation to config single or multiple reference configuration(s) cross CUs.

Considering that there are only a maximum of 8 candidate cells, having numerous reference configurations would have marginal benefit and may complicate the signalling design. We calculate the possible maximum number of full configurations under different cases and try to find a suitable maximum number of reference configs:

- Scenarios: 8 candidate cells across 1,2 or 3 CUs with the worst case (4+4 in 2 CUs and 3+3+2 in 3 CUs).

- Reference config number: allow at most 1,2 or 3 reference configuration(s).

The maximum full configuration number required in different scenarios with different reference configurations can be easily calculated and shown in the following table:

Table 1: Maximum full configuration for different CUs while allowing different reference configurations

| Maximum full config number↘ | 1 CU scenario | 2 CUs scenario | 3 CUs scenario |
| --- | --- | --- | --- |
| Allow 1 reference config | 1 | 5 | 6 |
| Allow 2 reference config | 1 | 2 | 4 |
| Allow 3 reference config | 1 | 2 | 3 |

As we can see from the table, 2 reference configs can evidently reduce the signalling overhead in 2 CU and 3 CU scenarios compared to a single reference configuration. However, 3 reference configs only offers a minimal reduction in full config overhead in a very specific scenario (8 candidate cells distributed across 3 CUs with a deployment of 3+3+2).

Observation 1: With 8 maximum candidate cells limitation, 2 reference configurations have clear benefit in inter-CU scenario, while the further benefit for 3 or more reference configurations is marginal.

Considering the insights gained from the table and the complexity on signalling design, we suggest limiting the maximum number of reference configurations to 2. Meanwhile, a similar UE capability can be introduced to indicate whether UE support multiple (double) reference configurations. If the reference configuration number is limit to 2, one bit indication is enough.

Proposal 2: The maximum reference configuration number is 2.

Proposal 3: A UE capability to indicate the support of multiple reference configurations is introduced.

Although multiple reference configurations may be introduced, it does not mean the coordination for inter-CU node can be omitted. As the last meeting agreement: “The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.” We still think one common CSI config is needed cross CU for L1 measurement. From UE point of view, UE follows the unique CSI resource configuration (cross CU) to select and measure the candidate RSs. Having multiple CSI resource configuration for different CUs may confuse and complicate UE's behaviour.

Since clarification has been made at inter-gNB perspective in the last meeting, a further step clarification can be made from the signalling perspective.

Proposal 4: One common LTM-CSI-resource-config-list is provided to UE, no matter there are single or multiple reference configuration(s) across CUs.

Mixture of inter/intra-CU LTMs

RAN2 has agreed the L2 reset (MAC reset, RLC and PDCP re-establishment) is performed upon inter-CU LTM execution. In the last meeting, RAN2 further discussed the design of no-reset ID, but no consensus was reached.

In Rel-18 LTM, an IE ltm-NoResetID is used to indicate whether to RLC re-establishment and PDCP recovery are required for inter-DU LTM. The ID from candidate cell and serving cell is compared to determine whether RLC re-establishment and PDCP recovery is needed.

For inter-CU LTM, LTM configuration may consist of candidate cells from different CUs (intra-CU and inter-CU). The reestablishPDCP in candidate configuration cannot be used to indicate PDCP re-establishment, as it does not support subsequent LTM. The Rel-18 no-reset-ID cannot be reused either, as there are cases where the UE only needs to re-establish RLC but not PDCP (e.g., inter-DU intra-CU case).

Therefore, a similar indication can be introduced for inter-CU cases to signify the need for PDCP re-establishment during inter-CU LTM. It's important to note that this ID does not convey any information about network deployment, since PDCP re-establishment may be indicated without a change in CU.

Proposal 5: An ID (similar to No-reset-ID) can be introduced in LTM-candidate IE to indicate whether to perform PDCP re-establishment. This ID does not indicate information about network deployment.

Besides, given that the PDCP re-establishment and security update may be decoupled from the network, and the decision to perform security update during LTM is pending SA3's response, RAN2 may need to postpone the discussion on whether to use the same PDCP no-reset ID to indicate security update after the method of security update is confirmed.

Proposal 6: Postpone the discussion on whether to use the same way for PDCP re-establishment to indicate security update until the method of security update is confirmed.

Signalling procedure

Stage-2 overall procedure

During the initial meeting for Rel-19 Mobility in RAN2, the components of the stage-2 signalling procedure were discussed but no consensus was reached. To facilitate progress in stage-2 running CR, we aim to briefly revisit this topic here.

Numerous contributions have presented their designs of stage-2 signalling overflow for inter-CU LTM. Some commonalities can be found among them:

- The source gNB and candidate gNB(s) are involved in the procedure.

- The interaction between source gNB and candidate gNB are reflected.

These assumptions can be taken as baselines for inter-CU LTM signalling procedure.

Meanwhile, some differences are found:

- Variation in gNB types, with mentions of 2 gNBs (source gNB + candidate gNB) or 3 gNBs (source gNB + target gNB + other candidate gNB(s)).

- Differences in CN involvement, including AMF, AMF + UPF, CN, or no CN.

Comparing the signalling procedure of legacy stage-2 signalling overflow for HO and CHO, it is essential to involve the core network to convey mobility control information. The AMF and UPF(s) should be separately introduced to reflect the transmission of control signalling and user data forwarding.

The gNB types should adopt the structure of CHO as the baseline, i.e., including the source gNB + target gNB + other candidate gNB(s). This is beneficial to compare the different behaviors of UE towards target cells and other candidate cells in the LTM execution stage, as well as providing a clear signalling flow for subsequent LTM when UE switches from the new source gNB to another gNB. However, the spilt of CU and DU should not be reflected in the stage-2 spec as it is more related to RAN3.

Additionally, it is recommended to unify the terminology, using "source gNB" for the initial gNB, “candidate/target gNB” for gNB indicated as target for LTM cell switch, “Other candidate gNB(s)” for additional candidate gNBs for LTM preparation, early sync or subsequent LTM. This is aligned with the design for the CHO signalling structure

Considering the discussion above, the proposal is made for the stage-2 overall procedure design:

Proposal 7: The following modification is made for the stage-2 overall procedure for inter-CU LTM:

- Source gNB, candidate/target gNB, other candidate gNB(s), AMF and UPF are involved

- The interaction between source gNB, candidate/target gNB and other candidate gNB(s) are reflected

- The interaction between gNBs to AMF and UPF are reflected.

LTM execution

For LTM execution triggering, it is a common understanding that the LTM cell switch decision should be determined at source gNB (we don’t identify CU and DU nodes in stage-2 spec), and the cell switch command MAC CE is also sent from the source gNB. Meanwhile, the interaction between the source gNB and target gNB should be reflected to show the signalling inaction and potential latency impact. This is also aligned with the current flowchart of legacy HO and CHO.

Proposal 8: The interaction between source and target gNB is reflected in LTM execution stage.

Regarding the order of triggering cell switch command MAC CE and interaction between the source gNB and target gNB, In Rel-18 LTM, RAN3 clarified that the serving DU may trigger the cell switch command first or inform the target DU before sending the cell switch command to the UE. RAN2 agreed that the former approach (serving DU triggering LTM command first and then informing the target DU) should be the baseline to reduce potential latency for LTM execution.

Observation 2: In Rel-18 intra-CU LTM, RAN2 confirm that serving DU triggering LTM command first and then informing the target DU is the baseline.

In an intra-CU scenario, the typical latency for the F1 interface is around 10ms for round trip time. This results in approximately 10ms from the source DU informing the target DU. The UE requires more than 10ms to perform RRC processing, target configuration application, etc. This time frame allows the network to prepare before the UE attaches to the target cell.

However, in an inter-CU scenario, it is worth discussing the latency for the source DU to inform the target DU for the initiation of the LTM command. The question that follows is, whether the network should inform the target DU before indicating the cell switch command to the UE. These steps could result in a longer latency from when the UE sends the MR to when it receives the cell switch command.

Typically, the latency for inter-CU communication is around 20ms for round trip time. If the latency for UE attaching to the target cell is shorter than 20ms (it depends on UE capability), the target cell may not be well-prepared and could lead to more interruptions. In such cases, the network may need to inform the target DU earlier to align with the UE's processing time.

Observation 3: The order of triggering cell switch command MAC CE and source gNB to inform target gNB may be changed due to the extra network latency in inter-CU scenario. The target cell may not be well-prepared when UE attach to the target cell, which may lead to unexpectable interruption.

Figure 1 Potential impact in Intra/inter CU network latency for LTM execution

Considering that UE have different capabilities on the latency during LTM execution (e.g., FG39-6, 39-7 to switch faster to target cell), network may adjust the order of sending CSC MAC CE to UE and indicating target gNB. This behaviour can be clarified and implemented by network.

Proposal 9: RAN2 assumes it is up to network implementation to adjust the order of sending CSC MAC CE to UE and indicating target gNB to always make sure target cell is prepared when UE completes the target configuration.

Initial discussion on inter-CU LTM with DC

According to the WID, the following NR-DC case is supported:

- When DC is configured with inter-CU LTM either in MN or in SN but not both at the same time: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

Inter-SN LTM

For Inter-SN LTM, several details require discussion. For example, the secondary key (S-KgNB) of SN is calculated and updated by the sk-counter indicated by RRC and the master key (KgNB) of the MN. In LTM, it is still not clear how to indicate and update the sk-counter value to update the SN security key.

Considering the subsequent LTM is also required for inter-CU LTM with DC, the inter-SN subsequent CPAC security update method in Rel-18 can be considered as a starting point.

Proposal 10: RAN2 to discuss the security key update for SCG inter-CU LTM. Specifically, RAN2 to discuss how to indicate the sk-counter for target SN to UE.

Inter-MN LTM

For the case that CU is acting as MN and SN is unchanged/released, it seems the security update mechanism of standalone inter-CU LTM should be taken as a baseline. However, if master security key changes, a secondary key change always follows, as the secondary key is derived from the master key. This process is executed through reconfigurationWithSync in legacy way.

Observation 4: If master security key changes, a secondary key change always follows, which is performed through reconfigurationWithSync in legacy way.

For MCG LTM in DC scenario, it is said in the WID that inter-CU LTM is configured either in MN or in SN but not both at the same time. But if reconfigurationWithSync is required for each Inter-MN LTM execution, it will weaken the benefit of LTM, especially when switching frequently by subsequent LTM.

Therefore, we suggest that RAN2 first clarify the following:

Proposal 11: RAN2 to clarify if the reconfigurationWithSync for SCG is allowed during Inter-MN LTM.

Conclusion

In this contribution, the following observations and proposals are made:

Reference configuration

Proposal 1: Multiple reference configurations are introduced for inter-CU scenario. It is up to network implementation to config single or multiple reference configurations cross CUs.

Observation 1: With 8 maximum candidate cells limitation, 2 reference configurations have clear benefit in inter-CU scenario, while the further benefit for 3 or more reference configurations is marginal.

Proposal 2: The maximum reference configuration number is 2.

Proposal 3: A UE capability to indicate the support of multiple reference configurations is introduced.

Proposal 4: One common LTM-CSI-resource-config-list is provided to UE, no matter there are single or multiple reference configuration across CUs.

Mixture of inter/intra-CU LTMs

Proposal 5: An ID (similar to No-reset-ID) can be introduced in LTM-candidate IE to indicate whether to perform PDCP re-establishment. This ID does not indicate information about network deployment.

Proposal 6: Postpone the discussion on whether to use the same way for PDCP re-establishment to indicate security update until the method of security update is confirmed.

Stage-2 overall procedure

Proposal 7: The following modification is made for the stage-2 overall procedure for inter-CU LTM:

- Source gNB, candidate/target gNB, other candidate gNB(s), AMF and UPF are involved

- The interaction between source gNB, candidate/target gNB and other candidate gNB(s) are reflected

The interaction between gNBs to AMF and UPF are reflected.

LTM execution

Proposal 8: The interaction between source and target gNB is reflected in LTM execution stage.

Observation 2: In Rel-18 intra-CU LTM, RAN2 confirm that serving DU triggering LTM command first and then informing the target DU is the baseline.

Observation 3: The order of triggering cell switch command MAC CE and source gNB to inform target gNB may be changed due to the extra network latency in inter-CU scenario. The target cell may not be well-prepared when UE attach to the target cell, which may lead to unexpectable interruption.

Proposal 9: RAN2 assumes it is up to network implementation to adjust the order of sending CSC MAC CE to UE and indicating target gNB to always make sure target cell is prepared when UE completes the target configuration.

Initial discussion on inter-CU LTM with DC

Proposal 10: RAN2 to discuss the security key update for SCG inter-CU LTM. Specifically, RAN2 to discuss how to indicate the sk-counter for target SN to UE.

Observation 4: If master security key changes, a secondary key change always follows, which is performed through reconfigurationWithSync in legacy way.

Proposal 11: RAN2 to clarify if the reconfigurationWithSync for SCG is allowed during Inter-MN LTM.

References

- R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements

- S3-242400 Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases