3GPP TSG-RAN WG2 Meeting #127	R2- 2406357

Maastricht, Netherlands, Aug 19th - Aug 23th, 2024

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Discussion on measurement and signalling design

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 discussed several aspects to support event triggered L1 measurement reporting for LTM, including use cases, filtering, triggering event, measurement RS, etc.

This contribution focuses the discussion on the measurement event and beam/cell level measurement issues, as well as the signalling design of event and resource configuration.

Discussion

Triggering events

In the last RAN2 meeting, several triggering events which similar to Ax events are agreed:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

However, the need of LTM1 (Beam of serving cell becomes better than absolute threshold) is still under discussion.

In L3 measurement report, the A1 event is defined as the serving cell's quality exceeds a certain absolute threshold. It is used to evaluate the QoS maintenance, to make sure that UE is in good radio link quality that meets the QoS requirements of the service. This also allows the network to get informed about load balancing and resource allocation information. Network can optimize the system performance and enhance user experience by dynamically adjusting resources to meet higher demand.

However, these functions are already existed in L3 measurement report configuration, which can still be configured during LTM if needed. For L1 measurement report, the purpose should only focus on the triggering of LTM, and the agreed events (especially LTM3 to LTM5) are more useful for triggering mobility.

Furthermore, the purpose of defining event triggered L1 measurement reports is to minimize the signalling overhead of periodic L1 measurement reporting. We should try to avoid unnecessary reporting when the quality of the serving cell remains good.

Proposal 1: The event LTM1 (Beam of serving cell becomes better than absolute threshold) is not needed for LTM.

Meanwhile, a possibly related discussion is whether to introduce s-measure like threshold to control the measurement behavior. In legacy L3 RRM, thresholds are introduced to control the non-serving cell measurement of UE. When the radio link quality of serving cell is good enough, measurement on non-serving cell is not needed.

However, the L1 MR is triggered by network together with LTM configurations to start LTM. For LTM mobility scenario, the radio link quality may not be too well and the L1 MR should be performed to avoid the miss of potential target cell which met the triggering event. Omitting L1 MR at this stage seems not very needed and may have negative impact on the performance of LTM.

Considering that this s-measure threshold was not introduced in L1 MR for Rel-18 LTM either, we suggest to not introduce this mechanism in Rel-19.

Proposal 2: The s-measure threshold is not needed for event triggered L1 MR.

Measurement result

In the last meeting, the beam or cell level measurement result was discussed for event triggered L1 MR and the beam level is considered as the baseline:

Agreement: For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

However, companies have discrepancies on cell level measurement, so it is postponed.

In the following agenda we made a further discussion on what beam to be measured and the clarification on beam level measurement, and make an further discussion on cell level measurement in 2.2.2.

Further discussion on beam level

Clarification on beam level

In the last meeting, the beam level measurement result is agreed, but the detailed behavior and definition are not discussed. In Rel-18 LTM, the UE choose the beams of candidate cells to be measured and report to network directly. It is up to network implementation to mix the information of different beams from one or more candidate cell and make the decision. However, for event triggered L1 MR, the triggering condition is initiated by UE, and it worth a discussion on the detailed measurement objects and measurement procedure.

Specifically, the optimal beam of candidate cells may change with time by the UE mobility and channel variation. The measurement for one report may span different measurement periods, and the measured beam index may change with time, which is up to UE implementation. For our understanding, the intention of the last meeting to agree beam level measurement was not to limit to measure a fix beam within an event triggering window.

Observation 1: The optimal beam of candidate cells may change with time by the UE mobility and channel variation.

In other word, if only one beam is measured within an MP for one cell, and the beam index changes with different MP, this is also referred to as beam-level measurement. The distinction between beam and cell level measurement should be whether multiple beams are measured in the same measurement period and whether beam consolidation is performed.

Proposal 3: Single beam measurement with changing beam index should be considered as beam level measurement.

What beam to be measured?

In the current L3 RRM, the UE is not required to measure specific beams of serving or neighboring cells. The choice of beam is left to the UE implementation, and only cell level measurement report is required. For L1 MR of Rel-18 LTM, the measurement of beams for different serving and neighboring cells depends on UE capabilities, and the network only indicates the total number of beams and cells without specifying which beams to be measured.

Considering network does not know the optimal beams/cells for each measurement occasion, it is straightforward to reuse the Rel-18 L1 measurement design, I.e., network indicates the number of beam and cells to be measured. The selection of beams for candidate/serving cells is determined by the UE implementation. The principle is to measure the optimal beam to represent the quality of serving/candidate cells.

Proposal 4: Follow the Rel-18 L1 measurement design for event triggered L1 measurement. I.e., network indicates the number of beam and cells to be measured. The selection of beams and cells from candidate/serving cells is up to UE implementation.

In Rel-18 LTM, the network configures the number of cells and beams per cell (i.e., nrOfReportedCells and nrOfReportedRS-PerCell) to be reported in L1 MR, and UE capabilities are introduced to indicates the maximum number to be measured and reported. For the measurement of event triggered L1 MR, new capabilities need to be introduced (as it is only for measurement, not for measurement & report). Considering the measurement behaviour should be similar, the maximum value for Rel-18 can serve as the baseline, and the measurement limit should not exceed this maximum number.

Proposal 5: Introduce signalling to indicate the number of cells and beams for event-triggered L1 measurement. The maximum number of cells and beams to be measured can refer to Rel-18 signalling. I.e., 4 candidate cells and 4 beams per cell.

In Rel-18, RAN4 has already defined the detailed L1 measurement capabilities (I.e., 39-3-1 to 39-3-6). We believe that should be enough to provide information for the measurement capabilities of event triggered L1 MR. Therefore, new measurement capability is not needed for event-triggered L1 MR.

Proposal 6: Reuse the Rel-18 RAN4 measurement capabilities for the measurement of event-triggered L1 MR.

Further discussion on cell level

With the clarification in section 2.2.1, the cell level we discussed here only focus on multiple beam measurement at the same MO for one candidate/serving cell. We think this kind of cell level measurement should not be considered for this agenda, with the following reasons:

- LTM is designed for fast cell switch, it is triggered by beam granularity measurement result in Rel-18, there is no intention to change the measurement metrics in Rel-19.

- If cell level is introduced, there are too much duplication design with legacy L3 RRM.

- The fluctuation caused by beam-level measurement can be solved by tuning TTT/offsets/hysteresis.

Therefore, we propose that cell level measurement is not introduced for event triggered L1 MR. To avoid ambiguity, we can say it another way:

Proposal 7: For one candidate/serving cell, multiple beam measurement at the same measurement occasion is not introduced for event triggered L1 MR.

Signalling design of event and resource config

Measurement configuration

In Rel-18 LTM, the LTM CSI resource configuration has been introduced in LTM-config to provide the group of measurement resources across all LTM candidate cells. Network need to collect the configurations and information of candidate cells and generates a common CSI resource configuration. For event triggered L1 MR, it seems reasonable to reuse the same structure for the resource configuration.

Observation 2: The LTM CSI resource configuration is common across all candidate/serving cells.

Proposal 8: The R18 LTM CSI resource configuration is reused as the measurement resource configuration for event triggered L1 MR.

Event configuration

There are two possible options for the signaling design of event configuration.

In Rel-18 LTM, the L1 measurement report configuration (LTM-CSI-ReportConfig-r18) has been introduced in csi-MeasConfig to provide cell-specific (or per CG) measurement report configuration. The reason is that the L1 MR is sent to the serving cell and the requirement and criteria for the judgement of LTM should be related to current serving cell.

If the event configuration is configured together, then this configuration is also per serving cell, which means the events are shared across all candidate cells under the current UE serving cell.

Option1: The event configuration is specific to the serving cell and indicated along with L1 measurement report configuration (LTM-CSI-ReportConfig).

- Pros: Simplified signalling design (reuse R18 LTM CSI report configuration)

- Cons: All candidate cells share a common event configuration.

On the contrary, considering the network deployment, the event parameters for each candidate cell may vary. For instance, the central cell might have easier trigger condition compared to surrounding cells due to denser deployment. Considering this, it seems reasonable to provide a cell-specific event configuration for each candidate cell to allow for individualized configurations. However, this needs new signalling design in the existing LTM configuration for candidate cells (outer config, e.g., new IE in LTM-Candidate-r18)

Option2: The event configuration is specific to each candidate cell, with a new IE added in LTM-candidate.

- Pros: Enables different event configurations for each candidate cell.

- Cons: May lead to a more complex signalling design. (New IE in LTM-candidate)

Note that no matter option1 or 2 is adopted, it overrules the agreement in RAN2 #125bis:

L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

We suggest RAN2 to analyse these two options with different pro sand cons and make the decision:

Proposal 9: RAN2 to discuss the following options for the signaling design of event configuration:

Option 1: The event configuration is specific to the serving cell and indicated along with L1 measurement report configuration.

Option 2: The event configuration is specific to each candidate cell, with a new IE added in LTM-candidate.

Conclusion

In this contribution, the following observations and proposals are made:

Triggering events

Proposal 1: The event LTM1 (Beam of serving cell becomes better than absolute threshold) is not needed for LTM.

Proposal 2: The s-measure threshold is not needed for event triggered L1 MR.

Clarification on beam level

Proposal 3: Single beam measurement with changing beam index should be considered as beam level measurement.

What beam to be measured?

Proposal 4: In triggering events which involves serving beam (e.g., LTM2 and LTM5), the beam quality of serving cell should be the best among all serving beams.

Proposal 5: In triggering events which involves neighbor cell beams (e.g., LTM3,4 and 5), the selection of candidate cells and candidate cell’s beams is up to UE implementation.

Proposal 6: FFS whether to introduce signalling and/or capabilities to control the measurement cells/beams number for event triggered L1 MR.

Further discussion on cell level

Proposal 7: For one candidate/serving cell, multiple beam measurement at the same measurement occasion is not introduced for event triggered L1 MR.

Measurement configuration

Proposal 8: The R18 LTM CSI resource configuration is reused as the measurement resource configuration for event triggered L1 MR.

Event configuration

Proposal 9: RAN2 to discuss the following options for the signalling design of event configuration:

Option 1: The event configuration is specific to the serving cell and indicated along with L1 measurement report configuration.

Option 2: The event configuration is specific to each candidate cell, with a new IE added in LTM-candidate.

References

- R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements