3GPP TSG-RAN WG2 Meeting #127	R2- 2406358

Maastricht, Netherlands, Aug 19th - Aug 23rd, 2024

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Discussion on the measurement reporting of event triggered L1 MR

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 discussed several aspects for event triggered L1 measurement reporting. This contribution focuses the discussion on the details of measurement reporting procedure, including what information in the measurement report, the need of multiple measurement reporting once triggering event is met.

Discussion

Report content

In legacy L3 measurement report, the physical cell ID, measurement quantity, RS type and related RS index are involved. We can consider these as starting point to further discuss the component of the event triggered L1 measurement report.

- Cell ID/Index: In Rel-18 LTM, the cell information is implicitly reported by the SSBRI, which represents the index of the resource configuration. However, UE is still able to calculate the specific physical cell ID from PSS and SSS when performing the measurement toward SSBs. For L1 MR in Rel-18, the network knows the cell corresponding to each reported RS index. However, the event triggered L1 MR may contain measurement result filtered from different RS index in different MP. It should be clearer if the physical cell ID can be indicated explicitly in the measurement report.

Proposal 1: The cell ID/index is indicated explicitly in the event triggered L1 MR. FFS whether PCI or cell index is used.

- RS index: Since there is no cross beams measurement in Rel-18 L1 MR, the RS index can clearly indicate the measured resource. However, for event triggered L1 MR, the measurement results may include results filtered from multiple resources in multiple MO, and reporting RS index will make the report more complex in this case. Therefore, RAN2 may discuss the following two options for the level of details:

  - Option 1: Only cell index (or ID) is included. The UE reports only one measurement quantity for each measured cells, which represents the average radio link quality of this cell during the period of event triggered L1 MR.

  - Option 2: The report includes both cell index and RS indexes. For each measured cell, the UE reports a matrix to record the values of different RS indexes measured in different MOs (RSs × MOs) during the period of event triggered L1 MR.

Considering the intention of this agenda is to indicate the “event” to the network, it seems more appropriate to allow the UE to perform measurement analysis and provide a comprehensive result. This may also be more friendly to signalling overhead (to MAC-CE or UCI) and latency. Therefore, we slightly prefer Option1.

Proposal 2: RAN2 to discuss the level of RS details in the report of event triggered L1 MR:

  - Option 1: Only cell index (or PCI) is included. The UE reports only one measurement quantity for each measured cells during the period of event triggered L1 MR, which represents the average radio link quality of this cell.

  - Option 2: The report includes both cell index and RS indexes. For each measured cell, the UE reports a matrix to record the values of different RS indexes measured in different MOs (RSs × MOs) during the period of event triggered L1 MR.

- Triggering event type: When multiple events are configured simultaneously, it could be beneficial to include the event type in the measurement report for network judgement if only a general quantity for the event is provided (i.e., option 1 in proposal 2). On the other hands, if the detailed RSs measurement results in each MO are included in the report (i.e., option 2 in proposal 2), there is no need to explicitly indicate the triggering event type, as the network can derive all the details, including the event type from the report.

Proposal 3: The indication of the event type depends on the level of RS detail to be reported:

  - If option 1 is adopted, the triggering event type is included in the report.

  - If option 2 is adopted, the triggering event type is not needed in the report.

- RS type: RS type is included in the legacy L3 measurement report, and it is also included in the WID of Rel-19 mobility (CSI-RS measurement enhancement). Therefore, it should be agreeable to introduce the RS type to represent the measured resource RS in the report, since the measured value of different RS could be very different.

- MeasQuantity: The measurement quantity (e.g., RSRP, RSRQ) is needed to represent the link quality of the measured beam/cell. It is a must in the report. RAN1 may further discuss which measurement quantity to be used in event triggered L1 MR.

Proposal 4: The RS type and measurement quantity are included in the report of event triggered L1 MR.

Report container

This part is covered in another co-source paper R2-2406921. In short, we support MAC-CE way and think the solution could be independent for these two WIDs (with MIMO) considering different use cases and requirements.

Subsequent reporting

Another discussion point is whether the event-triggered L1 measurement reporting is a one-shot report or if multiple report is introduced. We understand that this is more related to the design of the report container. If a MAC-CE based solution is adopted, reliability is ensured by the HARQ mechanism, and it seems unnecessary to introduce multiple reporting to address reliability concerns.

Observation 1: If MAC-CE way is adopted, there is no need to introduce retransmissions/multiple reporting to address reliability concerns.

On the other hand, some may worry about the fluctuation for a single report. we understand the fluctuation could be smoothed by adjusting the parameters of hysteresis, TTT, and offset. The introduction of retransmissions or multiple reporting will not bring benefits but instead weaken the benefit of signalling overhead reduction achieved by the event-triggered mechanism.

Observation 2: Fluctuation concern can be address by adjusting the parameter of hysteresis, TTT and offset. No need to introduce retransmissions/multiple reporting to address this concern.

Therefore, we propose:

Proposal 5: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration)

Conclusion

In this contribution, the following observations and proposals are made:

Report content

Proposal 1: The cell ID/index is indicated explicitly in the event triggered L1 MR. FFS whether PCI or cell index is used.

Proposal 2: RAN2 to discuss the level of RS details in the report of event triggered L1 MR:

  - Option 1: Only cell index (or PCI) is included. The UE reports only one measurement quantity for each measured cells during the period of event triggered L1 MR, which represents the average radio link quality of this cell.

  - Option 2: The report includes both cell index and RS indexes. For each measured cell, the UE reports a matrix to record the values of different RS indexes measured in different MOs (RSs × MOs) during the period of event triggered L1 MR.

Proposal 3: The indication of the event type depends on the level of RS detail to be reported:

  - If option 1 is adopted, the triggering event type is included in the report.

  - If option 2 is adopted, the triggering event type is not needed in the report.

Proposal 4: The RS type and measurement quantity are included in the report of event triggered L1 MR.

Subsequent reporting

Observation 1: If MAC-CE way is adopted, there is no need to introduce retransmissions/multiple reporting to address reliability concerns.

Observation 2: Fluctuation concern can be address by adjusting the parameter of hysteresis, TTT and offset. No need to introduce retransmissions/multiple reporting to address this concern.

Proposal 5: The event triggered L1 MR is a one-shot report. Multiple reporting for one trigger event is not introduced. (This does not preclude multiple report by multiple trigger events configuration)

References

- R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements