3GPP TSG-RAN WG2 Meeting #127	 R2-2406364

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	KDDI Corporation

Title:	Considerations on RLC re-transmission related enhancements for XR

Tdoc Type:	Discussion

Agenda Item:	8.7.5 RLC enhancements

Document for: 	Discussion

Release:       	Rel-19

1	Introduction

In this contribution, we will share our views on the following objective, RLC re-transmission related enhancements [1].

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

2	Discussion

  - Avoid unnecessary retransmissions

    - Tx initiated approach vs Rx initiated approach

RAN2 discussed the Tx initiated approach and Rx initiated approach and had some common understanding on both options as followings.

For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

For Tx initiated approach, in our view, notifying the obsolete SDUs from the Tx side to the Rx side is necessary, but stopping retransmitting obsolete SDUs at the Tx side is not required if the Rx side performs proper advancement of the receiving windows for obsolete SDUs. In other words, we cannot avoid the impact on the Rx side, regardless of whether we adopt stopping retransmitting obsolete SDUs at the Tx side. There may be a small benefit in starting to stop unnecessary retransmissions a little earlier compared to the Rx-initiated approach.

Observation 1: The Tx-initiated approach also requires impacts on the Rx side (The Tx-initiated approach cannot avoid impacts on the Rx side).

Observation2: Stopping retransmitting obsolete SDUs at the Tx side is not required if Rx side performs proper advancing of the transmitting window for obsolete SDUs.

Observation3: Stopping retransmitting obsolete SDUs at the Tx side would have a small benefit in terms of starting to stop unnecessary retransmissions a little earlier than Rx initiated approach.

For Rx initiated approach, in our view, have an advantage in terms of the specification impacts, the Rx initiate approach can reduce the specification impacts on the Tx side. Specifically, Tx side just processes the status report as in legacy and it can avoid the specification impacts on retransmission procedure for obsolete SDUs instead. Regarding how Rx side determines that an SDU should be abandoned, we believe that Rx side should determines based on the notification of obsolete SDUs from Tx side.

Observation4: Rx initiate approach can reduce the specification impacts on the Tx side.

Proposal 1: RAN2 agree to adopt Rx initiated approach with the notifications of obsolete SDUs from Tx side.

    - Identification of discarded RLC SDUs

To avoid this unnecessary retransmission, first transmitting side of an AM RLC entity in UE needs to identify discarded RLC SDUs. Regarding this identification, we confirm that the current PDCP, RLC specifications specify that PDCP indicates the discard to RLC. Also, the current specification does not allow to introduce an RLC SN gap. However, if we consider discarding all in-flight old data, all discarded PDCP SDUs, it will result in RLC SN gap(s).

TS38.323

5.3 SDU discard When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

TS38.322

5.4 SDU discard procedures When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

    - How to introduce RLC SN gap(s)

To consider introducing RLC SN gaps, first we must discuss whether the transmitting side notifies the RLC sequence numbers of discarded RLC PDUs to the receiving side. In our view, this indication of RLC sequence numbers is mandatory for the case where transmitting side will not retransmit the discarded RLC PDUs. Without this indication, the receiving side will continue to request the retransmission of the discarded RLC PDUs forever, nevertheless the transmitting will not retransmit the discarded RLC PDUs.

Proposal 2: RAN2 agree to introduce the indication of the RLC sequence numbers of discarded RLC PDUs, the indication is from the transmitting side to the receiving side.

The next discussion will be the behaviour of the receiving side after the indication. In our view, it is better to have the receiving side refrain from requesting the retransmission of discarded RLC PDUs to the transmitting side and consider the discarded RLC PDUs as if they were received correctly. It means that the receiving side of an AM RLC entity will not include NACK_SN information of discarded RLC PDUs and include ACK_SN information of discarded RLC PDUs instead, in other word, this STATUS PDU will not request the transmission of discarded RLC PDUs.

Proposal 3: RAN2 agree that after the SN indication of discarded RLC PDUs, the reviving side AM entity will not include NACK_SN information of discarded RLC PDUs in STATUS PDU, will include ACK_SN information of discarded RLC PDUs in STATUS PDU instead.

Final discussion is the behaviour of the transmitting side after the SN indication. The transmitting side will receive the STATUS PDU including ACK_SN information of discarded RLC PDUs. At this point the transmitting side can treat discarded RLC PDUs as if those were received correctly at the receiving side. Furthermore, the transmitting side will not receive NACK_SN information of discarded RLC PDUs, thus no more retransmission for the discarded RLC PDUS is triggered.

Proposal 4: RAN2 agree that after the SN indication of discarded RLC PDUs, when the transmitting side AM entity receive the STASU PDU including ACK_SN information of discarded RLC PDUs from the receiving side, then the transmitting side treat the discarded RLC PDUs as if those were received correctly at the receiving side.

  - Timely RLC retransmission(s)

RAN2 agreed to explore the possible options for timely retransmissions on RLC layer for XR traffic. At the same time, RAN2 was aware of the drawback of the impact on capacity as below.

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

We believe that, regardless of the options we choose, we must consider the impact on capacity such as increased consumption of radio resources for retransmissions and STATUS PDUs especially in the scenarios where the congestions are detected at the gNBs side. Thus, we propose the followings.

Proposal 5: RAN2 agree to introduce enhancements for timely retransmissions only applicable to the cases where there are no capacity concerns (no congestions).

Proposal 6: RAN2 agree to introduce a configuration to enable/disable enhancements for timely retransmissions, if RAN2 agree to specify timely retransmissions on RLC layer for XR traffic.

3	Conclusion

We make the following proposals:

Proposal 1: RAN2 agree to adopt Rx initiated approach with the notifications of obsolete SDUs from Tx side.

Proposal 2: RAN2 agree to introduce the indication of the RLC sequence numbers of discarded RLC PDUs, the indication is from the transmitting side to the receiving side.

Proposal 3: RAN2 agree that after the SN indication of discarded RLC PDUs, the reviving side AM entity will not include NACK_SN information of discarded RLC PDUs in STATUS PDU, will include ACK_SN information of discarded RLC PDUs in STATUS PDU instead.

Proposal 4: RAN2 agree that after the SN indication of discarded RLC PDUs, when the transmitting side AM entity receive the STASU PDU including ACK_SN information of discarded RLC PDUs from the receiving side, then the transmitting side treat the discarded RLC PDUs as if those were received correctly at the receiving side.

Proposal 5: RAN2 agree to introduce enhancements for timely retransmissions only applicable to the cases where there are no capacity concerns (no congestions).

Proposal 6: RAN2 agree to introduce a configuration to enable/disable enhancements for timely retransmissions, if RAN2 agree to specify timely retransmissions on RLC layer for XR traffic.

4	Reference

[1]　 RP-240791	Revised WID on XR (eXtended Reality) for NR Phase 3		Nokia (Rapporteur)