3GPP TSG-RAN WG2 Meeting #127	R2-2406365

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

Discussion

Scenario

Remote

First relay

Last relay

gNB

The discussion in this paper will focus on one additional hop scenario as indicated in the WID, the sketch of one additional hop multi-hop U2N relaying is as follows:

Figure 1. Sketch of multi-hop U2N relaying

In multi-hop U2N relay, since the additional hop of relay, there is one scenario issue that needs to be clarified before the detailed mechanism design: Whether more than one path can be supported for a single first relay.

gNB

Last relay

Last relay

first relay

first relay

remote

remote

Case A (no MP-like topology)

gNB

Last relay

first relay

first relay

remote

remote

Case B (no MP-like topology)

gNB

Last relay

Last/first relay

remote

remote

gNB

Last relay

Last relay

first relay

remote

remote

Case D (I+I MP-like topology)

Case C (D+I MP-like topology)

For the topology of multi-hop U2N Relay, there are the following 4 cases:

Figure 2. Topology for multi-hop U2N Relay

As shown in above Figure 2, in case A and B, each relay has one single path to the network, while in case C and case D, one single first relay has 2 paths to the network, especially in case D, the 2 legs are both indirect path, which is not a supported scenario in R18 multi-path relay.

For L2 multi-hop U2N Relay, the case that a single first relay UE has two paths towards the network, either via multiple last relays or via both direct path and indirect paths is a multi-path like topology for the first relay UE.

The multi-path and multi-hop L2 U2N Relay combination is not in the scope of RAN2 WID, especially for the 2 indirect path case. Therefore, the clarification on whether the multi-hop relay UEs can have more than one paths to the network, i.e., whether multi-hop and multi-path combination should be supported.

The multi-path (direct + indirect paths and 2 indirect path) and multi-hop L2 U2N Relay combination is not in the scope of current WID.

Besides the scope of the WID, the multi-path and multi-hop combination also makes the scenario complex, for example, whether the different legs can connect to different cells or even gNBs, the impact to bearer configurations and how to handle the path switch…

The multi-path (direct + indirect paths and 2 indirect path) and multi-hop L2 U2N Relay combination makes the scenario of multi-hop U2N Relay complex.

Considering the scope, complexity and limited TU, it is suggested to not consider such scenario in this release for L2 multi-hop U2N Relay.

RAN2 to discuss whether one multi-hop U2N relay UE can have more than one path (e.g., two indirect paths, or one direct path plus one indirect path) towards the network in this release.

If RAN2 agrees that multi-hop and multi-path L2 U2N Relay combination should be excluded, for the detailed mechanism, there are the following 2 aspects:

- To avoid D+I multi-path, the Relay UE in multi-hop case can either be the first relay or the last relay but not both at the same time;

- To avoid I+I multi-path, the first relay UE in multi-hop case can only connect to a single last relay.

To exclude multi-path and multi-hop combination, it needs to be excluded at the relay discovery stage, which is more of SA2 scope, so it can be up to SA2 to enforce.

If multi-hop U2N relay UE cannot have more than one path (e.g., two indirect paths, or one direct path plus one indirect path) towards the network in this release, RAN2 to confirm it is up to S2 to enforce it (e.g., : 1) To avoid D+I multi-path, the Relay UE in multi-hop case can either be the first relay or the last relay but not both at the same time; 2) To avoid I+I multi-path, the first relay UE in multi-hop case can only connect to a single last relay.).

Multi-hop U2N Discovery

SA2 has concluded that both Model-A and Model-B are supported for multi-hop U2N Relay, and together with the following conclusions:

- Standalone discovery & link setup/management is considered and both Model A and Model B are supported. - The 5G ProSe Direct Discovery message will be extended with an indication that multi-hop relay is supported, along with the hop count and the maximum number of hops. - The maximum number of hops is provisioned by the network to the 5G ProSe Remote UE, Intermediate Relay and UE-to-Network Relays per RSC. - The 5G ProSe Direct Discovery message may be dropped if the maximum number of hops is reached.

RAN2 wait for SA2 progress on multi-hop U2N Relay discovery higher layer mechanism, e.g., hop-count indication, maximum number of hops….

From AS layer perspective, the following aspects are discussed in R17/R18 relay discovery:

Table 1. RAN2 related issues for discovery

| Issues | Mechanism in R17/R18 | R19 multi-hop U2N Relay |
| --- | --- | --- |
| Discovery configuration | In pre-configuration/SIB and dedicated RRC message | Follow legacy |
| Protocol stack | SL-SRB4 | Follow legacy |
| Resource pool | Both shared and dedicated pool | Follow legacy |
| AS threshold condition | Uu threshold is defined for U2N relay, PC5 threshold is defined for U2U relay | To further discuss |

As shown in the above table, for the discovery configuration mechanism, protocol stack, and resource pool, the legacy mechanism can be followed in the R19 multi-hop U2N Relay discovery.

For multi-hop U2N relay discovery, RAN2 to confirm the following aspects:

- Remote UE and Relay UE in multi-hop U2N relay can acquire discovery configuration via pre-configuration, SIB, or dedicated configuration as in R17/R18.

- Multi-hop U2N relay discovery re-uses the protocol stack for SL-SRB4 defined in R17/R18.

- Multi-hop U2N relay discovery uses the same resource pool as single-hop U2N Relay and U2U Relay.

For the AS threshold condition for discovery transmission, in R17 U2N Relay and R18 U2U Relay, the following conditions are defined to restrict discovery transmission:

- For R17 U2N discovery transmission, Uu threshold is defined for U2N Relay UE (both lower and upper bound) and U2N Remote UE (upper bound);

- For R18 U2U discovery transmission, for U2U Relay UE, per-hop PC5 threshold is defined for Model-B discovery solicitation message forwarding and Model-A neighbour UE determination; For U2U Remote UE, E2E PC5 threshold and per-hop PC5 threshold are defined for the remote UE to determine whether to transmit/respond Model-B discovery.

In R17/R18 U2N/U2U Relay, Uu/PC5 thresholds are defined to restrict discovery transmission for both relay and remote UE.

In multi-hop U2N Relay, the AS channel quality evaluation for discovery transmission is also needed to restrict discovery transmission.

For multi-hop U2N Relay, RAN2 to confirm AS threshold condition for discovery transmission is needed as in R17/R18, FFS for details.

The detailed analysis for multi-hop U2N Relay is as follows:

Table 1. Threshold restriction for discovery transmission

|  | Uu threshold | PC5 threshold |
| --- | --- | --- |
| Last Relay | Same as single-hop relay, i.e., reuse the Uu threshold condition for single-hop relay UE. | Same as single-hop relay, i.e., no need for PC5 threshold. |
| First Relay | The Uu threshold condition is needed to avoid cell-center UE acting as first Relay in multi-hop U2N Relay | For the PC5-hop with the last Relay: similar to R17 single-hop U2N Relay Uu threshold condition, the link quality judgement for the link between the parent node is needed to ensure the link between first relay and last relay is good. For the PC5-hop with the Remote UE, by following R17 single-hop U2N mechanism, the PC5 threshold condition is not needed since there is no need for U2N relay UE to do discovery filtering. |
| Remote | Same as single-hop remote, i.e., i.e., reuse the Uu threshold condition for single-hop remote UE. | Same as single-hop relay, i.e., no need for PC5 threshold. |

As shown in the above table, for Uu threshold configuration, for the last relay and remote UE, there is no difference compared to single-hop U2N Relay discovery transmission, i.e., the last relay should be in-coverage and the remote UE can be either in-coverage or out-of-coverage.

For the discovery transmission at remote UE and the last U2N Relay UE, the last relay should be in-coverage and the remote UE can be either in-coverage or out-of-coverage.

Then for the first relay UE, for the Uu threshold configuration, different from the last relay restriction, it doesn’t need to have good Uu quality to act as first relay UE, it can either in-coverage or out-of-coverage as long as it is not in the cell center (since in that case, it should be the last relay), which means only the upper bound is needed. From this perspective, it is similar to U2N Remote UE, which means an upper bound of Uu quality is needed. For the detailed configuration, it can be based on same or separate value configuration.

The first relay can either be in-coverage or out-of-coverage to transmit discovery message.

For the discovery transmission by the last relay UE, at least a lower bound and an upper bound of Uu channel quality is needed.

For the discovery transmission by the first relay UE and remote UE, at least an upper bound of Uu channel quality is needed, FFS whether it is based on the same configuration or separate configurations.

For the PC5 threshold, as in single-hop U2N Relay, there is no PC5 condition to be checked for the remote UE and the last relay UE, so the PC5 link quality is not needed.

For discovery transmission by the last relay and remote UE, no PC5 threshold configuration is needed

And for the PC5 threshold for the first relay UE, the PC5 link quality with the last relay needs to be checked to ensure the E2E multi-hop relay link quality, which means the first relay should only transmit discovery message if the PC5 link quality between the last relay UE above a configured threshold.

The first relay only transmits discovery message if the PC5 link quality between the last relay UE is above a configured threshold.

Relay (re)selection

For the relay selection and reselection trigger condition, the following trigger condition is defined in R17 single-hop U2N relay and R18 U2U relay case:

The U2N Remote UE triggers U2N Relay selection in following cases: - Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold; - Indicated by upper layer of the U2N Remote UE. The U2N Remote UE may trigger U2N Relay reselection in following cases: - PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold; - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling; - When U2N Remote UE receives a PC5-S link release message from U2N Relay UE; - When U2N Remote UE detects PC5 RLF; - Indicated by upper layer. The U2U Remote UE may trigger U2U Relay selection in the following cases: - When the SL-RSRP or SD-RSRP between U2U Remote UEs is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication to trigger U2U relay selection from the upper layer of the UE. The U2U Remote UE may trigger U2U Relay reselection in the following cases: - When the SL-RSRP or SD-RSRP of the current U2U Relay UE is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication from the upper layer due to detecting PC5 RLF; - When L2 U2U Remote UE receives an indication from the upper layer due to receiving the PC5 RLF indication from the L2 U2U Relay UE; - When U2U Remote UE receives a PC5-S link release message from U2U Relay UE; - When U2U Remote UE receives an indication to trigger U2U relay reselection from the upper layer of the UE.

For multi-hop relay selection, it should be same as single-hop relay, i.e., indicated by upper layer and direct link quality is below a configured threshold.

For multi-hop U2N relay selection, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For Relay reselection, in R17 single-hop U2N Relay and R18 U2U Relay, the trigger condition can be summarized into four aspects:

- Upper layer triggered;

- Link quality with the current relay below a configured threshold;

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

- The link between the relay UE and other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger:

- Upper layer triggered;

- Link quality with the current relay below a configured threshold;

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

- The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In R19 multi-hop U2N Relay, we can follow the spirit and consider relay reselection triggers from the four aspects. For the upper layer trigger, link quality degradation with the current direct connected relay, and the link between the remote UE and direct connected relay breaks (RLF or PC5-S release), it is similar for single-hop U2N Relay and multi-hop U2N Relay, so the R17 U2N Relay trigger condition can be reused.

For multi-hop U2N Relay reselection trigger, reuse the following R17 single-hop U2N Relay reselection trigger condition:

- PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold;

- When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE;

- When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE;

- Indicated by upper layer.

For the indication of the other link problem, firstly for the Uu link between the relay UE and network, when Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure happens, it is reasonable for the remote UE to trigger relay reselection since group handover is not supported.

Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

And for the PC5 link between two multi-hop U2N Relay UEs, similar to Uu link, when the PC5 radio link between 2 multi-hop U2N Relay UEs fails, the multi-hop U2N Relay UE which is closer to the remote UE should indicate such information to the remote UE so that the remote UE can know the multi-hop relay link is broken.

The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link is broken and relay reselection is needed.

The unclear part is how for the multi-hop U2N Relay UE to indicate such information to the remote UE considering the legacy Notification message is a per-hop PC5 RRC message. So the indication can be hop-by-hop which needs RRC message (Notification message) forwarding between the UEs or can be E2E indication if there is PC5-RRC connection and the notification message can be a E2E RRC message. This depends on the control plane design which can be further discussed by RAN2.

How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

- cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network;

- PC5 RLF between the multi-hop U2N Relay UE(s);

For the relay (re)selection criterion, SA2 has made the following conclusions on the higher layer criterion:

- The relay path is selected based on e.g. the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during the normative phase.

Rely on SA2 for the multi-hop relay (re)selection higher layer criterion, e.g., maximum hop number-based restriction.

The AS criterion in single hop U2N Relay is the PC5 link quality with the candidate relay UE, this should be reused in multi-hop case.

As in R17 single hop U2N Relay (re)selection, SL-RSRP or SD-RSRP between Remote UE and the multi-hop U2N relay UE is used as multi-hop Relay (re)selection AS criterion.

Conclusion

We have the following observations:

Observation 1	For L2 multi-hop U2N Relay, the case that a single first relay UE has two paths towards the network, either via multiple last relays or via both direct path and indirect paths is a multi-path like topology for the first relay UE.

Observation 2	The multi-path (direct + indirect paths and 2 indirect path) and multi-hop L2 U2N Relay combination is not in the scope of current WID.

Observation 3	The multi-path (direct + indirect paths and 2 indirect path) and multi-hop L2 U2N Relay combination makes the scenario of multi-hop U2N Relay complex.

Observation 4	In R17/R18 U2N/U2U Relay, Uu/PC5 thresholds are defined to restrict discovery transmission for both relay and remote UE.

Observation 5	For the discovery transmission at remote UE and the last U2N Relay UE, the last relay should be in-coverage and the remote UE can be either in-coverage or out-of-coverage.

Observation 6	The first relay can either be in-coverage or out-of-coverage to transmit discovery message.

Observation 7	In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger:

-	Upper layer triggered;

-	Link quality with the current relay below a configured threshold;

-	The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

-	The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

Observation 8	Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

Observation 9	The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link is broken and relay reselection is needed.

Observation 10	How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

We have the following proposals:

Proposal 1	RAN2 to discuss whether one multi-hop U2N relay UE can have more than one path (e.g., two indirect paths, or one direct path plus one indirect path) towards the network in this release.

Proposal 2	If multi-hop U2N relay UE cannot have more than one path (e.g., two indirect paths, or one direct path plus one indirect path) towards the network in this release, RAN2 to confirm it is up to S2 to enforce it (e.g., : 1) To avoid D+I multi-path, the Relay UE in multi-hop case can either be the first relay or the last relay but not both at the same time; 2) To avoid I+I multi-path, the first relay UE in multi-hop case can only connect to a single last relay.).

Proposal 3	RAN2 wait for SA2 progress on multi-hop U2N Relay discovery higher layer mechanism, e.g., hop-count indication, maximum number of hops….

Proposal 4	For multi-hop U2N relay discovery, RAN2 to confirm the following aspects:

-	Remote UE and Relay UE in multi-hop U2N relay can acquire discovery configuration via pre-configuration, SIB, or dedicated configuration as in R17/R18.

-	Multi-hop U2N relay discovery re-uses the protocol stack for SL-SRB4 defined in R17/R18.

Proposal 5	For multi-hop U2N Relay, RAN2 to confirm AS threshold condition for discovery transmission is needed as in R17/R18, FFS for details.

Proposal 6	For the discovery transmission by the last relay UE, at least a lower bound and an upper bound of Uu channel quality is needed.

Proposal 7	For the discovery transmission by the first relay UE and remote UE, at least an upper bound of Uu channel quality is needed, FFS whether it is based on the same configuration or separate configurations.

Proposal 8	For discovery transmission by the last relay and remote UE, no PC5 threshold configuration is needed

Proposal 9	The first relay only transmits discovery message if the PC5 link quality between the last relay UE is above a configured threshold.

Proposal 10	For multi-hop U2N relay selection, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

Proposal 11	For multi-hop U2N Relay reselection trigger, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold;

-	When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE;

-	Indicated by upper layer.

Proposal 12	Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network;

-	PC5 RLF between the multi-hop U2N Relay UE(s);

Proposal 13	Rely on SA2 for the multi-hop relay (re)selection higher layer criterion, e.g., maximum hop number-based restriction.

Proposal 14	As in R17 single hop U2N Relay (re)selection, SL-RSRP or SD-RSRP between Remote UE and the multi-hop U2N relay UE is used as multi-hop Relay (re)selection AS criterion.