3GPP TSG-RAN WG2 Meeting #127	R2-2406366

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.3

Source:	OPPO

Title:	Control plane procedures of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the control plane procedures of multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

Discussion

Uu RRC connection

The RRC state of multi-hop U2N relay needs to be discussed. In single hop U2N relay, the following agreements are made:

Proposal 16: For L3 UE to NW relay, a relay UE must be in RRC_CONNECTED to perform relaying of data. Proposal 18: For L2 UE to NW relay, both relay UE and remote UE must be in RRC CONNECTED to perform active relaying of data. Proposal 3 [easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_IDLE is excluded Proposal 4 [easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_INACTIVE is excluded Proposal 6 [easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_INACTIVE and relay UE in RRC_CONNECTED is supported Proposal 8 [easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_INACTIVE and relay UE in RRC_INACTIVE is supported Proposal 9 [easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_IDLE and relay UE in RRC_INACTIVE is supported

In R17 single hop U2N Relay, the relay UE has to be in RRC connected state to serve a connected remote UE’s data, and can be in RRC IDLE/INACTIVE to serve RRC IDLE/INACTIVE remote UEs.

Remote

First relay

Last relay

gNB

In R17 single hop U2N Relay, the relay UE has to be in RRC connected state to serve a connected remote UE’s data.

Figure 1. sketch of multi-hop U2N relaying

As in the above figure, the last relay UE which is the UE closest to the network in multi-hop relaying, must be in RRC connected state to perform data transmission/reception to/from the network.

The relay UE which is closest to the network in multi-hop relay must be in RRC connected state to perform data transmission/reception to/from the network.

For the first relay which is the relay UE closest to the remote UE, since it doesn’t need to perform data transmission/reception directly with the network but just acts as the relay on the PC5 link between remote UE and the last relay, it seems not that straightforward to must be in RRC connected state to serve the connected remote UE’s data. However, there are problems if the non-connected multi-hop relay is supported, the first concern is the Uu BEARER is forwarded by an IDLE/INACTIVE multi-hop relay UE without gNB’s authorization and control, which is not aligned with the legacy principle.

There is security concern that Uu BEARER is forwarded by an IDLE/INACTIVE multi-hop relay UE without gNB’s authorization and control.

Then, it is also complex to allow IDLE/INACTIVE Relay UE to serve connected remote UE’s data transmission. The following table summarizes the control plane handling for remote UE’s data transmission via multi-hop relaying when all the multi-hop Relay UEs are in RRC connected or can be in RRC IDLE/INACTIVE.

Table 1. control plane handling for remote UE’s data transmission

| Issues | All MH-Relay UEs in RRC CONNECTED state | MH-Relay UE has no direct Uu link can be in RRC IDLE/INACTIVE state |
| --- | --- | --- |
| QoS split | Controlled by network as in R17 | Issue-1: Whether QoS split is controlled by network or split based on Relay UE implementation or in a combined manner; Issue-2: How to coordination the split QoS between IDLE/INACTIVE Relay and network. |
| UE ID allocation | Controlled by network as in R17 | Issue-1: Whether network or relay UE to do the UE ID allocation; Issue-2: How to avoid UE ID collision. |
| BEARER ID configuration | Controlled by network as in R17 | Issue-1: Whether network or relay UE to configure the BEARER ID; Issue-2: Whether the BEARER ID can be different on different hops |
| Egress RLC channel configuration | Controlled by network as in R17 | Issue-1: How for the IDLE/INACTIVE multi-hop Relay UE to derive the RLC configuration for the Uu bearer; |

As shown in the above table, allowing IDLE/INACTIVE multi-hop relay UE will make the control plane handling complex and leave many issues to solve.

Allow non-connected multi-hop U2N Relay (first relay) increases the complexity of the control plane design of multi-hop U2N Relay.

Besides, there is no problem for the case that all the multi-hop relay UE are in RRC connected state to serve the CONNECTED remote UE’s data, and that case anyway needs to be supported.

The case when all the multi-hop U2N Relay UEs are in RRC CONNECTED state to serve the CONNECTED remote UE’s data works and should anyway be supported.

So considering the limited TU allocated for R19 multi-hop U2N Relay, it is suggested to begin with the simple and baseline scenario and can consider the enhancement to support IDLE/INACTIVE multi-hop U2N Relay UE to serve CONNECTED remote UE’s data if time allows.

For L2 multi-hop U2N relay, R2 discuss whether all the relay UEs have to be in RRC_CONNECTED to serve the RRC_CONNECTED remote UE.

If all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, as analysis in the above Table 1, the QoS split and SRAP configurations which includes UE ID, BEARER ID and RLC channel configuration can be handled by the network as in single-hop case.

When all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, the QoS split and SRAP configurations which include UE ID, BEARER ID and RLC channel configuration can be handled by the network as in R17 single hop U2N Relay.

For RRC CONNECTED L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to split the E2E QoS to per-hop QoS as in R17 single-hop U2N Relay.

For RRC CONNECTED L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to do the SRAP configuration (e.g., UE ID, BEARER ID and egress RLC channel configurations).

PC5 RRC connection

Besides Uu RRC connection, in multi-hop U2N Relay, since the additional Relay UEs, there are also non-adjacent UEs on PC5 link, for example, the remote UE and the relay UE closet to the gNB. Whether the E2E PC5 RRC connection is needed for such UEs need to be discussed.

Firstly, if there is data transmission between the 2 UEs, the E2E connection is needed but it will use a separate link and L2 ID since it is a U2U communication and different RSC will be used. So from AS perspective, there is nothing to worry.

The sidelink data transmission between 2 UEs and U2N Relay data transmission will not share the same PC5 link.

Then, if there is no data transmission between the 2 UEs, whether the PC5 RRC connection is needed only for control signalling. Currently, between the U2N Remote UE and Relay UE, the following PC5 RRC procedures are defined:

- RRCRconfigurationSidelink to add/release/modify PC5 RLC channel;

- Notification message to indicate the Uu link status;

- Uu information transfer to forward Paging and SIBs.

For L2 U2N Relay, PC5-RRC is used to: 1) add/release/modify PC5 RLC channel via RRCRconfigurationSidelink; 2) indicate the Uu link status via Notification message; 3) forward Paging and SIBs via Uu information transfer.

Firstly, for RRCRconfigurationSidelink to add/release/modify PC5 RLC channel, since there is no direct PC5 connection, there is no RLC channel between two non-adjacent UEs at multi-hop U2N Relay link. Which means the PC5 RLC channel add/release/modify is not needed between two non-adjacent UEs at multi-hop U2N Relay link.

PC5 RLC channel add/release/modify is not needed between two non-adjacent UEs at multi-hop U2N Relay link.

Then for Notification message and Uu information transfer, without E2E PC5 RRC, they can also work in a per-hop manner, e.g., the intermediate relay UE can help to forward the UEInformationRequestSidelink from the remote UE and UEInformationResponseSidelink/ NotificationMessageSidelink from the relay UE which is closest to the gNB.

The Uu information transfer and Notification message procedure can work without E2E PC5 RRC between two non-adjacent UEs by the intermediate relay UE forwarding (in a re-generation manner) the UEInformationRequestSidelink, UEInformationResponseSidelink and NotificationMessageSidelink.

Besides, as indicated in the WID, the extensibility to a second additional hop in this WI is considered as a design criterion. With the 2 additional Relays, supporting the E2E PC5 connection between the U2N Remote UE and the U2N Relay UE closest to the gNB means multi-hop U2U Relay is also supported, which is out of scope of the WID.

In two additional Relays case, the supporting of E2E PC5 connection between the U2N Remote UE and the U2N Relay UE closest to the gNB means multi-hop U2U Relay is also supported, which is out of scope of the WID.

Therefore, it is suggested RAN2 to discuss whether the E2E PC5 RRC connection between the non-adjacent UEs in the multi-hop U2N Relay link is needed.

RAN2 to discuss whether the E2E PC5 RRC connection between the non-adjacent UEs in the multi-hop U2N Relay link is needed (e.g., remote UE and last relay).

Conclusion

We have the following observations:

Observation 1	In R17 single hop U2N Relay, the relay UE has to be in RRC connected state to serve a connected remote UE’s data.

Observation 2	The relay UE which is closest to the network in multi-hop relay must be in RRC connected state to perform data transmission/reception to/from the network.

Observation 3	There is security concern that Uu BEARER is forwarded by an IDLE/INACTIVE multi-hop relay UE without gNB’s authorization and control.

Observation 4	Allow non-connected multi-hop U2N Relay (first relay) increases the complexity of the control plane design of multi-hop U2N Relay.

Observation 5	The case when all the multi-hop U2N Relay UEs are in RRC CONNECTED state to serve the CONNECTED remote UE’s data works and should anyway be supported.

Observation 6	When all the L2 multi-hop U2N relay UEs are in RRC connected state to serve the CONNECTED remote UE’s data, the QoS split and SRAP configurations which include UE ID, BEARER ID and RLC channel configuration can be handled by the network as in R17 single hop U2N Relay.

Observation 7	The sidelink data transmission between 2 UEs and U2N Relay data transmission will not share the same PC5 link.

Observation 8	For L2 U2N Relay, PC5-RRC is used to: 1) add/release/modify PC5 RLC channel via RRCRconfigurationSidelink; 2) indicate the Uu link status via Notification message; 3) forward Paging and SIBs via Uu information transfer.

Observation 9	PC5 RLC channel add/release/modify is not needed between two non-adjacent UEs at multi-hop U2N Relay link.

Observation 10	The Uu information transfer and Notification message procedure can work without E2E PC5 RRC between two non-adjacent UEs by the intermediate relay UE forwarding (in a re-generation manner) the UEInformationRequestSidelink, UEInformationResponseSidelink and NotificationMessageSidelink.

Observation 11	In two additional Relays case, the supporting of E2E PC5 connection between the U2N Remote UE and the U2N Relay UE closest to the gNB means multi-hop U2U Relay is also supported, which is out of scope of the WID.

We have the following proposals:

Proposal 1	For L2 multi-hop U2N relay, R2 discuss whether all the relay UEs have to be in RRC_CONNECTED to serve the RRC_CONNECTED remote UE.

Proposal 2	For RRC CONNECTED L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to split the E2E QoS to per-hop QoS as in R17 single-hop U2N Relay.

Proposal 3	For RRC CONNECTED L2 multi-hop U2N relay, R2 discuss whether to rely on gNB to do the SRAP configuration (e.g., UE ID, BEARER ID and egress RLC channel configurations).

Proposal 4	RAN2 to discuss whether the E2E PC5 RRC connection between the non-adjacent UEs in the multi-hop U2N Relay link is needed (e.g., remote UE and last relay).