3GPP TSG-RAN WG2 Meeting #127	R2-2406367

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.5

Source:	OPPO

Title:	Discussion on RLC re-transmission related enhancements

Document for:	Discussion, Agreement

Introduction

For the RLC retransmission related enhancements, there are enhancement options identified in the last RAN2 meeting, this paper will analyze the details of each enhancement to ensure timely RLC retransmission or to avoid unnecessary retransmission for XR.

Discussion

Unnecessary retransmissions

To avoid unnecessary retransmissions, the following agreements are made:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - 1. Rx initiated approach - 2. Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

For the 2 directions for avoiding unnecessary retransmissions, it is agreed to do a down-selection after detailed comparisons. Before comparison, it is better to understand the details of each option.

Figure 1 Sketch of Tx initiated approach

As shown in above Figure 1, for Tx initiated approach, there are mainly 3 steps, the step-by-step detailed specification work analysis is as follows:

Table 1 detailed steps of Tx initiated approach

| Step | Analysis |
| --- | --- |
| Step 1, Tx side PDCP finds the delay budget requirement cannot be satisfied, i.e., discard timer expires, it indicates to RLC to discard the related SDUs, and RLC entity moves the Tx window. | The mechanism is easy and specification impact is minor, e.g., “set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet and not indicated as discarded by the upper layer.” |
| Step 2, Tx side sends SN gap report to Rx side | R18 PDCP SN gap report mechanism can be used as a reference |
| Step 3, Rx side updates its Rx variables accordingly | The Rx variables handling is basically same as “an AMD PDU is placed in the reception buffer” |

As analyzed in the above table, the Tx initiated approach is quite clear without further unclear points/issues for further discussion. And the specification impact is not complex.

The detailed step of Tx initiated approach is as follows:

- Step 1, Tx side PDCP find the delay budget requirement cannot be satisfied, i.e., discard timer expires, it indicates to RLC to discard the related SDUs, and RLC entity moves the Tx window.

- Step 2, Tx side sends SN gap report to Rx side

- Step 3, Rx side updates its Rx variables accordingly

The Tx initiated approach is already clear with no main issue to be further discussed.

For Rx initiated approach, the procedure is as follows:

Figure 2 Sketch of Rx initiated approach

For each step, the analysis is as follows:

Table 2 detailed steps of Rx initiated approach

| Step | Analysis |
| --- | --- |
| Step 1, Rx entity identifies unnecessary SDUs and abandons them | This is the main issue for the Rx initiated approach, i.e., how for the Rx side to know a specific SDU is unnecessary considering it doesn’t know the delay budget of SDUs. |
| Step 2, Rx side updates its Rx variables accordingly | Same as Tx initiated approach |
| Step 3, Rx side indicates SDUs have been abandoned | If based on the status report, i.e., “fake ACK”, the Tx side impact can be avoided. However the status report may not be timely sent (which is the motivation for timely retransmission), so the Tx processing (e.g., if considering blind retransmission) cannot be saved before the status report. |

As analyzed in the above table, there are some unclear issues for the Rx initiated approach, e.g., in step-1, how for the Rx UE to determine a SDU is not necessary without knowing the delay budget; and in step-3, since the status report may not be timely sent, unnecessary retransmission may still be performed.

The detailed step of Rx initiated approach is as follows:

- Step 1, Rx entity identifies unnecessary SDUs and abandons them

- Step 2, Rx side updates its Rx variables accordingly

- Step 3, Rx side indicates SDUs have been abandoned

There are some unclear issues for Rx initiated approach which require further effort, e.g., how for the Rx UE to determine a SDU is not necessary without knowing the delay budget and how to ensure the sync between abandon and retransmission of unnecessary SDUs considering they are performed by different entity and the possibility of status report delay.

For the complexity/specification impact analysis, Tx initiated approach needs a new indication from Tx to Rx to report the SA gap, which is similar to what we have did in R18 PDCP SN gap report; while Rx initiated approach may need a new timer for the Rx entity to identify unnecessary SDUs. From this perspective, both solutions need new things.

From complexity perspective, both Tx initiated and Rx initiated approach need new things, i.e., SN gap indication in Tx initiated approach and new timer (or something else) to determine unnecessary SDUs.

Tx initiated approach is preferred since it can solve the “unnecessary retransmission” issue more cleanly.

For avoiding unnecessary RLC AM retransmissions, RAN2 to adopt the Tx initiated approach.

Besides, the RLC AM retransmission is used to detect RLF, i.e., if RLC failure happens, it means the UL transmission is broken, e.g., due to interference or congestion, the UE should be able to detect the failure case in time to recover it.

RLC failure-based RLF needs to be detected in time for quick UL transmission recovery.

With the enhancement of in-flight data discard, the RLC-based RLF detection can be impacted, i.e., the maxRetxThreshold as the bottom line of UL transmission quality cannot be reached due to the discard of PDUs even when the UL transmission is broken. One example can be seen in the following Figure-1. This seems a typical case since the bad UL transmission situation causes UL transmission failure which leads to delay budget requirement cannot be satisfied and PDU discard.

In-flight data discard will impact RLC-based RLF detection, i.e., the maxRetxThreshold as the bottom line of UL transmission quality cannot be reached due to the discard of PDUs even when the UL transmission is broken.

Figure 3. In-flight PDU discard impact to RLC-based RLF

RLC AM retransmission is used to detect RLF, allowing in-flight SDU discard in AM mode may cause RLF detection issues, especially in a congestion situation.

Therefore, RAN2 should discuss RLF impact due to the in-flight RLC AM PDU discard enhancement.

RAN2 to discuss how to solve RLC-based RLF miss-detection issue (i.e., maxRetxThreshold cannot be reached) caused by in-flight PDU discard enhancement.

Timely RLC retransmission(s)

For timely retransmissions, the following agreements are made:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

For the enhanced status report, it is agreed that the enhancement should focus on UL traffic, and the status report for UL traffic is from gNB which is based on gNB implementation to trigger, so the enhanced status report is not needed, i.e., gNB implementation can achieve the timely status report for UL traffic.

The timely status report for UL traffic can be achieved by gNB implementation.

For enhanced polling from UE for the UL traffic, it is controlled by the configuration from network via polling related parameters (i.e., pollByte, pollPDU, and t-PollRetransmit). So faster polling can be achieved by appropriate network configuration, e.g., a small value of pollByte/pollPDU/t-PollRetransmit.

Based on the current mechanism, the faster polling from UE to network for UL traffic can be achieved by configuring smaller value (existing value or introducing new value) of related parameters (i.e., pollByte, pollPDU, t-PollRetransmit).

For the faster polling in RLC AM, RAN2 discuss to rely on proper configuration (the existing value range or introducing smaller values) of related parameters (i.e., pollByte, pollPDU, t-PollRetransmit).

For faster retransmission, there are some discussions on additional retransmission trigger conditions besides the reception of NACK. There are also concerns about the impact on system capacity since blind retransmission may result in increased latency and reduced throughput, especially considering a congestion system.

Autonomous retransmission can impact the overall system capacity as it may result in increased latency and reduced throughput, especially considering a congestion system.

So the enhancement of autonomous retransmission should be carefully evaluated regarding the benefit and capacity impact.

Firstly, autonomous retransmission is to further increase the redundancy for reliable transmission. Actually, besides RLC AM ARQ, there are PDCP duplications and HARQ re-transmissions can be used to secure reliable transmission.

On top of RLC AM retransmission, PDCP duplication and HARQ re-transmissions can be used to further secure the reliable transmission by redundant transmission.

Besides, the RLC AM retransmission without status report can be performed if t-PollRetransmit expires.

- 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

The RLC SDU can be retransmitted without feedback if t-PollRetransmit expires.

With proper t-PollRetransmit configuration, for example, network to configure the t-PollRetransmit value by consideration of the delay budget of the RLC channel, the timely retransmission can be achieved.

For autonomous retransmission without feedback, RAN2 to discuss relying on the legacy t-PollRetransmit expiry triggered retransmission with proper t-PollRetransmit configuration.

If proposal 4 cannot be agreed, i.e., after analysis, RAN2 concludes the additional trigger condition is needed, then as indicated in the WID, the objective is for the XR data with a small delay budget, which means the delay budget is the key point, so the trigger of retransmission without feedback should be based on the remaining delay budget.

For autonomous retransmission without feedback, if new trigger condition besides PollRetransmit expiry is needed, the trigger of autonomous retransmission should be based on the remaining delay budget, e.g., based on a configured remaining delay threshold.

One possible way to solve the capacity issue is to rely on the network to judge whether/how many blind retransmissions should be performed since the network has a better knowledge of the overall radio resource status.

Network has a better knowledge of the overall radio resource status.

If the RLC retransmission without feedback is supported, it is suggested that RAN2 to discuss to rely on the network to enable and control whether/how the retransmission without feedback is performed.

For autonomous retransmission without feedback, if new trigger condition besides PollRetransmit expiry is needed, RAN2 to discuss to rely on network to enable/disable UL retransmission without feedback and to configure the maximum blind retx number.

Conclusion

We have the following observations:

Observation 1	The detailed step of Tx initiated approach is as follows:

Step 1, Tx side PDCP find the delay budget requirement cannot be satisfied, i.e., discard timer expires, it indicates to RLC to discard the related SDUs, and RLC entity moves the Tx window.

Step 2, Tx side sends SN gap report to Rx side

Step 3, Rx side updates its Rx variables accordingly

Observation 2	The Tx initiated approach is already clear with no main issue to be further discussed.

Observation 3	The detailed step of Rx initiated approach is as follows:

Step 1, Rx entity identifies unnecessary SDUs and abandons them

Step 2, Rx side updates its Rx variables accordingly

Step 3, Rx side indicates SDUs have been abandoned

Observation 4	There are some unclear issues for Rx initiated approach which require further effort, e.g., how for the Rx UE to determine a SDU is not necessary without knowing the delay budget and how to ensure the sync between abandon and retransmission of unnecessary SDUs considering they are performed by different entity and the possibility of status report delay.

Observation 5	From complexity perspective, both Tx initiated and Rx initiated approach need new things, i.e., SN gap indication in Tx initiated approach and new timer (or something else) to determine unnecessary SDUs.

Observation 6	RLC failure-based RLF needs to be detected in time for quick UL transmission recovery.

Observation 7	In-flight data discard will impact RLC-based RLF detection, i.e., the maxRetxThreshold as the bottom line of UL transmission quality cannot be reached due to the discard of PDUs even when the UL transmission is broken.

Observation 8	RLC AM retransmission is used to detect RLF, allowing in-flight SDU discard in AM mode may cause RLF detection issues, especially in a congestion situation.

Observation 9	The timely status report for UL traffic can be achieved by gNB implementation.

Observation 10	Based on the current mechanism, the faster polling from UE to network for UL traffic can be achieved by configuring smaller value (existing value or introducing new value) of related parameters (i.e., pollByte, pollPDU, t-PollRetransmit).

Observation 11	Autonomous retransmission can impact the overall system capacity as it may result in increased latency and reduced throughput, especially considering a congestion system.

Observation 12	On top of RLC AM retransmission, PDCP duplication and HARQ re-transmissions can be used to further secure the reliable transmission by redundant transmission.

Observation 13	The RLC SDU can be retransmitted without feedback if t-PollRetransmit expires.

Observation 14	Network has a better knowledge of the overall radio resource status.

We have the following proposals:

Proposal 1	For avoiding unnecessary RLC AM retransmissions, RAN2 to adopt the Tx initiated approach.

Proposal 2	RAN2 to discuss how to solve RLC-based RLF miss-detection issue (i.e., maxRetxThreshold cannot be reached) caused by in-flight PDU discard enhancement.

Proposal 3	For the faster polling in RLC AM, RAN2 discuss to rely on proper configuration (the existing value range or introducing smaller values) of related parameters (i.e., pollByte, pollPDU, t-PollRetransmit).

Proposal 4	For autonomous retransmission without feedback, RAN2 to discuss relying on the legacy t-PollRetransmit expiry triggered retransmission with proper t-PollRetransmit configuration.

Proposal 5	For autonomous retransmission without feedback, if new trigger condition besides PollRetransmit expiry is needed, the trigger of autonomous retransmission should be based on the remaining delay budget, e.g., based on a configured remaining delay threshold.

Proposal 6	For autonomous retransmission without feedback, if new trigger condition besides PollRetransmit expiry is needed, RAN2 to discuss to rely on network to enable/disable UL retransmission without feedback and to configure the maximum blind retx number.