3GPP TSG-RAN WG2 Meeting #127						R2-2406377

Maastricht, The Netherlands, 19-23 August 2024

Agenda item:			8.2.2

Source:			Intel Corporation

Title:		Required functions for A-IoT

Document for:	 	Discussion and decision

# Introduction

At RAN2#125bis and RAN2#126, RAN2 discussed the supported CP and UP functions for A-IoT. However further discussions are still needed for following issues:

Other WGs related issues:

- Issue 0: Support of security related aspects (SA3/SA2 input required)

- Issue 1: FFS how we handle segmentation case (RAN1 input required)

- Issue 2: Support of Proximity

- Issue 3: FFS how the resource configuration is provided to the device and scheduling (RAN1 input required)

RAN2 centric issues:

- Issue 4: Device type handling

- Issue 5: Higher layer repetition

- Issue 6: QoS handling

- Issue 7: FFS whether further indication of device message size/status is needed;

- Issue 8: FFS about the level of visibility required by the reader and what information is necessary for AS layer operations

- Issue 9: FFS on “Command only” use case

Considering issues 0-3 are tightly related to other WGs, RAN2 should wait for further inputs from those WGs before the discussing them. In this contribution, we continue the discussion on RAN2 centric issues 4-9.

# Discussion

## Device type handling (issue 4)

In RAN2#125bis meeting, RAN2 agreed that RRC connection management is not support, and RRC layer is not necessary. Then open issue 3 is “FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)”.

In cellular system, based on UE capability, RRC is used to configure physical layer parameters, e.g. frequency domain, time domain resources, reference signal, etc, and L2 parameters per radio bearer.

Based on RAN1 discussion, type 1, type 2a and type 2b devices may be supported in Rel-19 A-IoT. The coverage may be different for different type of A-IoT devices. For instance, coverage of type 2 device should be larger than type 1 device, and therefore for type 1 device, Topology 2 may be needed in order to extend the coverage. However, considering that low cost, low complexity and low power are key metrics for A-IoT devices, there should not be huge difference among different type of devices from UE capability perspective. Therefore, capability-based resources allocation should be avoided. On other hand, RAN2 needs to also consider how network can be aware of topology 1 or 2 should be used. To our understanding, the possible way to address this issue is to let the CN to provide the device type information to RAN before RAN sends the initial trigger message. Based on that device type information, RAN can decide whether topology 1 or 2 should be used for a device.

Proposal 1: Capability based resource allocation is not supported. RAN should be aware of device type based on information provided by CN. Send LS to SA2 and Cc RAN3 on this.

## Higher layer repetition (issue 5)

In order to improve the link coverage from the device to reader in A-IoT system design, it may be more beneficial to utilize repetition for PDRCH transmission. At the RAN1#116b meeting, it was agreed to study D2R transmission in the physical layer using repetition as following and left the discussion whether higher layer repetition is supported to RAN2.

Agreement Study D2R transmission in the physical layer using repetition - Note: Discussions regarding higher-layer repetitions are up to RAN2.

At RAN1#117, RAN1 further agreed:

Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times Agreement For D2R, study at least block-level and bit-level repetition type 1 and type 2.

In last RAN2 meeting, RAN2 concluded that

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

During the discussion, companies’ main question is whether it precludes the reader to send initial trigger message again for the same session for different devices.

As defined in RFID, AIoT devices can only send D2R message upon receiving the R2D request. RAN2 agreed a similar statement for AIoT as shown below:

- Random Access is triggered by the reader

If a Reader wants to trigger the RACH procedure for multiple AIoT devices, the Reader has to send the R2D messages several times since every D2R message shall be triggered by R2D message, i.e. there is no device initiated D2R message. It is also related to D2R transmission timing described in [2], the device needs to know the timing to transmit the MSG1 based on R2D message. Figure 1 below illustrates an example of the behaviour just described.

Figure 1: AIoT RACH procedure for multiple AIoT devices

Proposal 2: To address a group of AIoT devices, Reader needs to send R2D message “Inventory request” multiple times.

## Support of QoS (issue 6)

The QoS requirements for A-IoT use cases are as follow:

    - max data rate is below 5kbps

    - max message size is below 1000bits

    - delay requirement in second level

    - no reliability requirement.

In addition, there is no requirement for AIoT devices to support parallel session/procedure, i.e. only single flow is supported. Therefore, the QoS requirement for different Messages should be the same which leads us to understand that there is no reason to introduce per packet QoS management functions.

Proposal 3: QoS requirements for AIoT devices are only defined for single flow, i.e. there is no support of parallel session/procedure neither per packet QoS.

## Indication of device message size/status (issue 7)

The issue is related to how to schedule the D2R message, and how the Reader is aware of the end of the D2R transmission.

At the RAN1#116b meeting, RAN1 discussed the issue, and concluded that:

Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information. Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information

RAN1 discussed this issue in RAN1#117 based on R1-2405380, but there was no additional conclusion between option 1 and 2.

For Option 2, the following two alternatives may be considered, which depends on whether the control information is indicated in the R2D or D2R transmission:

  - Alternative 1: reader determines the payload size intended for D2R transmission by the A-IoT device.

  - Alternative 2: reader lacks the knowledge of the payload size intended for D2R transmission by the A-IoT device.

Based on RAN2 agreements, legacy NR BSR/SR has been excluded. Therefore, the only solution for option 2 is message size which is similar to RAN1 discussion, i.e. TBS.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

In Option 1, a postamble may be appended after the PRDCH transmission to indicate the end of PRDCH transmission. In this case, the length of data packet may not be explicitly indicated in the control information for scheduling R2D transmission. However, for this option 1, given that A-IoT device needs to perform sequence detection on postamble, miss detection of postamble may lead to decoding failure of the data packet as A-IoT device may continue to decode the packet even if the reader already stops the transmission. Clearly, such a miss detection would also result in an unnecessary power consumption increase, which is not desirable for low power A-IoT device.

In Option 2, for R2D data transmission, data packet may be immediately transmitted after the L1 control information, which would carry certain scheduling information for the associated data transmission. The scheduling information may include the length of the data packet so that the device may derive the end of PRDCH transmission. As discussed in our companion contribution [3], separate CRCs may be appended for L1 control information and data packet in the PRDCH transmission. Under such a scenario, miss detection probability when using CRC in Option 2 would be lower compared to sequence detection in Option 1.

Observation 1. To indicate the end of PDRCH transmission, Option 1 of D2R postamble immediately following the PDRCH seems less preferable to Option 2 which relies on control information that carries the size as the miss detection probability in Option 1 (which uses separate CRCs) would be lower compared to sequence detection in Option 1. Moreover, when this miss detection happens for Option 1, this will also result in an unnecessary power consumption increase, which is not desirable for low power A-IoT device.

Given that the maximum payload size for A-IoT is 1000 bits, it is envisioned that a limited set of TBS may be considered for R2D transmission. In this case, if Option 2 is supported, the number of bits required to indicate the length of the packet or TBS may be relatively small. For instance, assuming 32-bit or 4-byte alignment for the data packet, 5-bit TBS indication would be sufficient.

Further, additional scheduling information, e.g., modulation and coding scheme if various data rate is supported for R2D transmission, A-IoT device ID or group ID may be included in L1 control information in the corresponding PRDCH transmission. In this regard, it is more appropriate to also include the field for the indication of length of packet in the L1 control information. This approach can help avoid the design of postamble for A-IoT, thereby minimizing specification effort.

Hence, in our view, Option 2, i.e., R2D control information is used to determine or derive the end of PRDCH transmission can be prioritized for A-IoT system design.

Observation 2. Option 2 relies on control information to carry the size of data. For this only a small number of bits is required as maximum payload is 1000bits. Control information could also be usable to convey scheduling information (e.g., modulation and coding scheme in used).

Similar to the determination of end of PRDCH transmission, for the reader to acquire the end of PDRCH transmission, Option 2, i.e., based on control information can be prioritized for study.

Proposal 4: For both the reader to acquire the end of PDRCH transmission and the device to acquire the end of PDRCH transmission, Option 2 which relies on control information to carry the size of data is preferable. I.e., R2D/D2R control information is used to determine or derive the end of PRDCH/PDRCH transmission.

## Visibility of AIoT information in RAN (issue 8)

RAN2 discussed what information should be visible in RAN, but no conclusion was made on this.

FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

To our understanding, the level of visibility and which information is actually required by the reader depends on whether the specific information is required by RAN/network, e.g. when determine the specific procedure to be used with the AIoT Device (s). For example, the following are information that might be helpful for RAN to know or have visibility:

- Multiple devices or single device: Random access procedure is needed if multiple devices are targeted, but random access procedure is not needed if only single device is requested;

- Device ID/Criteria: RAN needs to know the device ID/criteria, otherwise RAN has no idea to which Device the message shall be sent;

- Service type: for combined inventory and command, individual inventory and command, the procedure is different;

Proposal 5: The level of visibility and which information is actually required by the reader depends on whether specific information is required by RAN/network e.g., when determine the specific procedure to be used with the AIoT Device(s). This support (i.e., the level of visibility and which information) needs to be discussed based on the feature/functionality supported/enabled.

Proposal 6: Reader needs to have visibility to (1) whether the procedure is for multiple or single devices, (2) the Device ID/Criteria and (3) service type (i.e., whether the procedure is for combined inventory and command).

## Command only use case (issue 9)

In SA2 discussion, Key issue#3 consists of following objectives:

The key issue will study the following aspects: - Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices. NOTE 2: The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities. - Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

SA2 discussed the procedure and protocol stack for Inventory and Command under SA2 Key issue#3 and, there are many solutions under consideration in SA2 discussion.

SA2 Solution 17 is the only SA2 solution which is related to command only use case:

Solution 17:

- AIoT NAS protocol between AIoT device and AMF to support Inventory/Command;

- Separate procedures for Inventory and Command; The procedure is maintained by the AMF, therefore the Reader just needs to follow the request from AMF, i.e. Reader does not need to automatically perform an Inventory before sending the command to AIoT device

Figure 2 illustrates Command only procedure for one device (without separate selection command, inventory procedure, random access procedure):

Figure 2 Command only procedure for single device

- Step 0-2:CN triggered Inventory procedure and get the device ID from Reader;

- Step 3: Based on device ID, the CN generate the command for the device;

Command only procedure

- Step 4: CoreNetwork (CN) sends the command “Command” with Device ID, action (e.g. Read, write) and corresponding data to the Reader, to request Reader to perform Command for a Device under an Area;

- Step 5: Reader sends “Command” , with Device ID, action (e.g. Read, write) and corresponding data to request the device under an Area to perform the action;

- Step 6: The device will execute the command received from Reader

- Step 7: Device sends response back to Reader

- Step8: Reader sends “Command Response” back to CN;

Proposal 7: Command only procedure is considered during RAN2 study.

RAN2 impact from Command procedure for a single AIoT devices:

Same as inventory, for RFID, all commands are specified in the same layer as Tag identification layer. For A-IoT, the content of Command may not be visible to the Reader. But at least the Reader needs to be aware of the device ID, otherwise the Reader has no idea whether the response is from the same Device or not. Therefore Command, Device Response for Command procedure shall be specified in Data link layer (e.g. A-IoT MAC). Do not need to introduce Paging like message in both RRC and MAC specification. The relationship with Command defined in SA2 can be discussed once SA2 situation is clear.

Proposal 8: To support Command procedure for single device, the message “Command” and “Device Response” shall be specified in Data link layer (e.g. as part of A-IoT MAC) for the reader to differentiate/identify UE’s response to the command. FFS on the relationship between the message defined in RAN2 and SA2.

Proposal 9: The initial trigger Message for Command procedure for single device is “Command” (which is sent by the Reader to the AIoT device(s)). I.e., RAN2 will not introduce additional Paging like message or initial trigger message.

# Conclusion

Based on the discussion, we have following proposals:

Device type handling

Proposal 1: Capability based resource allocation is not supported. RAN should be aware of device type based on information provided by CN. Send LS to SA2 and Cc RAN3 on this.

Higher layer repetition

Proposal 2: To address a group of AIoT devices, Reader needs to send R2D message “Inventory request” multiple times.

Support of QoS

Proposal 3: QoS requirements for AIoT devices are only defined for single flow, i.e. there is no support of parallel session/procedure neither per packet QoS.

Indication of device message size/status

Observation 1. To indicate the end of PDRCH transmission, Option 1 of D2R postamble immediately following the PDRCH seems less preferable to Option 2 which relies on control information that carries the size as the miss detection probability in Option 1 (which uses separate CRCs) would be lower compared to sequence detection in Option 1. Moreover, when this miss detection happens for Option 1, this will also result in an unnecessary power consumption increase, which is not desirable for low power A-IoT device.

Observation 2. Option 2 relies on control information to carry the size of data. For this only a small number of bits is required as maximum payload is 1000bits. Control information could also be usable to convey scheduling information (e.g., modulation and coding scheme in used).

Proposal 4: For both the reader to acquire the end of PDRCH transmission and the device to acquire the end of PDRCH transmission, Option 2 which relies on control information to carry the size of data is preferable. I.e., R2D/D2R control information is used to determine or derive the end of PRDCH/PDRCH transmission.

Visibility of AIoT information in RAN

Proposal 5: The level of visibility and which information is actually required by the reader depends on whether specific information is required by RAN/network e.g., when determine the specific procedure to be used with the AIoT Device(s). This support (i.e., the level of visibility and which information) needs to be discussed based on the feature/functionality supported/enabled.

Proposal 6: Reader needs to have visibility to (1) whether the procedure is for multiple or single devices, (2) the Device ID/Criteria and (3) service type (i.e., whether the procedure is for combined inventory and command).

Command only use case

Proposal 7: Command only procedure is considered during RAN2 study.

Proposal 8: To support Command procedure for single device, the message “Command” and “Device Response” shall be specified in Data link layer (e.g. as part of A-IoT MAC) for the reader to differentiate/identify UE’s response to the command. FFS on the relationship between the message defined in RAN2 and SA2.

Proposal 9: The initial trigger Message for Command procedure for single device is “Command” (which is sent by the Reader to the AIoT device(s)). I.e., RAN2 will not introduce additional Paging like message or initial trigger message.

# Reference

- R1-2403966	Discussions on frame structure and timing aspects for A-IoT	 Intel

- R2-2406378	Consideration on initial trigger message	Intel

- R1-2406012	 AIoT PhyChannel	Intel