3GPP TSG-RAN WG2 Meeting #127						R2-2406378

Maastricht, The Netherlands, 19-23 August 2024

Agenda item:			8.2.3

Source:			Intel Corporation

Title:		Consideration on initial trigger message

Document for:	 	Discussion and decision

# Introduction

At RAN2#125bis, RAN2 made following agreements:

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

At RAN2#126, RAN2 made further agreements:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this contribution, we continue the discussion on FFS points for the initial message used to trigger.

# Discussion

## The content of Initial trigger message

### 2.1.1 Initial trigger message for Inventory procedure

The purpose of the Inventory procedure is to discover and acquire the identifier of a single or group of AIoT devices by a Reader as explained in our RAN2#125bis companion TDoc [1].

Figure 1. Inventory procedure for multiple devices

The Reader shall send the Inventory request to devices when receiving the Inventory operation request from CN, i.e. the initiate trigger message for Inventory procedure is Inventory request.

Proposal 1: The initial trigger message for Inventory procedure is Inventory request (which is sent by the Reader to the AIoT device(s)).

Proposal 1.1: Reader triggers the inventory procedure upon receiving inventory request from the CN.

The inventory request is used to request an AIoT device or a group of AIoT devices or all AoT devices to provide the ID. In addition, based on RAN2 conclusion, both contention based and contention free access could be applied. Therefore the following information should be needed:

- Selection criteria: Used to indicate which AIoT Device(s) should reply to the inventory request. It may consist of Device ID, Device ID with mask or particular address and the corresponding data stored in A-IoT device’s memory which can be detected by the A-IoT device. It may indicate a particular device or a group of devices.

- Control information: scheduling information to indicate the frequency/time resource for the AIoT device to respond.

- Access type, Contention based access or contention free based access

  - 2 Step or 3 Step access;

  - Random access control information (if RACH is needed): Counter or Backoff indication to distribute AIoT devices in different slots to trigger the RACH procedure.

Proposal 2: The Inventory request of proposal 1 may include selection criteria, control information and Access type (e.g. contention free access or contention-based access, Random-access control info, etc.)

### 2.1.2 Initial trigger message for Command procedure

For Command procedure, depending on SA2 design, the Reader may need to trigger Inventory procedure first in order to know which UE is under the range of the Reader, and then send the Command accordingly.

Figure 2. Command procedure for multiple devices

For Command procedure, the Reader may send the Inventory request to devices when receiving the Command operation request from CN, i.e. the initiate trigger message for Command procedure could be Inventory request or Command. This part is also related to SA2 discussion. RAN2 can only make a final decision once SA2 procedure is clear.

Proposal 3: From Reader perspective, the initial trigger message for Command procedure could be Inventory request or Command (which is sent by the Reader to the AIoT device(s)). FFS on further details (RAN2 needs to wait for SA2 progress).

It is still open on whether Command can be included as the first trigger message. To our understanding, we do not see the use case that the CN/Reader send the Command as the first trigger message, as this operation of reading/writing blindly to the devices under the reader’s coverage could lead to a waste of resources, and is difficult to also provide security protection on the message itself.

Observation 1. When CN/Reader sends Command as its first trigger message, it could lead to a waste of resources, and is difficult to also provide security protection of the message itself.

Proposal 4: Network (Reader or CN) shall only send Command to a device after performing successfully Inventory procedure with a device, i.e. command shall not be included as the first trigger message.

As discussed in [3], there could be Command only use case:

Figure 3 Command only procedure for single device

To support command only use case, an ID is needed between Reader/Device and CN to maintain the device context after Inventory. One way is to use permanent ID, i.e. device ID for this purpose. But it may cause security issue, and resource/energy issue since the size might be too big assuming the large number of AIoT devices under its network. Another option is to use a random ID which will be already introduced for random access procedure, and it could be easily reused for maintaining UE context purpose.

Proposal 5: The temporary ID (random ID) provided during Inventory procedure should be stored in the device, reader and CN. This temporary ID will be used later between Reader/device and CN to identify a device when performing command only procedure.

Based on proposal 4, contention-based access is not needed before the reader sends the command. Therefore, for Command Request message itself, following information may be needed:

- Temporary ID: to identify the AIoT device

- Operation Info: Read, Write (including data to be written), etc.

- Control info: scheduling information to indicate the frequency/time resource for the AIoT device to response.

On other hand, some of information may also be transparent to Reader if it is unrelated to AS layer, e.g. operation.

Proposal 6: The Command request should include temporary ID (of proposal 5), Operation info and control info. FFS on which information should be captured within AS layer and/or upper layer.

In last RAN2 meeting, companies discussed whether to introduce a common NR like paging message to cover both inventory and command. In our understanding, the following two scenarios need to be discussed in this context to understand what “common” would mean:

Scenario 1: Command is the first trigger message without Inventory. We do not see the need for this scenario 1 since before Inventory procedure, the CN has no idea which device is under the coverage of which Reader. Such blind command will just waste resources. For scenario 1, the common part for Inventory and Command could be “the RACH information”, selection criterion and control info, e.g. scheduling.

From paging and RACH functionality perspective, there is no big difference between “Inventory” and “command” However, from service functionality perspective, the content in the first message “paging” for  Inventory request and the command are different. Note: The Inventory message/command message from CN could be transparent in RAN as container or explicit information in MAC (RAN2 needs to discuss this).

If AIoT message provided by CN is contained as container in “paging” message, then the main difference should be the paging purpose and the necessary information for Inventory/command which is also related to the discussion on visibility in Reader.

On other hand, it could be understood as a modelling issue whether we still call it as “paging” or “inventory request” and “command” since it depends on whether we model the purpose as message code, which is similar to LCID in MAC or purpose field as RRC.

On summary, RAN2 will make a modelling message decision once the details of message and protocol is clearer.

Scenario 2: Command message can only be sent after successful inventory procedure. For this scenario 2, the only common part between Inventory and Command should be the control info, i.e. scheduling information.

RAN1 is still discussing the details of scheduling information and whether it should be part of physical layer. If it is part of the physical layer, then no common part between Inventory and command message.

Based on the above analysis, we do not see the need to introduce common “paging” message and AIoT Paging concept for now. We can use message name, e.g. Inventory Request, Command Request to discuss the initial trigger message, and decide whether a common message can cover multiple service operations once the details of messages and protocol are clear enough.

Proposal 7: Use the message name, Inventory Request, Command Request instead of AioT Paging when discussing the initial trigger message to avoid confusions.

Proposal 8: FFS whether to introduce a common (i.e., single) message for multiple service operations (e.g. inventory, read, write). RAN2 postpones this discussion until the details of actual message(s) and protocol(s) are clearer

## 2.3 Monitoring of DL message and determination of transmission/access occasion

In RAN2#125bis TDocs [2] [3] discussed the boundaries for AIoT device to send the response. To our understanding, there are two issues:

- Issue 1: when should the AIoT device monitor the R2D message?

- Issue 2: Determination of transmission/access occasion, i.e. when should the AIoT device transmit the response message?

### 2.3.1 When should the AIoT device monitor the R2D message

RAN2 has agreed that

Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

As defined in TS22.369:

Energy harvesting. An Ambient IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). Energy is provided through harvesting of radio waves, light, motion, heat, or any other suitable power source. Energy harvesting can be continuous or incidental (e.g. based on the vibration that a vibration sensor has to report). It cannot be assumed that Ambient IoT devices always have enough power to initiate or receive communication. Communication characteristics. Ambient IoT devices can have a variety of communication characteristics, different from other IoT devices, based on how the Ambient IoT devices are powered by energy harvesting and whether / how the harvested energy can be stored. Ambient IoT devices will only be able to communicate when they have enough power. This can be an issue, especially when communication is initiated towards the Ambient IoT device, while it is not known whether the Ambient IoT device has enough power to receive this communication. For communications initiated by the Ambient IoT device, the Ambient IoT device cannot transmit data until it has harvested / stored enough energy. Some Ambient IoT devices can be powered on demand when they need to communicate. Additionally, some Ambient IoT devices will be able to communicate on a regular basis and have communication characteristics similar to regular IoT devices.

At RAN2#126, RAN2 further agreed:

From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

RANP discussed the issue as well, but there was no conclusion. We expect that RAN1 will continue the discussion on this issue, and we suggest that RAN2 can just wait for further inputs from RAN1. To our understanding, AIoT device can get power based on energy harvesting, as a reasonable implementation, Reader or other network entities shall transmit radio wave to help AIoT device for energy harvesting. A simple way is that the network guarantees that there is enough time for AIoT device to get power, and an AIoT device shall always monitor the R2D message if it has enough energy to handle the operation. If the error case happens, i.e. an AIoT device has not enough energy to complete the procedure, the AIoT device should be allowed to skip the procedure and wait for the next round of operation. Therefore, we do not see the need to introduce additional solution to control when an AIoT device should monitor or not the R2D message. Said this, our preference is that the decision is driven by RAN1.

Proposal 9: The paging monitoring details should be concluded in RAN1 first before RAN2 starts the discussion.

### 2.3.3 Determination of transmission/access occasion

At the RAN1#117 meeting, two options were identified for study for the time interval between a R2D transmission and the corresponding D2R transmission, as illustrated in Figure 4.

- Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].

- Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

Figure 4. Time interval between a R2D transmission and the corresponding D2R transmission

Based on RAN1 agreements, we can see that:

RAN1 option 1:

- The transmission of D2R is always triggered by R2D message, and the AIoT device shall respond based on TR2D_min, and TR2D_max .

- The Reader can transmit next R2D message take into account of TD2R_min, TR2D_R2D_min and TD2R_D2R_min.

RAN1 option 2:

- The corresponding D2R transmission timing TD2R following a R2D transmission is determined based on the control information in the R2D transmission, where TD2R ³ TR2D_ min

From RAN2 perspective, we do not see any concern or blocking issue for any of RAN1 solutions. We suggest waiting for further inputs from RAN1.

Proposal 10: From RAN2 point of view, both options considered by RAN1 are feasible and with no concerns for an AIoT device to transmit D2R message as the response of R2D message in the period between two consecutive R2D messages within min/max of R2D response timing (RAN1 option 1) or based on the timing information determined from the control information in the R2D message (RAN1 option 2). RAN2 should wait for RAN1 conclusion on which option and the corresponding details.

# Conclusion

Based on the discussion, we have following proposals and observations:

The content of initial trigger message for Inventory:

Proposal 1: The initial trigger message for Inventory procedure is Inventory request (which is sent by the Reader to the AIoT device(s)).

Proposal 1.1: Reader triggers the inventory procedure upon receiving inventory request from the CN.

Proposal 2: The Inventory request of proposal 1 may include selection criteria, control information and Access type (e.g. contention free access or contention-based access, Random-access control info, etc.)

The content of initial trigger message for Command

Proposal 3: From Reader perspective, the initial trigger message for Command procedure could be Inventory request or Command (which is sent by the Reader to the AIoT device(s)). FFS on further details (RAN2 needs to wait for SA2 progress).

Observation 1. When CN/Reader sends Command as its first trigger message, it could lead to a waste of resources, and is difficult to also provide security protection of the message itself.

Proposal 4: Network (Reader or CN) shall only send Command to a device after performing successfully Inventory procedure with a device, i.e. command shall not be included as the first trigger message.

Proposal 5: The temporary ID (random ID) provided during Inventory procedure should be stored in the device, reader and CN. This temporary ID will be used later between Reader/device and CN to identify a device when performing command only procedure.

Proposal 6: The Command request should include temporary ID (of proposal 5), Operation info and control info. FFS on which information should be captured within AS layer and/or upper layer.

Common paging message for Inventory and Command

Proposal 7: Use the message name, Inventory Request, Command Request instead of AioT Paging when discussing the initial trigger message to avoid confusions.

Proposal 8: FFS whether to introduce a common (i.e., single) message for multiple service operations (e.g. inventory, read, write). RAN2 postpones this discussion until the details of actual message(s) and protocol(s) are clearer

When should the AIoT device monitor the R2D message:

Proposal 9: The paging monitoring details should be concluded in RAN1 first before RAN2 starts the discussion.

Determination of transmission/access occasion:

Proposal 10: From RAN2 point of view, both options considered by RAN1 are feasible and with no concerns for an AIoT device to transmit D2R message as the response of R2D message in the period between two consecutive R2D messages within min/max of R2D response timing (RAN1 option 1) or based on the timing information determined from the control information in the R2D message (RAN1 option 2). RAN2 should wait for RAN1 conclusion on which option and the corresponding details.

# Reference

- R2-2404192	Overall procedures for Inventory and Command use cases, Intel

- R2-2402548	Discussion on random access for Ambient IoT	CMCC

- R2-2403114	Random access-like procedure for Ambient IoT	Huawei, HiSilicon

- R2-2406377 Required functions for A-IoT