3GPP TSG-RAN WG2 Meeting #127						R2-2406379

Maastricht, The Netherlands, 19-23 August 2024

Agenda item:			8.2.4

Source:			Intel Corporation

Title:		Consideration on A-IoT Random access

Document for:	 	Discussion and decision

# Introduction

In RAN2#125bis and RAN2#126, RAN2 discussed the random access for AIoT device and made good progress.

Chair’s guidance on this topic is “Contributions should focus on A-IoT random access steps for both 2-step and 3-steps RA, content required for the different procedures, and any additional aspects related to CFRA and CBRA procedures.  Failure handling (i.e. msg 3 or paging) will be treated in this AI.”

Based on previous discussions in RAN2, following detailed issues should also be addressed:

- Size of random ID, how to generate the random ID

- 2 step CB RA, FFS what device ID is and whether an additional random ID is needed. A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

- contention-free access procedure (how to address the device, temporary ID or permanent ID)

In this contribution, we continue the discussion on those open issues.

# Discussion

## 3 step RA

In RAN2#126, RAN2 agreed:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

RAN2 agreed 3 step RA in last meeting which is similar to the RACH procedure defined in RFID.

Based on RFID RACH, the RACH procedure for AIoT is illustrated in the figure 1 as below:

Figure 1: AIoT RACH procedure

In particular, the following steps may be included in AIoT RACH procedure:

- Step 0: reader may send an Inventory Request periodically to a group of A-IoT devices. The Inventory Request message may contain specific broadcast information, e.g., the backoff indication as defined for NR. Further, the reader may adjust the backoff indication in the following Inventory message, which may depend on the collision rate that is detected in the A-IoT system. The Inventory request/“paging” should include:

  - Selection criteria: Used to indicate which AIoT Device(s) should reply to the inventory request. It may consist of Device ID, Device ID with mask or particular address and the corresponding data stored in A-IoT device’s memory which can be detected by the A-IoT device. It may indicate a particular device or a group of devices.

  - Control information: scheduling information to indicate the frequency/time resource for the AIoT device to respond.

  - Access type, Contention based access or contention free based access

    - 2 Step or 3 Step access;

    - Random access control information (if RACH is needed): Counter or Backoff indication to distribute AIoT devices in different slots to trigger the RACH procedure.

- Step 1: According to the backoff indication, the group of A-IoT devices may randomly generate a backoff counter and perform random backoff for transmitting the first message to the reader. In this step, a contention resolution ID that is randomly generated from the A-IoT devices can be included in the first D2R message during the contention-based access procedure.

Note-1: when an A-IoT device receives the Inventory message, it may decrement the counter by 1 for random backoff.

Note-2: the contention resolution ID used in RFID is 16 bits. The probability of collision defined in RFID is shown as below:

Probability of simultaneously identical sequences: For a Tag population of up to 10,000 Tags, the probability that any two or more Tags simultaneously generate the same sequence of RN16s shall be less than 0.1%, regardless of when the Tags are energized.

Considering the density requirements described in the SID is “150 devices per 100 m2 for indoor scenarios” and the coverage requirement is “the maximum distance of 10 – 50 m for indoor”, the maximum number of devices in the coverage for indoor scenario should be around 376 devices to 11775 devices. Therefore if 16 bits RN is used, the worst collision probability should still be less than 0.1% which is similar to RFID.

Observation 1: 16 bits RN can achieve similar performance as RFID for all scenarios defined in 3GPP.

- Step 2 (contention resolution): After correctly receiving the initial D2R message from A-IoT device, the reader would send the acknowledgement message that may include the decoded contention resolution ID. If the contention resolution ID is same as the one sent by the device in step 1, contention resolution is successful, and the device is allowed to send the additional message with data. Device sends “Device Response” with Device ID, Data, etc information back to Reader;

Proposal 1: For A-IoT 3 step RACH procedure, the following aspects can be considered:

  - Step 0: R2D message “Inventory Request” or “Paging” is used to trigger the AIoT RACH procedure (sent by the Reader to the AIoT device(s)). Following information shall be included:

  - Control information: scheduling information to indicate the frequency/time resource for the AIoT device to respond.

  - Access type, Contention based access or contention free based access

    - 2 Step or 3 Step Access;

    - Random access control information (if RACH is needed): Counter or Backoff indication to distribute AIoT devices in different slots to trigger the RACH procedure.

  - Step 1: AIoT devices randomly generate a backoff counter based on indicated backoff values for contention-based access. 16 bits Contention resolution ID which is carried in the initial D2R message “A-IoT RACH MSG 1” for AIoT RACH (sent by the AIoT device to the Reader).

  - Step 2 (which has already been agreed): Reader generates R2D message “A-IoT RACH MSG 2” for AIoT RACH Contention resolution, include Contention resolution ID. Reader sends this “A-IoT RACH MSG 2” to the AIoT device.

## 2 step RA

In RAN2#126, RAN2 agreed:

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

The main question is whether Msg 2 is needed, and whether additional random ID is needed.

To our understanding, the main motivation to introduce 2 step RACH is that the device can send device ID at the first message instead of Msg1 for RACH and Msg 3 for ID as defined in 3 step RACH. The benefit for this option is to reduce steps, and therefore:

- It can reduce the energy consumption

- It can also reduce the E2E latency

However, if the radio channel situation is bad, e.g. in congestion situation, it will increase the congestion situation since it needs more resources to send the Device ID in the first message for contention resolution. Therefore, the network shall only use this option when the radio channel situation is good, i.e. the load and the collision ratio are both low.

Proposal 2: 2 step RACH shall only be used for low load and low collision scenario.

For contention resolution, there are two options:

Option 1: same as 3 step RACH, device ID is contained in Msg 2 as contention resolution. The device ID could be permanent ID or random ID.

As discussed in next section 2.3 on failure handling, when contention resolution fails or the network does not receive/decode D2R message successfully, a device shall wait for next round of procedure triggered by the network. Therefore, we may not need Msg2 at all for 2 step RACH, i.e. option 2

Option 2: Do not introduce Msg2 R2D message as contention resolution for 2 step RACH. If the network does not receive/decode D2R message successfully, the network should trigger the procedure again, and the device only needs to act based on network’s request.

Option 2 is simple and can reduce a message from network which looks preferable over option 1. In addition, if Device ID is included in the Msg1, it can be treated as the response of Msg0. Therefore, we do not see a strong need to introduce Msg 2.

Proposal 3: Not to introduce Msg2 for 2 step RACH (since Msg 1 is the response of Msg 0-Inventory request).

If temporary ID is used between device/Reader and CN for procedure after the inventory, random ID shall be introduced in Msg 1.

Proposal 4: For 2 step RACH, temporary ID is included in Msg 1 if it is used between device/Reader and CN for the procedure after Inventory.

## Failure handling

[1] discussed how to handle the case if the AIoT device has no power to complete the required operation. “The simple way is that the network guarantees that there is enough time for AIoT device to get power, and an AIoT device always monitors the R2D message if it has enough energy to handle the operation. As described in [1], AIoT is an unsynchronized system since there is not accurate timing  information. SFN.. The AIoT device can only know the timing based on DL message e.g. our companion TDoc [1] explains  “Proposal 10: From RAN2 point of view, both options considered by RAN1 are feasible and with no concerns for an AIoT device to transmit D2R message as the response of R2D message in the period between two consecutive R2D messages within min/max of R2D response timing (RAN1 option 1) or based on the timing information determined from the control information in the R2D message (RAN1 option 2). RAN2 should wait for RAN1 conclusion on which option and the corresponding details.” Therefore if the error case happens, i.e. an AIoT device has no enough energy to complete the procedure, the DL message is the triggering for the AIOT device to know when to transmit the UL for the same session, i.e., the AIoT device should just skip the procedure and wait for the next round of operation.”.

Proposal 5: AIoT device shall wait for the reader to trigger the next round of operation when it does not have enough energy to complete the requested operation.

For RACH related failure cases:

- Case 1 (contention resolution failure): Collision with other devices, and therefore the RN received in ACK is different from RN transmitted by the AIoT device;

- Case 2 (others): no response from the Reader in the period between two consecutive R2D messages within min/max of R2D response timing;

For these RACH related failure cases, same handling can apply, i.e. the AIoT device could skip the procedure and wait for the next round of operation.

Proposal 6. AIoT RACH Failure includes case (1) failure of contention resolution (i.e. a collision with other devices that leads to the RN to be received in an ACK which is RN of a different AIoT device) and case (2) other failure scenarios (e.g., when AIoT device does not respond from the Reader in the period between two consecutive R2D messages within min/max of R2D response timing).

Proposal 7: AIoT device shall wait for the reader to trigger the next round of operation upon detection of an AIoT RACH failure.

## ID used during contention free access

To our understanding, the contention free access should be used when only one device is addressed. Normally it should be the time after Inventory procedure, i.e. command operation after inventory. As discussed in [2]

To support command only use case, an ID is needed between Reader/Device and CN to maintain the device context after Inventory. One way is to use permanent ID, i.e. device ID for this purpose. But it may cause security issue, and resource/energy issue since the size is too big. Considering a random ID will be introduced for random access procedure, it can be reused for maintaining UE context purpose. Proposal 5: The temporary ID (random ID) is used between Reader/device and CN to identify a device after Inventory procedure.

Therefore, it is also applied for contention free access.

Proposal 8: The temporary ID (random ID) is used between Reader/device and CN to identify a device when contention free access is performed.

# Conclusion

Based on the discussion, we have following proposals:

AIoT 3 step RACH procedure design

Observation 1: 16 bits RN can achieve similar performance as RFID for all scenarios defined in 3GPP.

Proposal 1: For A-IoT 3 step RACH procedure, the following aspects can be considered:

  - Step 0: R2D message “Inventory Request” or “Paging” is used to trigger the AIoT RACH procedure (sent by the Reader to the AIoT device(s)). Following information shall be included:

  - Control information: scheduling information to indicate the frequency/time resource for the AIoT device to respond.

  - Access type, Contention based access or contention free based access

    - 2 Step or 3 Step Access;

    - Random access control information (if RACH is needed): Counter or Backoff indication to distribute AIoT devices in different slots to trigger the RACH procedure.

  - Step 1: AIoT devices randomly generate a backoff counter based on indicated backoff values for contention-based access. 16 bits Contention resolution ID which is carried in the initial D2R message “A-IoT RACH MSG 1” for AIoT RACH (sent by the AIoT device to the Reader).

  - Step 2 (which has already been agreed): Reader generates R2D message “A-IoT RACH MSG 2” for AIoT RACH Contention resolution, include Contention resolution ID. Reader sends this “A-IoT RACH MSG 2” to the AIoT device.

AIoT 2 step RACH procedure design

Proposal 2: 2 step RACH shall only be used for low load and low collision scenario.

Proposal 3: Not to introduce Msg2 for 2 step RACH (since Msg 1 is the response of Msg 0-Inventory request).

Proposal 4: For 2 step RACH, temporary ID is included in Msg 1 if it is used between device/Reader and CN for the procedure after Inventory.

Failure handling

Proposal 5: AIoT device shall wait for the reader to trigger the next round of operation when it does not have enough energy to complete the requested operation.

Proposal 6. AIoT RACH Failure includes case (1) failure of contention resolution (i.e. a collision with other devices that leads to the RN to be received in an ACK which is RN of a different AIoT device) and case (2) other failure scenarios (e.g., when AIoT device does not respond from the Reader in the period between two consecutive R2D messages within min/max of R2D response timing).

Proposal 7: AIoT device shall wait for the reader to trigger the next round of operation upon detection of an AIoT RACH failure.

ID used during contention free access

Proposal 8: The temporary ID (random ID) is used between Reader/device and CN to identify a device when contention free access is performed.

# Reference

- R2-2404194	Consideration on initial trigger message	Intel

- R2-2406378	Consideration on initial trigger message	Intel