3GPP TSG-RAN WG2 Meeting #127						R2-2406380

Maastricht, The Netherlands, 19-23 August 2024

Agenda item:			8.2.5

Source:			Intel Corporation

Title:		Topology 2 considerations

Document for:	 	Discussion and decision

# Introduction

In RAN#104, based on [2], RAN discussed Architecture for Topology 2 and concluded that:

RP-241612 Moderator's summary for offline discussion about ambiant IoT Huawei RP-241688 Moderator's summary for offline discussion about ambiant IoT Huawei Replaces RP-241612 conclusion: all 3 conclusions of section 4 are endorsed

In this contribution, we analyse the RAN2 impact for Topology 2.

# Discussion

During RANP study phase, 5 deployment scenarios and 4 topologies were considered. Based on the SI [1], only Topologies 1 and 2 are considered in this study.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor

Figure 1. Topology 1 and 2 for A-IoT applications

At RAN2#125bis, RAN2 aimed to have a common (i.e. single) design on the interface between reader and A-IoT device for topology 1 and topology 2.

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies.

The main difference between topology 1 and 2 on the interface between device and intermediate node (i.e. Reader) is that, with the help of intermediate node, the coverage of topology 2 could be smaller than topology 1. As discussed in [3], the network needs to determine the suitable topology based on UE type. However, no RAN2 impact is foreseen.

Based on RAN1 discussion, type 1, type 2a and type 2b devices may be supported in Rel-19. The coverage may be different for different type of A-IoT devices. For instance, coverage of type 2 device should be larger than type 1 device, and therefore for type 1 device, Topology 2 may be needed in order to extend the coverage. However, considering that low cost, low complexity and low power are key metrics for A-IoT devices, there should not be huge difference among different type of devices from capability perspective. Therefore, capability-based resources allocation should be avoided. On other hand, RAN2 needs to consider how network can be aware of topology 1 or 2 should be used. To our understanding, the feasible way is to let the CN provide the device type information to RAN before RAN sending the initial trigger message. Based on device type information, RAN can decide whether topology 1 or 2 should be used for a device. Proposal 1: Capability based resource allocation is not supported. RAN should be aware of device type based on information provided by CN.

Observation 1: The topology is selected based on UE type, which can be done via CN without RAN2 impact.

[2] discussed the support of topology 2 as:

RP-241270 summarized discussions in RAN3 on Ambient IoT architecture, and remarked that RAN2 should be involved in discussions on functionalities of UE Reader and its architecture, considering the UE Reader is just a UE with AIoT capabilities. Proposals from Tdocs - [RP-241270] In Topology 2, the UE Reader (i.e., intermediate node) should be just considered as a UE with AIoT capabilities and not considered as part of an “AIoT RAN” (i.e., not as an infrastructure element) - [RP-241270] Revise the SID to explicitly capture that RAN2 should be involved in the architecture discussions for Topology 2 e.g., - RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. RAN2 is also expected to be involved in the architecture discussions for Topology 2. Moderator’s proposal - Discuss the [TBD] proposal from RAN2 Chair and RAN3 Chair. Offline conclusion: Clarification for the RAN#104 report RAN3 is the responsible WG for RAN architecture discussion on Ambient IoT. Coordination between RAN2 and RAN3 can be achieved by LS when needed on aspects that impact UE as intermediate node.

The main impact to RAN2 on topology 2 would be the interface (Uu) and corresponding handling between the intermediate node (i.e. Reader) and the RAN. It is under RAN2 scope, i.e. RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

Proposal 1: RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

Since Uu interface is used between the intermediate node (UE) and RAN, it is natural to reuse RRC layer to capture AS layer related configurations for topology 2, e.g. resources (frequency, timing, etc) used for A-IoT.

Proposal 2: RRC is reused to configure AS layer related configurations ( e.g., resources (including frequency, timing, etc) used dedicated for A-IoT) to the intermediate node (i.e. Reader).

As in the discussion in [3] (with related reference copied here below), the Reader needs to know whether the procedure is for multiple devices or single device and corresponding device ID/criteria, which applies for topology 2 as well.

RAN2 discussed what information should be visible in RAN, but no conclusion was made on this. FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. To our understanding, the level of visibility and which information is actually required by the reader depends on whether the specific information is required by RAN/network, e.g. when determine the specific procedure to be used with the AIoT Device (s). For example, the following are information that might be helpful for RAN to know or have visibility: - Multiple devices or single device: Random access procedure is needed if multiple devices are targeted, but random access procedure is not needed if only single device is requested; - Device ID/Criteria: RAN needs to know the device ID/criteria, otherwise RAN has no idea to which Device the message shall be sent; - Service type: for combined inventory and command, individual inventory and command, the procedure is different; Proposal 5: The level of visibility and which information is actually required by the reader depends on whether specific information is required by RAN/network e.g., when determine the specific procedure to be used with the AIoT Device(s). This support (i.e., the level of visibility and which information) needs to be discussed based on the feature/functionality supported/enabled. Proposal 6: Reader needs to have visibility to (1) whether the procedure is for multiple or single devices, (2) the Device ID/Criteria and (3) service type (i.e., whether the procedure is for combined inventory and command).

Proposal 3: The visibility information in RAN for topology 1 is also applicable for topology 2.

Proposal 4: For topology 2, the intermediate node (i.e. Reader) needs to have visibility to (1) whether the procedure is for multiple or single devices, (2) the Device ID/Criteria and (3) service type (i.e., whether the procedure is for combined inventory and command).

The protocol stack to support topology 2 could be:

Figure 2. Protocol stack for A-IoT (UE as Reader)

Note: The details of A-IoT layers in UE, intermediate node (i.e., Reader), RAN and CN should be decided in SA2.

Similar to topology 1, the information in Proposal 3 and 4 shall come from CN. However, there is no conclusion in SA2 on whether the intermediate node (i.e., reader) can get the information directly from the CN, e.g. via NAS, or get the information from the RAN. RAN2 should wait for further inputs from SA2 on this before making decision.

Proposal 5: RAN2 should wait for SA2 further inputs before making decision on the protocol stack and how the intermediate node (i.e., reader) obtain the AIoT service related information since they are tightly related to SA2 discussion.

# Conclusion

Based on the discussion, we have following proposals:

Observation 1: The topology is selected based on UE type, which can be done via CN without RAN2 impact.

Proposal 1: RAN2 is responsible for the interface between intermediate node (i.e., Reader) and RAN for topology 2.

Proposal 2: RRC is reused to configure AS layer related configurations ( e.g., resources (including frequency, timing, etc) used dedicated for A-IoT) to the intermediate node (i.e. Reader).

Proposal 3: The visibility information in RAN for topology 1 is also applicable for topology 2.

Proposal 4: For topology 2, the intermediate node (i.e. Reader) needs to have visibility to (1) whether the procedure is for multiple or single devices, (2) the Device ID/Criteria and (3) service type (i.e., whether the procedure is for combined inventory and command).

Proposal 5: RAN2 should wait for SA2 further inputs before making decision on the protocol stack and how the intermediate node (i.e., reader) obtain the AIoT service related information since they are tightly related to SA2 discussion.

# Reference

- RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, December 2023

- RP-241688 Moderator summary on R19 Ambient IoT

- R2-2406377 Required functions for A-IoT, Intel