3GPP TSG RAN WG2 Meeting #127               	                	                          R2-2406382

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda Item:	8.1.2.2

Source:	Intel Corporation

Title:	Beam management UE-sided model LCM signaling

Document for:	Discussion and Decision

# Introduction

During RAN2 #126 meeting, following agreements were made for UE-side model LCM:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Understanding of supported/configured/applicable functionalities and the detail signallings of proactive and reactive reporting were also further discussed in [POST126][032][AI/ML PHY] LCM (Intel/Samsung).

In this contribution, we mainly discuss further details of beam management UE-side model life cycle management aspects.

# Discussion

## Configured Functionality Terminology and Relationship

Following terminologies with companies’ consensus were proposed during Phase 1 discussion of [POST126][032][AI/ML PHY] LCM (Intel/Samsung).

Proposal 1: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) signalling. Proposal 3: Applicable functionalities refers to functionalities that the UE is ready to apply for model inference. Proposal 5: Activated functionalities refers to functionalities already activated enabled for performing inference.

There were also some discussions during Phase 1 regarding to definition of configured functionalities.

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed. Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities. Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In our view, understanding of configured functionalities for AI/ML is the same as legacy feature, which means the functionalities for which UE receives full configuration that allows UE to perform AI/ML beam management inference operation.

Regarding to the relationship between configured functionality and applicable functionality, it has two different understandings, depending on when the configuration is provided to UE.

| Scenario 1: Full inference configuration is provided after applicable functionality reporting | Scenario 2: Full inference configuration is provided before applicable functionality reporting |
| --- | --- |
| Network only provides configurations to the applicable functionalities, which can help network to save efforts by reducing unnecessary configurations. In this scenario, configured functionalities are subset of applicable functionalities. | Besides NW-side additional condition, UE may decide its applicable functionalities by also considering full inference configuration provided by the network. In this scenario, applicable functionalities are subset of configured functionalities. |

Recalling RAN2 agreed to support proactive reporting, which makes Scenario 1 is always a valid scenario.

However, since RAN2 also agreed to support reactive reporting, hence, it is possible that both scenarios and types of relationship between configured functionalities and applicable functionalities may exist. As summarized in Phase 2 of [POST126][032][AI/ML PHY] LCM (Intel/Samsung), whether sends full inference configuration to UE before applicable functionality reporting is still FFS and up to RAN1 discussion on the understanding of NW-side additional condition. It seems this relationship can be naturally concluded after RAN2 has a clear understanding on whether full inference configuration is sent to UE before applicable functionality reporting or not. Considering the relationship doesn’t have any specification impact, it seems there’s no need to discuss it explictly.

Proposal 1: Same as legacy features, configured functionalities for AI/ML BM means the functionalities for which UE receives the full inference configuration (e.g. CSI-RS resource, etc) that allows UE to perform AI/ML beam management inference operation.

Proposal 1-1: Configured functionalities can either be universe or subset of applicable functionalities, depending on whether full inference configuration will be provided to UE before applicable functionality reporting or not.

## Applicable Functionality Reporing

NW-side Additional Condition and Inference Configuration

As proposed in Phase 2 of [POST126][032][AI/ML PHY] LCM (Intel/Samsung), it is FFS on the RRC signaling for NW-side additional condtion and inference configuration provided before applicable functionality reporting.

Recalling that the main objective of providing NW-side additional condition is to keep consistency between training and inference for UE-sided model. For UE-sided model, it might be difficult for UE to train a model with generalization performance, which is different from NW-sided model.

For a functionality without generalized performance or a functionality optimized for a specific scenario, a model (Model A) measuring Set B of beams with beam index a-b to predict Set A of beams with beam index a-d might be different from the model (Model B) measuring Set B of beams with beam index c-d to predict Set A of beam with beam index c-f. Although the number of Set A and Set B beams are the same, UE may consider using different models/functionalities for different network configurations. Therefore, UE may at least need to understand some configurations related to the consistency of Set A and Set B beams (especially for measurement) across training and inference. For example, CSI-RS resource configuration of Set A and Set B, etc.

Observation 1: Before applicable functionality reporting, it is helpful for UE to understand some configurations (e.g. CSI-RS resource configuration) related to consistency of Set A/B beams across training and inference.

Inference configuration and NW-side additional condition may not be mandatory for UE to decide applicable functionality.

On one hand, knowing inference configuration and NW-side additional condition can help UE to reduce the message size of applicable functionality reporting, since such information can help UE further down-select among supported functionalities before reporting applicable functionalities.

However, we also noticed that RAN1 hasn’t clearly define what is NW-side additional condition and how UE understands if the same associated ID is provided. Furthermore, there’s also no agreement on the content of inference configuration. For those details, RAN2 can wait for further RAN1 progress.

Proposal 2: NW-side additional condition (e.g. associated ID) and inference configuration (e.g. CSI-RS resource config of Set A/B, etc) are optionally sent to UE before applicable functionality reporting.

Proposal 2-1: It is up to RAN1 on the relationship between NW-side additional condition (e.g. associated ID) and inference configuration, and the content of inference configuration provided before applicable functionality reporting. RAN2 wait for further progress in RAN1 to decide RRC signaling of NW-side additional condition and inference configuration.

On the other hand, if that consistency information (NW-side additional condition and configuration for Set A/B beams) supported by network are not known by UE before applicable functionality reporting, UE can still report applicable functionalities based on UE internal information (e.g. model availability, UE-side additional condition, etc). The UE may also need to report the corresponding NW-side additional condition and configuration of the applicable functionality (as model meta info) to network, so that the network can provide suitable configuration to the applicable Set A/B of beams. If such meta info associated to applicable functionalities are not reported by UE, network may configure UE blindly and the configured Set A/B beams may not be supported by the UE. Since the configuration cannot be applied by UE, UE may need to reestablish RRC connection, which introduces unnecessary network interruption.

Proposal 3: If consistency information (e.g. NW-side additional condition and/or inference configuration) is not provided to UE before applicable functionality reporting, UE needs to report meta info (e.g. NW-side additional condition and/or inference configuration) of the corresponding applicable functionality to network, together with applicable functionality reporting.

Signaling of applicable functionality reporting

Then, the question is what RRC signaling is used for applicable functionality reporting? There are two options proposed in Phase 2 of [POST126][032][AI/ML PHY] LCM (Intel/Samsung).

Option 1: UAI

Option 2: RRCReconfigurationComplete

In our understanding, the key difference between two options is whether there’s a need for UE to report its applicable functionality immediately after receiving network configuration.

In existing UAI framework, UE can report UE assistance information in two different approaches:

      - UAI without network restriction on reporting information:

UE reports UE assistance information if it is configured so and the reporting information is defined in UEAssistanceInformation. Network can provide configurations on when UAI can be reported, e.g. prohibit timer, etc.

Examples: Overheating assistance information, delay budget reporting, etc.

      - UAI with network configured restriction on reporting information:

Network first provides a list of candidate configuration (i.e. network cared/supported values) for UE assistance information reporting in OtherConfig, based on which, UE further reports its preference in UEAssitanceInformation within the configured candidate values.

Examples: Candidate serving frequency list in IDC assistance information.

If NW-side additional condition and inference configuration are provided before UE’s applicable functionality reporting, it can be considered similar to the 2nd approach, i.e. UAI with network configured restriction on reporting information, by providing a candidate list of network-cared inference configurations to UE. The configuration and NW-side additional condition can be considered as a filtering information for applicable functionality reporting.

With above observation, it seems the existing UAI framework can be used to report applicable functionality with or without network-configured NW-side additional condition and/or inference configuration.

Proposal 4: Existing UAI is used to report applicable functionality, no matter NW-side additional condition and/or inference configuration is provided to UE before reporting or not.

Furthermore, companies also consider that RRCReconfigurationComplete message can be used during UE’s handover. However, the applicability of a UE-side model at a neighbouring cell is questionable. Note that, the UE decides the applicability based on following conditions:

1) NW-side additional condition and inference configuration (if available)

2) UE-side additional condition (e.g. memory, power, etc)

3) model availability

It is difficult to make sure that NW-side additional condition and inference configuration are the same across different cells. Then, whether the current applicable functionalities at the serving cell can still be applicable at the serving cell is a question. Furthermore, during RAN1 simulation in Rel-18, only intra-cell beam management scenario was studied. Considering BM functionality cannot predict beams of a neighbouring cell, AI/ML beam management inference operation does not need to be activated during handover.

Alternatively, UE can deactivate AI/ML functionalities during handover and report applicable functionality again to the target cell per network configuration. Before new applicable functionalities are activated by the target cell, the UE keeps using legacy functionalities without AI/ML. There’s no requirement to continuously using AI/ML functionality during handover and reactivates AI/ML functionality immediately afterwards.

Proposal 5: UE deactivates AI/ML functionalities during handover and reports applicable functionality via UAI again to the target cell per target cell’s configuration.

## Management

Initial Activation

It is proposed in Phase 2 of [POST126][032][AI/ML PHY] LCM (Intel/Samsung) that the funcitonlaity is initially activated by receiving its configuration after applicable functionality reporting, if such inference configuration is not provided to UE.

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

On one hand, if the network provides inference configuration after applicable functionality reporting, it is straightforward that the functionality is activated upon receiving network configuration, especially considering the configuration includes all information that is needed by the UE for inference operation. Network considers the functionality is successfully activated upon receiving RRCReconfigurationComplete message (i.e. after Step 5).

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided after applicable functionality reporting. Network considers the functionality is successfully activated upon receiving the corresponding RRCReconfigurationComplete message.

On the other hand, if the network provides inference configuration before applicable functionality reporting, considering the inference configurations provided by network are mainly provided based on known supported functionalities, it is not clear how the AI/ML functionality is activated at the UE side.

Since the applicable functionality is decided at the UE side, with all inference configuration known by the UE, UE can consider this functionality is activated once UE decides this functionality is applicable internally and reports to the network. Network can further consider the functionality is activated upon receiving applicable functionality reporting.

Proposal 7: If configuration is provided before applicable functionality reporting, the functionality is initially activated when UE reports applicable functionality to network. Network considers the functionality is successfully activated upon receiving applicable functionality reporting.

Meaning of Deactivation

During last RAN2 meeting, the meaning of deactivation is not clear.

RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc.

It’s not clear whether multiple functionalities of one sub-use case (e.g. BM Case 1) can activated simultaneously or not, also it’s not clear whether multiple activated functionalities can predict the same Set A of beams. If yes, then deactivation of one functionality means switching to another functionality. Otherwise, UE needs to fall back to legacy on the measurements for Set A of beams, which were previously predicted by UE-side model. However, UE may still have activated model to predict other set of beams. All above assumptions depend on RAN1 discussion.

If single functionalities of one sub-use case is activated, deactivation of functionality means fall back to legacy (i.e. none-AIML).

Proposal 8: Deactivation can refer to 1) fall back to legacy (none-AI/ML) 2) measurement of Set A of beams fall back to legacy (AI/ML for predicting other set of beams is still activated) 3) switching to another functionality to predict same Set A of beams. The exact meaning is up to RAN1’s decision on whether multiple functionalities of one sub-use case (e.g. BM Case 1) can activated simultaneously.

# Conclusion

In this contribution, we mainly discussed the remaining open issues of applicable functionality reporting.

We have following observations and proposals:

Proposal 1: Same as legacy features, configured functionalities for AI/ML BM means the functionalities for which UE receives the full inference configuration (e.g. CSI-RS resource, etc) that allows UE to perform AI/ML beam management inference operation.

Proposal 1-1: Configured functionalities can either be universe or subset of applicable functionalities, depending on whether full inference configuration will be provided to UE before applicable functionality reporting or not.

Observation 1: Before applicable functionality reporting, it is helpful for UE to understand some configurations (e.g. CSI-RS resource configuration) related to consistency of Set A/B beams across training and inference.

Proposal 2: NW-side additional condition (e.g. associated ID) and inference configuration (e.g. CSI-RS resource config of Set A/B, etc) are optionally sent to UE before applicable functionality reporting.

Proposal 2-1: It is up to RAN1 on the relationship between NW-side additional condition (e.g. associated ID) and inference configuration, and the content of inference configuration provided before applicable functionality reporting. RAN2 wait for further progress in RAN1 to decide RRC signaling of NW-side additional condition and inference configuration.

Proposal 3: If consistency information (e.g. NW-side additional condition and/or inference configuration) is not provided to UE before applicable functionality reporting, UE needs to report meta info (e.g. NW-side additional condition and/or inference configuration) of the corresponding applicable functionality to network, together with applicable functionality reporting.

Proposal 4: Existing UAI is used to report applicable functionality, no matter NW-side additional condition and/or inference configuration is provided to UE before reporting or not.

Proposal 5: UE deactivates AI/ML functionalities during handover and reports applicable functionality via UAI again to the target cell per target cell’s configuration.

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided after applicable functionality reporting. Network considers the functionality is successfully activated upon receiving the corresponding RRCReconfigurationComplete message.

Proposal 7: If configuration is provided before applicable functionality reporting, the functionality is initially activated when UE reports applicable functionality to network. Network considers the functionality is successfully activated upon receiving applicable functionality reporting.

Proposal 8: Deactivation can refer to 1) fall back to legacy (none-AI/ML) 2) measurement of Set A of beams fall back to legacy (AI/ML for predicting other set of beams is still activated) 3) switching to another functionality to predict same Set A of beams. The exact meaning is up to RAN1’s decision on whether multiple functionalities of one sub-use case (e.g. BM Case 1) can activated simultaneously.