3GPP TSG RAN WG2 Meeting #127              	                	                          R2-2406383

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.3

Source:	Intel Corporation

Title:	Enhancements for NW-side model training data collection

Document for:	Discussion and Decision

# Introduction

During RAN2 #126 meeting, following agreements were made on NW-side model training data collection:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

In this contribution, we continue discuss the procedure details and potential enhancements to support NW-sided model training data collection.

# Discussion

## gNB-centric training data collection

### Measurement Duration and Memory Comsumption

During RAN2 #126 meeting, supporting data logging at UE side is agreed as an optional feature

For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.

First of all, to train a NW-side model with good performance and better generalization for different scenarios, the network needs massive amount of data for model training and may configure the UE to perform beam measurement towards multiple beams with multiple instances. With the number of beams and instances needed for model training increasing significantly, the UE may not want to continuously perform beam measurement for NW-side model training data collection.

Observation 1: UE may not want to continuously perform beam management for multiple beam instances for NW-side training data collection.

In our understanding, it is desirable that the measurements for Set A of beams is not spread over a very long period as the purpose is to train model(s) on relative quality of beams. The network may need to control the time duration of when UE performs beam measurement. There are two options:

Option 1: Network monitors a timer by implementation, and releases training data collection configuration upon completion

Though there’s no specification impact of this option, the complexity at network increases, as network needs to maintain different timers across different UEs.

Option 2: UE performs training data collection according to network-configured timer/time window.

In this option, each UE can maintain its own timer based on network configuration. The measured training data collected within this time period can be logged at the UE side until UE is configured by the network to report it periodically or based on other report triggers.

This could also be applicable to OAM-centric training data collection, as RRC signaling is shared between gNB-centric and OAM-centric training data collection.

Proposal 1: Network controls the time duration of how UE performs training data collection in below two options:

Option 1: Network monitors a timer by implementation and releases training data collection configuration upon completion,

Option 2: Network configures a time window/timer of when UE performs training data collection via RRC signaling.

Memory Reduction

Considering the training data collected for beam management are normally L1-RSRP of beam measurement, similar as logged data for logged MDT framework, this data should also be stored at AS layer. Furthermore, as baseline, the UE should at least support a minimum AS layer memory size to make sure UE can store enough amount of logged data until it is reported to network.

Proposal 2: UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.

On the other hand, it is also impossible for UE to log training data with an unlimited buffer size. Based on UE’s implementation, UE might be able to store logging data within a limited buffer (which is designed/dynamically allocated by UE based on implementation). When the AS layer memory for logging data storage reaches UE’s limitation, UE can either stop measurements and reports the logged measurement reports to network based on network configuration or continuously perform training data collection measurement, releases the most oldest data in the buffer, then reports to network based on network configuration.

Proposal 3: When UE reaches its buffer limitation, following two options can be considered up to UE’s implementation:

1) stop measurements and reports the logged measurement reports to network based on network configuration

2) continuously perform training data collection measurement, releases the most oldest data in the buffer, then reports to network based on network configuration

Furthermore, network can configure a buffer threshold as measurement reporting trigger.

If the memory size of current logged training data at UE side becomes higher than a threshold, UE can initiate the measurement reporting procedure and reports the logged training data (and together with other measurement data) to the network, according to a new measurement report trigger event.

Proposal 4: Buffer-based event is introduced as a new measurement reporting trigger for NW-side model training data reporting. UE reports the logged data upon buffer-based event is triggered.

### Traininig data collection in RRC_IDLE/INACTIVE

Recalling that NW-side model training data collection for beam management should focus on RRC_CONNECTED state for both data generation and reporting, as captured in TR 38.843 [1].

At least for the use cases studied in this study item, it is assumed that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed. Note that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.

It is also possible that UE goes to RRC_IDLE or RRC_INACTIVE state while collecting training data for NW-side model. In this section, we will discuss the UE behavior on NW-side model training data collection when UE goes to RRC_IDLE or RRC_INACTIVE state.

Configuration

When UE goes to RRC_INACTIVE state, similar as other network configuration, UE can temporarily store NW-side model training data collection configuration in AS layer. According to the above context in TR 38.843 [1], i.e. data collection focuses on RRC_CONNECTED state, UE should further stop training data measurement and logging.

When UE goes to RRC_IDLE state, similar as other functionalities and configurations, it is straightforward to release NW-side model training data collection configuration. The UE further stops training data measurement and logging, similar as UE behavior in RRC_INACTIVE state.

Proposal 5: When UE goes to RRC_INACTIVE and RRC_IDLE state, UE stops measurement and logging for NW-side model training data collection.

Proposal 5-1: UE temporarily stores NW-side model training data collection configuration in AS layer when UE goes to RRC_INACTIVE state.

Proposal 5-2: UE releases NW-side model training data collection configuration when UE goes to RRC_IDLE state.

Reports

Moreover, it is possible that there are still available logged data at UE side that were not be able to report to network before UE goes to RRC_INACTIVE state. In this circumstance, network can configure UE to report all available logged data to current serving cell in RRC signaling, indicating the remaining logged data should be reported before UE goes to RRC_INACTIVE state.

Proposal 6: Before UE goes to RRC_INATIVE and RRC_IDLE state, network configures UE to report remaining logged data to current serving cell via RRC signaling. .

Below is a summarized table of UE behaviours when UE goes to RRC_INACTIVE/IDLE:

|  | RRC_INACTIVE | RRC_IDLE |
| --- | --- | --- |
| Configuration | Temporarily stored at AS layer | Release |
| Measurement/Logging | Stop |  |
| Logged data (if any) | Report to network based on network configuration (e.g. RRCRelease) |  |

### Continuity of training data collection

On one hand, for NW-sided model, training data collection configurations are specifically designed by one cell considering its NW-side additional conditions, which may not be applicable to another cell, and model transfer/sharing between NG-RAN nodes are not studied/supported by RAN3. The usefulness of collected/logged data for NW-sided model is questionable when UE is handed-over from source cell to target cell. Moreover, if the target cell does not have the capability to support certain AI/ML functionality, the logged data of that functionality also doesn’t need to be reported to this target cell.

On the other hand, it is still possible that the two cells may share the same type of NW-side additional condition and inference configuration, if the two cells are co-located. This makes the remaining logged data useful for the target cell.

Therefore, during handover request, source cell can check the applicability of the functionality at the target cell, then decides whether the remaining logged data needs to be released or stored (then report to target cell). The network can configure UE whether to store those remaining logged data before UE’s handover or to release in RRCReconfigruationWithSync.

If the remaining logged data is stored, UE can include an indication to target cell, indicating there’s remaining logged data, then further sends to target cell according to network request/configuration. Otherwise, UE releases the remaining logged data before handover.

Proposal 7: In gNB-centric training data collection, network configures whether the UE releases or temporarily stores the remaining logged data (if any) for NW-side model training data collection during handover. UE may report the stored logged data to target cell after handover according to target cell’s request/configuration.

## OAM-centric training data collection

It was agreed that immediate MDT framework is considered as baseline for OAM-centric training data collection. Therefore, both signalling-based MDT and management-based MDT can be reused for OAM-centric training data collection. Additionally, same as immediate MDT framework, the management-based MDT configuration should not overwrite signalling based MDT configuration.

Furthermore, as defined in TS 32.422 [2], that gNB may also temporarily store the received MDT measurements, i.e. whenever the gNB receives the MDT measurements it shall save it to a “Trace Record”. The time and the criteria when the “Trace Records” are sent to the TCE is vendor specific. This principle shall also be applicable for training data collection.

Proposal 8: Same as immediate MDT framework, following principles should be followed by OAM-centric training data collection:

1. Both signalling-based MDT and management-based MDT are supported for OAM-centric training data collection.

2. Management-based MDT configuration for training data collection should not overwrite signalling based MDT configuration for training data collection.

3. Whenever the gNB receives the MDT measurements it shall save it to a Trace Record. The time and the criteria when the Trace Records are sent to the TCE is vendor specific

Recalling that the main motivation of using immediate MDT as baseline is because the RRC signaling over air interface is shared between RRM measurement and immediate MDT framework.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

Therefore, the above discussion and proposals in Section 2.1.1, Section 2.1.2 and Section 2.1.3 are also applicable to OAM-centric data collection.

Proposal 9: Enhancements for gNB-centric training data collection (i.e. Proposal 1-7) are also applicable to OAM-centric data collection.

# Conclusion

In this contribution, we discussed potential enhancements for gNB-centric and OAM-centric training data collection:

We observed and proposed the followings:

Observation 1: UE may not want to continuously perform beam management for multiple beam instances for NW-side training data collection.

Proposal 1: Network controls the time duration of how UE performs training data collection in below two options:

Option 1: Network monitors a timer by implementation and releases training data collection configuration upon completion,

Option 2: Network configures a time window/timer of when UE performs training data collection via RRC signaling.

Proposal 2: UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.

Proposal 3: When UE reaches its buffer limitation, following two options can be considered up to UE’s implementation:

1) stop measurements and reports the logged measurement reports to network based on network configuration

2) continuously perform training data collection measurement, releases the most oldest data in the buffer, then reports to network based on network configuration

Proposal 4: Buffer-based event is introduced as a new measurement reporting trigger for NW-side model training data reporting. UE reports the logged data upon buffer-based event is triggered.

Proposal 5: When UE goes to RRC_INACTIVE and RRC_IDLE state, UE stops measurement and logging for NW-side model training data collection.

Proposal 5-1: UE temporarily stores NW-side model training data collection configuration in AS layer when UE goes to RRC_INACTIVE state.

Proposal 5-2: UE releases NW-side model training data collection configuration when UE goes to RRC_IDLE state.

Proposal 6: Before UE goes to RRC_INATIVE and RRC_IDLE state, network configures UE to report remaining logged data to current serving cell via RRC signaling. .

Proposal 7: In gNB-centric training data collection, network configures whether the UE releases or temporarily stores the remaining logged data (if any) for NW-side model training data collection during handover. UE may report the stored logged data to target cell after handover according to target cell’s request/configuration.

Proposal 8: Same as immediate MDT framework, following principles should be followed by OAM-centric training data collection:

1. Both signalling-based MDT and management-based MDT are supported for OAM-centric training data collection.

2. Management-based MDT configuration for training data collection should not overwrite signalling based MDT configuration for training data collection.

3. Whenever the gNB receives the MDT measurements it shall save it to a Trace Record. The time and the criteria when the Trace Records are sent to the TCE is vendor specific

Proposal 9: Enhancements for gNB-centric training data collection (i.e. Proposal 1-7) are also applicable to OAM-centric data collection.

# Reference

[1] TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2] TS32.422, Trace control and configuration management