3GPP TSG RAN WG2 Meeting #127             	                	                          R2-2406384

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.4

Source:	Intel Corporation

Title:	Discussion on UE-sided model training data collection

Document for:	Discussion and Decision

# Introduction

After RAN2 #126 meeting discussion, following table of potential solutions for training data collection for UE-sided model is captured in TR 38.843 [1]:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Network control level | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |

The highlighted contents still need further discussion to conclude the details of each solution. Furthermore, whether solutions from the above table can be applicable to positioning use case was not discussed before.

In this contribution, we discuss the remaining FFS in the above table of training data collection for UE-sided model solutions, then discuss how to support dataset transfer for positioning UE-sided model training (i.e. positioning accuracy enhancement Case 1).

# Discussion

## Beam Management UE-sided model

Candidate solutions for Option 2/3 with Full Visibility

During RAN2 #126 meeting discussion, the understanding of “partial visibility” is not clear and there’s no consensus on whether Option 2/3 support partial visibility of data content and data format.

Except standardizing specific IEs over NAS/RRC signalling (i.e. same as RRM measurement reporting, etc), the collected training data of UE-sided model can also be transferred as a container over the air interface, by standardizing the structure (e.g. data content and format) with full visibility, similar as QoE container.

Following QoE and MDT data collection, full visibility is normally considered in 3GPP by standardizing data content and format being transferred if CP tunnel is used. Defining training data collection for UE-sided model could be similar to QoE container defined by SA4 and logged MDT container defined by SA5. During data transmission, this container is transparent to RAN, but can be interpreted by the first termination entity (i.e. CN in Option 2 and OAM in Option 3).

Observation 1: Network can also use container for training data collection of UE-sided model and achieve full visibility by standardizing data content and format of container via CP tunnel, e.g. same as QoE and MDT data collection, i.e. container with a standardized structure,

To support full visibility of standardized data and full controllability, as discussed in the above section, both Option 2 and 3 can consider transmitting training data for UE-side model via either standardized IEs or standardized container over NAS/RRC signaling.

As replied by RAN1 in R1-2310681 [2], beam management NW-side model and UE-side model share the same training data:

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |

Recalling that, RAN2 agreed to support OAM-centric data collection for NW-side model by using immediate MDT framework as baseline:

Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

Therefore, Option 3 for UE-side model, which is also terminated at OAM, can simply reuse OAM-centric data collection for NW-side model, i.e. immediate MDT framework as baseline. All enhancements of immediate MDT framework for NW-side model training data collection can also be applicable to Option 3 for UE-side model training data collection.

Observation 2: OAM-centric data collection for NW-side model (i.e. enhanced immediate MDT) can be reused for UE-side model training data collection Option 3 with standardized data.

In summary, the following candidate solutions can be considered for Option 2 and 3:

Proposal 1: If full visibility and full controllability is required in Option 2 and 3, RAN2 to endorse following candidate solutions in TR 38.843:

|  | Option 2 | Option 3 |
| --- | --- | --- |
| Candidate 1 | Standardized data with specified IEs via NAS signaling | OAM-centric data collection for NW-side model (i.e. enhanced immediate MDT) |
| Candidate 2 | Container with standardized structure via NAS signaling | Container with standardized structure via RRC signaling (e.g. similar as QoE and MDT) |

Control of Visibility

It is noticed that supporting partial visibility and no visibility are FFS in Option 2 and Option 3. This has been discussed over many RAN2 meetings without consensus. As discussed above, container can be considered as one of the options to carry training data collected from UE. The container can also be designed as a non-standardized container to protect proprietary data of UE device, i.e. transparent to 3GPP. Then, the visibility can be achieved either way, i.e. standardised container with full visibility or non-standardised container with no visibility.

As captured in TR 38.843, the concern of data transfer risks and regulation restrictions are mostly from operators. To provide more controllability to operators on UE-side training data collection, a CHOICE structure between standardised container and a non-standardised container can be introduced in RRC/NAS signaling. This provides the flexibility to deploy either a standardised or non-standardised container depending on the operator preference/regional requirement etc. If the UE is using a non-standardised container where it is not allowed to, this will be visible to the network/operator from the branch in the CHOICE structure, allowing the network to potentially even discard the data. With this, operators can still have the controllability over how the container will be structured between standardised and non-standardised containers while providing the deployment flexibility.

Proposal 2: Operator can control visibility of Option 2 and Option 3 by supporting both standardised and non-standardised containers using a CHOICE structure in NAS/RRC signalling for UE-side training data collection.

## Positioning Case 1

The next question is whether the above solutions are also applicable to positioning accuracy enhancement Case 1.

A working assumption was made in RAN1 #116bis meeting:

Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

Based on the above working assumptions, it is observed that data generation entity for positioning Case 1 is the same as legacy positioning method. In our understanding, for both standardized data and proprietary data, the existing LPP signaling can be reused to collect training data from PRU and/or non-PRU UE to LMF, then further propagate to server for positioning Case 1 training data collection if needed.

Proposal 3: Option 1a/1b/2/3 are not applicable to positioning accuracy enhancement Case 1. To support training data collection for positioning case 1, PRU and/or Non-PRU UE transfers training data to LMF via LPP signaling, then if needed, LMF transfers the collected training data to server of training data collection for UE-side model.

Furthermore, the above analysis for Option 2/3 on candidate solutions and choice structure for visibility controllability can also be applicable to positioning Case 1 training data collection.

Proposal 4: Proposal 1 and Proposal 2 are also applicable to LPP signaling enhancement for UE-side training data collection.

# Conclusion

In this contribution, we discussed the remaining open issues for UE-side training data collection, with following observations and proposals:

Observation 1: Network can also use container for training data collection of UE-sided model and achieve full visibility by standardizing data content and format of container via CP tunnel, e.g. same as QoE and MDT data collection, i.e. container with a standardized structure,

Observation 2: OAM-centric data collection for NW-side model (i.e. enhanced immediate MDT) can be reused for UE-side model training data collection Option 3 with standardized data.

Proposal 1: If full visibility and full controllability is required in Option 2 and 3, RAN2 to endorse following candidate solutions in TR 38.843:

|  | Option 2 | Option 3 |
| --- | --- | --- |
| Candidate 1 | Standardized data with specified IEs via NAS signaling | OAM-centric data collection for NW-side model (i.e. enhanced immediate MDT) |
| Candidate 2 | Container with standardized structure via NAS signaling | Container with standardized structure via RRC signaling (e.g. similar as QoE and MDT) |

Proposal 2: Operator can control visibility of Option 2 and Option 3 by supporting both standardised and non-standardised containers using a CHOICE structure in NAS/RRC signalling for UE-side training data collection.

Proposal 3: Option 1a/1b/2/3 are not applicable to positioning accuracy enhancement Case 1. To support training data collection for positioning case 1, PRU and/or Non-PRU UE transfers training data to LMF via LPP signaling, then if needed, LMF transfers the collected training data to server of training data collection for UE-side model.

Proposal 4: Proposal 1 and Proposal 2 are also applicable to LPP signaling enhancement for UE-side training data collection.

# Reference

[1] TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2] R1-2310681, Reply LS on Data Collection Requirements and Assumptions (Part B), RAN1