3GPP TSG RAN WG2 Meeting #127                                                        R2-2406385

Maastricht, Netherlands, Aug 19th - 23th, 2024

Agenda item:		8.3.2.2

Source:	China Telecom

Title:	Discussion on AI/ML based RRM measurement prediction

Document for:		Discussion

Introduction

A study item of Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved in RAN#102 meeting [1] and revised in RAN#103 meeting [2]. In this contribution, we discuss the AI/ML based RRM measurement prediction for mobility in NR and present our views on cluster approach, sub-use cases and metrics aspects.

Discussion

In RAN2#126 meeting, following agreements about RRM prediction are made.

Agreements - To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain. - To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window. - We will start the evaluation with measurements prediction accuracy and model complexity. We can discuss system performance after we see which scenarios have good measurements prediction accuracy. - Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. - average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged. - For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result. - Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured

In the following section, we discuss the RRM measurement prediction cluster approach, sub-use cases and metrics respectively.

Cluster Approach and sub-use cases

Regarding the discussion on simulation assumption and evaluation methodology in the former meeting, RAN2 has agreed to define cluster approach.

Agreements To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain.

In RAN2#126 meeting, it was proposed that the definition of the cluster approach could consider the temporal domain, frequency domain, and spatial domain relationships between input and output cells. Cluster cells can be cells with the same frequency or similar geographical location. Constructing datasets through the clustering of cells for model training can enhance the performance of predictive models, such as improving prediction accuracy. For intra-frequency cases, measuring cells in the same frequency layer does not increase the UE's additional measurement burden, as the UE can measure SSBs from both the serving cell and neighbouring cells simultaneously. For inter-frequency cases, measuring cells across different frequency layers can reduce the measurement burden, or enhance handover performance without reducing the measurement burden.

To compare the generalization performance of cluster-based model, we define site-based model as a baseline which is trained on data from a single cell. The cluster-based model, on the other hand, is trained on data from multiple cells, allowing it to capture a wider range of scenarios and network conditions. This comparison helps us evaluate the cluster-based model's ability to adapt to different environments and its overall robustness.

Table 1. Scenario priority

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Consider the quantitative relationship between input and output cells, and the scenario priority mentioned in RAN2#126 meeting, we can outline four possible options, focusing on the high-priority cases 2 and 3, as shown in Table 1.

- Option 1: Cluster-based AI/ML model training with 1 to 1.

Based on the training of cluster AI/ML models, where the input data is the RSRP from a single cell and the output data is the RSRP for a potentially different single cell. Following we only consider high-priority sub-use cases, e.g., Case 2 and Case 3.

For temporal domain sub-use case, the input data can be the past RSRP of cell A, and the output data is the predicted RSRP of cell A.

For frequency domain sub-use case, cell A and cell B can be co-located or neighbouring frequency cells. Consider co-located cells that are inter-frequency, the input data is the RSRP from cell A and the output data is the RSRP from cell B. Co-located cells are defined as cells that share the same physical site as shown in Figures 1(a). For neighbouring cells, the input data is the RSRP from cell A, and the output data is the RSRP from inter-frequency cell B as shown in Figures 1(b).

Figure 1(a). The definition of co-located cells

Figure 1(b). The definition of neighbouring cells

- Option 2: Cluster-based AI/ML model training with N to 1.

Based on the training of cluster AI/ML models, where the input data is the RSRP from N(N>1) cells and the output data is the RSRP for a single cell. Considering high-priority sub-use cases.

For temporal domain sub-use case, the input data include the past RSRP of cell A and surrounding cells, which together form cluster cell A', the output data is the predicted RSRP of cell A.

For frequency domain sub-use case, the scenarios include co-located cells and neighbouring cells. Consider co-located cells that are inter-frequency, the input data include the RSRP from cell A and surrounding cells, which together form cluster cell A', the output data is the RSRP of cell B. It can enhance measurement prediction accuracy by reducing ping-pong effects through learning handover characteristics and optimizing decisions across multiple cells. Cell A and cell B are co-located, and the surrounding cells are also co-located with cell A. Cell B is included within the surrounding cells, and the input may not necessarily include cell B. Additionally, the surrounding cells and cell A may not necessarily be co-located. Consider neighbouring cells that are inter-frequency, the input data include the RSRP from cell A and surrounding cells, which together form cluster cell A', the output data is the RSRP of cell B. Cell B is not necessarily included within the surrounding cells, which can reduce the measurement overhead.

- Option 3: Cluster-based AI/ML Model training with N to K.

Based on the training of cluster AI/ML models, where the input data is the RSRP from N(N>1) cells and the output data is the RSRP for a single cell. Considering high-priority sub-use cases.

For temporal domain sub-use case, the input data include the past RSRP of cell A and surrounding cells, which together form cluster cell A', the output data is the predicted RSRP of cell A and surrounding cells, which together form cluster cell a'.

For frequency domain sub-use case, the scenarios include co-located cells and neighbouring cells. Consider co-located cells that are inter-frequency, the input data include the RSRP from cell A and surrounding cells, which together form cluster cell A', the output data include the RSRP from cell B and surrounding cells, which together form cluster cell B'. Consider neighbouring cells that are inter-frequency, the input data include the RSRP from cell A and surrounding cells, which together form cluster cell A', the output data include the RSRP from cell B and surrounding cells, which together form cluster cell B'. The model is more complex than Option 2, with supporting multiple N-to-1 predictions in the frequency domain.

- Option 4: Site-based AI/ML model training with 1-to-1.

Based on the training of site AI/ML models, where the input data is the RSRP from a single cell and the output data is the RSRP for a potentially different cell. Considering high-priority sub-use cases. For temporal domain sub-use case, the input data can be the past RSRP of cell A, and the output data is the predicted RSRP of cell A.

For frequency domain sub-use case, cell A and cell B can be co-located or neighbouring frequency cells. Consider co-located cells that are inter-frequency, the input data is the RSRP from cell A and the output data is the RSRP from cell B.

Consider neighbouring cells that are inter-frequency, the input data is the RSRP from cell A, and the output data is the RSRP from cell B. Option 4 serves as a baseline and can be compared with the performance of the cluster AI/ML model.

Proposal 1: At least, following cases can be studied at first for high priority sub-use cases (Case 2 and Case 3).

- Option1: cluster-based AI/ML model training with 1-to-1.

- Option2: cluster-based AI/ML model training with N-to-1.

- Option3: cluster-based AI/ML model training with N-to-K.

- Option4: site-based AI/ML model training with 1-to-1.

Table 2. Site/Cluster-based cell Options

| Options | AI/ML Model | Number of input/output cells | Domain | comments |
| --- | --- | --- | --- | --- |
| Option 1 | Cluster-Based | 1-to-1 | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 2 | Cluster-Based | N-to-1 | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 3 | Cluster-Based | N-to-1 (N>1) | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 4 | Site-Based | N-to-K (N>1,K>1) | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |

Proposal 2: At least, for cluster cells, the co-location scenario can be considered.

Metrics

Based on the discussion about cluster approach and sub-use cases, proper metrics should be considered to reasonable evaluate the performance of AI/ML based mobility enhancement.

- Intermediate KPIs

Agreements Average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged. To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window.

In RAN2#126 meeting, there is an agreement that average RSRP difference is taken as prediction accuracy metric and aligning the prediction window should be FFS.

RSRP diff= |Actual RSRP – Predicted RSRP|

Average RSRP:

The RSRP difference is related to the length of the prediction window T, so different prediction window lengths should be evaluated. It is recommended that the model output can be either L1 RSRP or L3 RSRP. The intermediate KPI calculations can use the model output or L3 RSRP as the baseline.

Proposal 3: The intermediate KPI calculations can use the model output or L3 RSRP as the baseline.

Proposal 4: Different prediction window lengths should be evaluated and the baseline value can be chosen for aligning the prediction window.

- Measurement reduction KPIs

In RAN2#126 meeting, there is an agreement that the measurement reduction metrics for intra-frequency scenario are MRRT and MRRS.

Agreements Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured / total beams to be measured

For measurement reduction, obviously measurement reduction KPIs should be considered as baseline. Different metrics should be defined for the sub-use cases for spatial domain RRM prediction and time domain RRM prediction. In this case, the main study goal is measurement reduction, system-level performance can also be provided by companies.

Proposal 5: It is recommended that for use cases providing measurement reduction KPI, system-level performance can be optional provided by the companies.

- System KPIs

Agreements We will start the evaluation with measurements prediction accuracy and model complexity. We can discuss system performance after we see which scenarios have good measurements prediction accuracy.

In RAN2#126 meeting, there is an agreement that We will start the evaluation with measurements prediction accuracy and model complexity.

For HO performance enhancement case, system-level metrics (e.g., total number of HOs, ping-pong HO rate, HO interruption time, HOF/RLF rate, short-time-of-stay rate) can be considered to demonstrate the technical feature effectiveness.

Proposal 6: It is recommended that ping-pong HO rate, HO interruption time, HOF/RLF rate, short-time-of-stay rate can be considered as the system performance evaluation metrics.

Conclusions

In this contribution, we discuss the AI/ML based RRM measurement prediction for mobility in NR and present our views on sub-use cases and metrics aspects:

Proposal 1: At least, following cases can be studied at first for high priority sub-use cases (Case 2 and Case 3).

- Option1: cluster-based AI/ML model training with 1-to-1.

- Option2: cluster-based AI/ML model training with N-to-1.

- Option3: cluster-based AI/ML model training with N-to-K.

- Option4: site-based AI/ML model training with 1-to-1.

Table 2. Site/Cluster-based cell Options

| Options | AI/ML Model | Number of input/output cells | Domain | comments |
| --- | --- | --- | --- | --- |
| Option 1 | Cluster-Based | 1-to-1 | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 2 | Cluster-Based | N-to-1 | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 3 | Cluster-Based | N-to-1 (N>1) | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |
| Option 4 | Site-Based | N-to-K (N>1,K>1) | Temporal | Past time/ Predicted time |
|  |  |  | Inter Frequency | Co-located/ Neighbouring cells |

Proposal 2: At least, for cluster cells, the co-location scenario can be considered.

Proposal 3: The intermediate KPI calculations can use the model output or L3 RSRP as the baseline.

Proposal 4: Different prediction window lengths should be evaluated and the baseline value can be chosen for aligning the prediction window.

Proposal 5: It is recommended that for use cases providing measurement reduction KPI, system-level performance can be optional provided by companies.

Proposal 6: It is recommended that ping-pong HO rate, HO interruption time, HOF/RLF rate, short-time-of-stay rate can be considered as the system performance evaluation metrics.

References

- [1] RP-234055, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR CMCC (moderator, RAN1 VC), RAN#102.

- [2] RP-240082, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, RAN#103.