3GPP TSG-RAN WG2 Meeting #127	R2-2406386

Maastricht, Netherlands, Aug 19th– 23rd, 2024

# Agenda item:	8.6.2

# Source:	ETRI

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#126 meeting, agreements related to inter-CU LTM are as follow [1]:

Agreements on inter-CU LTM: Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. Xn-based and N2-based inter-CU LTM - Xn-based inter-CU LTM is prioritized in Rel-19. Stage-2 signaling flows and procedures - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). Preparation: RRC Configuration/structure - The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. LTM Candidate ID - For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. Max number of LTM candidate IDs - The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. Early sync: RAR based option - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. Execution: LTM Cell Switch Command - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. LTM Cell switch completion - Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. - The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this paper, we further discuss the open issues of NR SA scenario and potential issues of NR-DC case.

- Discussion

  - Inter-CU LTM in SA

Reference configuration

In the last RAN2 meeting, there was a discussion about whether to use a single reference configuration or multiple reference configurations when generating LTM candidate cell configurations. Some companies proposed that it would be simpler and more efficient to use a single reference configuration because the maximum number of candidate cells is 8 and the number of the candidate cells belonging to different CUs is not expected to be high. However, it could be beneficial to use either a single reference configuration or multiple reference configurations depending on the combination of candidate cells. For example, if only 1 or 2 of the 8 candidate cells belong to a different CU, a single reference configuration could be sufficient to reduce signaling overhead efficiently. However, given the low commonality among CUs, the more candidate cells that belong to different CUs, the more beneficial it becomes to use multiple reference configurations. If it could be beneficial to have multiple reference configurations in some cases, there is no need to limit the number of reference configurations to one from a specification perspective, and it would be better to let the serving CU determine the number of reference configurations based on how many candidate cells belong to different CUs.

Observation 1: Depending on the case, it could be beneficial to use either a single reference configuration or multiple reference configurations.

In Rel-18, a single reference configuration plus delta configurations is used due to the high commonality of candidate cells under the same CU. Therefore, if multiple reference configurations are used for candidate cell configurations for inter-CU LTM, to define a reference configuration per CU can be one possible solution. The structure of LTM-Config information elements in Rel-18 is designed to configure all candidate cell configurations within a CU based on a single reference configuration.  If a reference configuration is defined per CU, the current LTM-Config can be reused without significant change and the benefit in using a reference configuration can be kept with relatively low complexity as shown in Figure 1. In this example, LTM-Config incudes a ltm-ReferenceConfiguration as a master reference configuration which applied for all candidate cells and LTM-GroupCandidate IE includes ltm-ReferenceConfiguration for candidate cells under the same CU. If a LTM-GroupCandidate does not include a ltm-ReferenceConfiguration, then the ltm-ReferenceConfiguration included in the LTM-Config is used for candidate cells in the LTM-GroupCandidate. Note that the structure of the LTM-GroupCandidate is same as the current Rel-18 LTM-Config.

Figure 1. an example of LTM-Config Structure

Observation 2: If a reference configuration is defined per CU, the current LTM-Config can be reused and the benefit in using a reference configuration can be kept.

Proposal 1: Multiple reference configurations can be provided per CU for inter-CU LTM. The source gNB-CU determines whether to use a single reference configuration for all candidate cells or multiple reference configurations per CU during the LTM preparation phase.

  - Inter-CU LTM in NR-DC

For inter-CU LTM in NR-DC, 2 scenarios have been specified as secondary priority in WID [2]:

- CU is acting as SN and MCG is unchanged

- CU is acting as MN and SCG is unchanged or released

Inter-CU LTM when a CU is acting as SN and MCG is unchanged

This scenario is an extended version of the subsequent CPAC defined in [3], where the UE context at the source SN is moved to the target SN while the UE context at the MN is retained. The difference between the subsequent CPAC and inter-CU SCG LTM is that the UE and the candidate SNs keep the candidate SCG configurations after the cell switch, and the cell switch is commanded by the source cell. Therefore, we can use the subsequent CPAC procedure as a baseline for inter-CU SCG LTM with the MCG unchanged.

Proposal 2: RAN2 takes Subsequent CPAC procedure as a baseline for inter-CU SCG LTM with MCG unchanged.

According to the subsequent CPAC procedure in [3], the inter-SN subsequent CPAC is initiated by the MN or the source SN which receives the L3 measurement report. Similarly, the source SN as well as the MN should be able to initiate inter-CU SCG LTM and recommend candidate cells for SCG configuration based on the latest measurement results.

Proposal 3: Inter-CU SCG LTM can be initiated by the MN or the source SN.

Figure 2 and Figure 3 illustrates the message flow for inter-CU SCG LTM initiated by the MN and the source SN, respectively. Only difference between them is the source SN transmit the MN SN CHANGE REQUIRED message to initiate inter-CU SCG LTM. In both scenarios, we assumed that L1 measurement report is transmitted to the MN and cell switch command MAC CE is also transmitted by the MN regardless of which node initiated inter-CU SCG LTM. However, this seems to require further discussion. Since the node receiving the L3 measurement results initiated inter-CU SCG LTM, it seems consistent for the node that initiated the procedure to receive the L1 measurement results and send the cell switch command MAC CE based on the measurement results. However, regardless of which node initiates the procedure, since the MN is in charge of SN change, it is reasonable for the MN to receive the L1 measurement results and determine the target. The L1 measurement results for determining the target should be based on the measurements from the node that initiated the inter-CU SCG LTM procedure.

Proposal 4: If the MS and the source SN can initiate inter-CU SCG LTM, RAN2 to discuss which node to receive L1 measurement and to transmit cell switch command MAC CE for LTM cell switch.

Figure 2. Procedure of Inter-CU SCG LTM initiated by MN

Figure 3. Procedure of Inter-CU SCG LTM initiated by SN

Inter-CU LTM  when a CU is acting as MN and SCG is unchanged or released

Inter-CU MCG LTM with SCG unchanged or released is similar to inter-Master Node handover with or without SN change in [3], in which the UE’s MCG changes while the SCG configuration is either kept or released depending on the target MN’s decision. If the target MN decides to keep the UE context in the source SN, the target MN performs an SN addition procedure with the target SN. If the target MN decides not to keep the UE context in the source SN, the target MN includes only the MCG configuration in the HANDOVER REQUEST ACKNOWLEDGE message so that the source MN performs the SN release procedure with the source SN before the LTM cell switch procedure, as illustrated in Figure 4 and Figure 5.

Proposal 5: RAN2 takes Inter-MN handover with/without SN change as a baseline for inter-CU MCG LTM with SCG unchanged or released.

Proposal 6: The candidate MN decides whether to keep or release the source SCG during inter-CU MCG LTM with SCG unchanged or released.

Proposal 7: If the candidate MN decides to keep the source SCG, it performs SN addition procedure with candidate SN and provides SCG configuration as part of LTM candidate configuration.

Proposal 8: If the candidate MN decides not to keep the source SCG, it includes only MCG configuration in the LTM candidate configuration so that the source MN performs SN release procedure with the source SN.

Figure 4. Procedure of inter-CU MCG LTM with SCG

Figure 5. Procedure of inter-CU MCG LTM with SCG release

- Conclusions

In this contribution, we discuss inter-CU LTM related issue. Following are the observations and proposals  made in this contribution:

Observation 1: Depending on the case, it could be beneficial to use either a single reference configuration or multiple reference configurations.

Observation 2: If a reference configuration is defined per CU, the current LTM-Config can be reused and the benefit in using a reference configuration can be kept.

Proposal 1: Multiple reference configurations can be provided per CU for inter-CU LTM. The source gNB-CU determines whether to use a single reference configuration for all candidate cells or multiple reference configurations per CU during the LTM preparation phase

Proposal 2: RAN2 takes Subsequent CPAC procedure as a baseline for inter-CU SCG LTM with MCG unchanged.

Proposal 3: Inter-CU SCG LTM can be initiated by the MN or the source SN.

Proposal 4: If the MS and the source SN can initiate inter-CU SCG LTM, RAN2 to discuss which node to receive L1 measurement and to transmit cell switch command MAC CE for LTM cell switch.

Proposal 5: RAN2 takes Inter-MN handover with/without SN change as a baseline for inter-CU MCG LTM with SCG unchanged or released.

Proposal 6: The candidate MN decides whether to keep or release the source SCG during inter-CU MCG LTM with SCG unchanged or released.

Proposal 7: If the candidate MN decides to keep the source SCG, it performs SN addition procedure with candidate SN and provides SCG configuration as part of LTM candidate configuration.

Proposal 8: If the candidate MN decides not to keep the source SCG, it includes only MCG configuration in the LTM candidate configuration so that the source MN performs SN release procedure with the source SN.

- References

- Chair notes RAN2#126;

- RP-240299 Revised Work Item: NR mobility enhancements Phase 4; Apple, China Telecom;

- 3GPP TS 37.340 V18.0.0; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2 (Release 18)