3GPP TSG-RAN WG2 Meeting #127		R2-2406387

Maastricht, Netherlands, 19th – 23rd Aug 2024

Source:	vivo

Title:          Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

The WID [1] was discussed at the latest two RAN2 meetings. There were the following agreements achieved:

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

In this contribution, we will focus on the functionality-based LCM for UE-sided model for Beam Management use case and share our opinions in RAN2.

# Discussion

## 2.1 Whether to introduce the term of available functionality?

In the phase 1 of email discussion [2], whether to define available functionality separately from applicable functionality was discussed. However, there was no consensus on this issue:

Summary from Q7 discussion: - Companies have quite diverged view on defining available functionalities separately from applicable functionalities. - Separate definition: - available functionalities are the super set of applicable functionalities e.g. due to NW sided additional conditions. - As a starting point, we could have two different definitions to discuss further details between UE and NW. - Not to define available functionalities - Applicability already imply that it has at least one available model. - Applicability is determined by UE i.e. applicability should not be defined from NW point of view only. - Need further discussion about specific cases and relationship between applicability and availability especially the case where there is no applicable functionality despite having available models. Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

From the view of the standard, before functionality activation/configuration, the network needs to know whether a functionality is applicable. If a functionality is applicable, the functionality should have at least one available model with both UE-sided additional conditions and NW-sided additional conditions being fulfilled. One issue is that whether the network needs to know the reason of functionality inapplicability, that is due to no available model or NW-sided additional conditions (being not fulfilled). Several companies think that if it is due to NW-sided additional conditions and there is at least one available model, the network can provide a suitable (inference) configuration to let the functionality become applicable. However, besides the indicator of the model being available, this requires that the network also knows the requirements on the network associated with the functionality, in order to provide the suitable (inference) configuration according to the NW-sided additional conditions. Whether it is feasible that the NW-sided additional conditions can be fulfilled via the changed (inference) configuration by the network may need further consideration, since currently the exact content of NW-sided additional conditions is still unclear. The introduction of available functionality increases the specification complexity and it seems to be unnecessary so far unless it is justified, e.g., after RAN1 defines the detailed content of NW-sided additional conditions.

- Do not introduce the term of available functionality unless it is justified, e.g., after RAN1 defines the detailed content of NW-sided additional conditions.

## 2.2 Relationship between configured functionalities and applicable functionalities

In the phase 1 of email discussion [2], the relationship between configured functionalities and applicable functionalities was discussed, however, there was no consensus on this issue:

Summary from Q5 discussion: - Companies have diverged view on relationship between configured functionalities and applicable functionalities. - View 1: configured functionalities may or may not be applicable. - View 2: configured functionalities are always applicable because NW should configure functionalities within applicable functionalities. - View 3: no definition of configured functionalities. Therefore, no need to discuss. Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

In the previous RAN2 discussions, some companies thought that a functionality can be configured before the functionality is confirmed to be applicable, which is like NeedForGap scheme. For the NeedForGap scheme, when the UE is configured to perform the measurement which requires the measurement gap, the UE sends the NeedForGap indicator to the base station to request for the measurement gap. Then, UE use the gap configuration to perform the measurement after receiving the measurement gap, as the gap can always be used when received by UE.

We think the case of configured functionality is different from NeedForGap case, because there may be numerous reasons why configured functionality is not applicable (when network configures UE), such as, UE is not in area where configured functionality is applicable, or other reasons. That means, after receiving the configured functionality, as far as all conditions for applicability are not fulfilled, UE cannot apply it. UE may also not know when the configured functionality can be applicable (e.g., 1mn, 1h, … is unknow to UE). From UE and network perspective, configuring functionality that UE may not be applicable, is just unnecessary signaling overhead.

Therefore, we propose that configured functionalities should be the applicable ones. Note that there may be a filtering scheme for the signaling overhead reduction of applicable functionality reporting, e.g., the network indicates the interested functionalities to the UE. The interested functionalities should not be regarded as configured functionalities.

- Configured functionalities are always applicable because NW should configure functionalities within applicable functionalities.

## 2.3 Clarification on the NW-side/UE-side additional conditions

When determining whether the (UE-sided or NW-sided additional) conditions are fulfilled, actually two concepts are related: one is the conditions/requirements of the functionality that need to be met by the current status/configuration of UE or network, e.g., the requirement of UE speed, which is associated to the functionality itself; the other is current status/configuration of UE or network, e.g., current UE speed, which is associated to the UE or network. For example, if the former requires the UE speed is low and the current UE speed is low, the UE-sided additional condition can be determined to be fulfilled; otherwise, the UE-sided additional condition is not fulfilled and the functionality is not applicable. In the previous RAN2 discussions, there is no clear distinction between the two concepts.

The additional conditions associated with functionality include UE-sided additional conditions and NW-sided additional conditions. A functionality is applicable only when both UE-sided additional conditions and NW-sided additional conditions are fulfilled. Therefore, there are the following 4 types of information related to NW-sided/UE-sided addition conditions:

- Info 1: UE-sided additional conditions/requirements (associated with the functionality), e.g., requirement of UE speed.

- Info 2: Current UE-sided additional status (associated with UE itself), e.g., current UE speed.

- Info 3: NW-sided additional conditions/requirements (associated with the functionality), e.g., required beam pattern configuration or required associated ID

- Info 4: Current NW-sided additional status/configuration (associated with NW itself), e.g., current beam pattern configuration, or current associated ID.

Observation 2:	There are the following 4 types of information related to NW-sided/UE-sided addition conditions:

- Info 1: UE-sided additional conditions/requirements (associated with the functionality).

- Info 2: Current UE-sided additional status (associated with UE itself).

- Info 3: NW-sided additional conditions/requirements (associated with the functionality

- Info 4: Current NW-sided additional status/configuration (associated with NW itself).

For the UE-sided model, RAN2 can assume that the UE has Info 1, Info 2, Info 3 and the network has Info 4 (if not considering model transfer/delivery). In the phase 2 discussion of email discussion [3], which entity determines whether a functionality is applicable or not (i.e., Q1-2) was discussed. The Rapporteur had the following proposals:

Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) It is optional for network to provide NW-side additional condition in OtherConfig. Check whether companies are ok to change it as mandatorily signalled. 3) FFS on other inference configuration of supported functionalities. Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device.

Based on the above discussion, we think that “NW-side additional condition” in Prop2 refers to Info 4, “NW-side additional conditions” in Prop3 refers to both Info 3 and Info 4, and “UE-side additional conditions” in Prop3 refers to both Info 1 and Info 2.

- Regarding Prop2 and Prop3 in the email discussion [3], RAN2 to confirm that：

- “NW-side additional condition” in Prop2 refers to “NW-sided additional status/ configuration”,

- “NW-side additional conditions” in Prop3 refers to both “NW-sided additional conditions/ requirements” and “NW-sided additional status/ configuration”,

- “UE-side additional conditions” in Prop3 refers to both “UE-sided additional conditions/ requirements” and “UE-sided additional status”.

## 2.4 Functionality activation/deactivation

In the phase 2 of email discussion [3], the initial state of configured functionality (i.e., Q3-2) was discussed as below:

After applicable functionality reporting, it is not clear what is the initial state (active/deactive) of the functionality after Step 5, and how an applicable functionality becomes an activated functionality. There are three options: Option 1: The applicable functionality is activated by receiving configuration for applicable functionalities in Step 5 (if needed). If configuration is not provided by the network, it means the functionality is not activated. Option 2: The applicable functionality is automatically activated if it is included in applicable functionality reporting (assuming the network configuration received in Step 3 is directly applied and the functionality is activated). Option 3: A functionality is activated based on a field in RRCReconfiguration in Step 3 or Step 5 (indicating the functionality activation status), and additionally via L1/L2 based activation/deactivation signaling. L1/L2 based activation/deactivation signaling is up to RAN1. […] Rapporteur Summary 12: As baseline, the applicable functionality is initially activated by receiving configuration (either in Step 3 or 5) for the corresponding applicable functionalities. FFS on additional L1/L2 signaling for activation/deactivation.

In general, a functionality is configured via RRC signaling. We think that the initial state of configured functionality and whether to need additional (L1/L2/RRC) signaling for activation/deactivation depends on the answers of the following 3 questions:

- Q1: whether the configured functionality (when network configures it) must be applicable?

- Q2: whether the activation/deactivation time between the UE and the network must strictly align?

- Q3: whether the activation/deactivation is frequent?

For Q1, if the answer is not, then further activation signaling is needed. However, as discussed in section 2.2 of the paper, we think that the configured functionality should always be applicable.

For Q2, if the answer is yes, e.g., due to the different beam reporting formats for AI-based scheme and non-AI-based scheme, L1 activation/deactivation signaling may be needed to align the activation/deactivation time between the UE and the network. If the answer is no, the initial state of configured functionality can be activated.

For Q3, if the answer is yes, L1/L2 activation/deactivation signaling may be needed since RRC signaling is quite slow and the signaling overhead is more.

Regarding Q2 and Q3, input from RAN1 is needed to further evaluate. To facilitate RAN2’s discussion, RAN2 can assume that the initial state of configured functionality is activated state, which can be revisited after getting more input from RAN1.

- RAN2 assumes that the initial state of the configured functionality is activated state. The necessity of L1/L2 activation/deactivation signaling is up to RAN1 decision.

## 2.5 Applicable functionality reporting

In the phase 2 discussion of email discussion [3], regarding the applicable functionality reporting, proactive reporting and reactive reporting were discussed. The Report [3] had the following proposals:

Proposal 1: Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) It is optional for network to provide NW-side additional condition in OtherConfig. Check whether companies are ok to change it as mandatorily signalled. 3) FFS on other inference configuration of supported functionalities. Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. Proposal 4: “Step 4”: UE considers to report applicable functionality in the following scenarios: 1) Upon change of applicable functionality change via UAI 2) As response to NW-side additional condition, FFS via UAI or RRCReconfigurationComplete, etc Proposal 5: Step 5: 1) Network configures full inference configuration to UE after applicable functionality reporting, if inference configuration is not provided in Step 3 (i.e. full inference configuration is provided in Step 5). 2) If inference configuration is provided to Step 3, it is up to network implementation whether to provide an updated configuration or not.

We think that RAN2 should further focus on unified and complete signaling procedure for proactive reporting and reactive reporting. An example of the unified and complete signaling procedure can be as below (Figure 1 and procedure steps show):

Figure 1: unified and complete procedure (including proactive reporting and reactive reporting)

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE via RRCReconfiguration including OtherConfig that UE is requested to provide its applicable functionalities (and also is requested to update the applicable functionalities when the applicable functionalities change).

Step 4-5:  Upon receiving the RRCReconfiguration including OtherConfig, UE responses the RRCReconfigurationComplete message, and also UEAssistanceInformation message including the applicable functionalities.

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

Step 7-8: UE sends updated applicable functionalities to network upon change of applicable functionalities.

Step 9: Based on updated applicable functionalities, start inference/monitoring based on network/UE activation/deactivation.

In Steps 4-5, one issue is whether the applicable functionalities can be included in the RRCReconfigurationComplete message and remove Step 5. We think that the UEAssistanceInformation message needs to be enhanced anyway to include the applicable functionalities.

- RAN2 to discuss the unified and complete signaling procedure (including proactive reporting and reactive reporting), and use the unified message, i.e., UEAssistanceInformation message, for reactive and proactive reporting.

In the above Step 3, we think that the filtering scheme for applicable functionality reporting can be supported to reduce the signaling overhead, e.g., the network indicates the interested functionalities in Step 3, then the UE only reports the applicable functionalities within the interested functionalities; or the network indicates an associated ID, the UE only reports the functionalities which is applicable for the associated ID. The content of the filtering configuration can be further discussed.

- Support the filtering scheme for applicable functionality reporting to reduce the signaling overhead. FFS on the content of filtering configuration, e.g., associated IDs, interested functionalities.

In the phase 2 discussion of email discussion [3], rapporteur provided the open question for further discussion based on companies’ contributions:

- Whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell?

The issue was raised due to AI mobility use case. However, AI mobility is out of the scope of this WI. Beam management use case is only for serving cell. Therefore, we think that neighbouring cell’s applicability does not need to be considered in this WI.

- Neighbouring cell’s applicability is not considered, as it is not in the scope of this WI.

# Conclusion

In the contribution, we have the following observations and proposals:

Whether to introduce the term of available functionality?

- Do not introduce the term of available functionality unless it is justified, e.g., after RAN1 defines the detailed content of NW-sided additional conditions.

The relationship between configured functionalities and applicable functionalities

- Configured functionalities are always applicable because NW should configure functionalities within applicable functionalities.

Clarification on the NW-side/UE-side additional conditions

Observation 2:	There are the following 4 types of information related to NW-sided/UE-sided addition conditions:

- Info 1: UE-sided additional conditions/requirements (associated with the functionality).

- Info 2: Current UE-sided additional status (associated with UE itself).

- Info 3: NW-sided additional conditions/requirements (associated with the functionality

- Info 4: Current NW-sided additional status/configuration (associated with NW itself).

- Regarding Prop2 and Prop3 in the email discussion [3], RAN2 to confirm that：.

- “NW-side additional condition” in Prop2 refers to “NW-sided additional status/ configuration”,

- “NW-side additional conditions” in Prop3 refers to both “NW-sided additional conditions/ requirements” and “NW-sided additional status/ configuration”,

- “UE-side additional conditions” in Prop3 refers to both “UE-sided additional conditions/ requirements” and “UE-sided additional status”.

Functionality activation/deactivation

- RAN2 assumes that the initial state of the configured functionality is activated state. The necessity of L1/L2 activation/deactivation signaling is up to RAN1 decision.

- RAN2 to discuss the unified and complete signaling procedure (including proactive reporting and reactive reporting), and use the unified message, i.e., UEAssistanceInformation message, for reactive and proactive reporting.

- Support the filtering scheme for applicable functionality reporting to reduce the signaling overhead. FFS on the content of filtering configuration, e.g., associated IDs, interested functionalities.

- Neighbouring cell’s applicability is not considered, as it is not in the scope of this WI .

# Reference

- RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R2-2407165 Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung) – Phase 1

- R2-2406381 Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2