3GPP TSG-RAN WG2 Meeting #127		R2-2406388

Maastricht, Netherlands, 19th – 23rd Aug 2024

Source:	vivo

Title:          Discussion on NW side data collection framework

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#126 meeting, on NW side data collection for beam management, RAN2 made the following agreements:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

In this contribution, we will further discuss some remaining issues on data collection frameworks for NW-side model training.

# Discussion

## 2.1 Data collection for beam management

Enhancements of immediate MDT may require UE to log a certain amount of measurement data before reporting them. Consequently, the reporting enhancement may also be frequent enough to adapt the collection requirement. Thus, it would impose some complexity on UE in terms of memory constraint, power consumption, signaling overhead, etc. Considering how to reduce/minimize UE power consumption and signaling overhead, UE may be allowed its preference on enhanced MDT data logging/reporting, therefore:

- For the enhancement of immediate MDT to support data logging, the following aspects should be taken into consideration:

- UE memory constraint: i.e., logging of data should be minimized to avoid excessive memory utilization.

- UE power consumption and signaling overhead: frequent real-time and/or long-duration data logging should be avoided. Example for UE is allowed to indicate its preference on enhanced MDT data logging/reporting.

- Signaling complexity, i.e., for both gNB-centric and OAM-centric, like current MDT procedure, UE is not expected to be configured with different data collection processes for gNB and OAM.

RAN2 has already agreed that enhanced immediate MDT framework support periodical reporting. On whether and what event-based reporting is supported and FFS on network request, as NW can configure UE with periodical reporting, there is no need to consider event-based reporting. However, when configured with periodical reporting, UE can always perform event triggered logging, if any, within the reporting period. That is, event-triggered logged data is reported in the periodical Report.

- There is no need to support event-based reporting of enhanced MDT report.

To facilitate data filtering and reduce data logging, event-triggered data logging should be supported. However, event-triggered logging does not necessarily mean event-based reporting, e.g., data can be reported periodically.

- To facilitate data filtering and reduce data logging, event-triggered data logging should be supported. And the event-triggered data logging can be sent by periodical reporting.

On the other hand, UE performs enhanced immediate data logging and reporting based on NW request out of energy saving issue. In such case, UE reports logged data only when NW requests, instead of periodical reporting.

- In addition to periodical reporting, the enhanced MDT report with logging data can be reported based on NW request.

Based on RAN2 previous progress and above Proposal 3/Proposal 4, a procedure of data logging and reporting can be considered as in below Proposal 5:

- Baseline procedure of enhanced immediate MDT for NW-sided data collection:

- Step 1: gNB configures enhanced immediate MDT (instances of logged L1 measurement) to a UE in RRC_CONNECTED state via a RRC message for data logging and reporting:

- Step 2: Upon reception of enhanced immediate MDT configuration, the UE stores the configuration, starts performing instances of logged L1 measurements.

- Step 3: The UE determines that reporting of measurements is triggered, including periodically, or requested by gNB, i.e.:

  - Periodic reporting (including of event triggered logged data)

  - Requested by NW based on UE indication of the stored enhanced immediate MDT data availability.

- Step 3: The UE reports the stored instances of logged L1 meas to NW via RRC message.

On how to handle case when single RRC message is not sufficient to report logged data, potential approaches can be considered in twofold:

- Option 1: RRC message segmentation, i.e., logged data are segmented in different RRC messages and reported to NW.

- Option 2: adopt similar logged data reporting framework like logged MDT, i.e., when single RRC is not sufficient to report logged data, UE would indicate to NW data it still has data to be reported. Based on UE indication, NW can further request to report the remaining logged data.

Both options can work, but option 2 may come with additional signaling for both UE and NW in terms of indication and reporting request signaling.

- When a single RRC message is not sufficient to report logged data, RRC message segmentation is used to report logged data.

## 2.2 Data collection for positioning enhancement

At the RAN2#123bis meeting, for positioning enhancement, the following were agreed regarding the NW-side data collection for model inference and model monitoring. However, the data collection for model training was not covered.

Agreements on NW-side data collection

Positioning

For LMF sided inference (case 2b, case 3b), RAN2 assumes LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

For LMF sided performance monitoring, RAN2 assumes LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

At the RAN2#122 meeting, RAN2 discussed the assumptions and requirements of data collection, and sent LS [1] to RAN1. RAN1 replied to part A of the LS (RAN2 assumptions) in [2] and to part B of the LS (Aspects of data collection) in [3]. In the LS, the training data can be generated by UE/PRU/gNB/LMF.

- For positioning enhancement use case: - For model training, training data can be generated by UE/PRU/gNB/LMF and terminated at LMF/OTT server.

From LMF perspective, the PRU functionality is realized by a UE with a known location, and the interaction between LMF and UE/PRU is via LPP. As the segmentation of SLPP message has been supported, the LPP seems well-suited for data collection from UE/PRU to LMF for model training.

- For NW-side model training of positioning accuracy enhancement, RAN2 assumes the data can be collected via LPP from UE/PRU to LMF.

Similarly to data collection for beam management, if the data is collected for model training purposes, the amount can be large enough. The current LPP supports periodical and event-triggered reporting for Location information provision of multiple instances in one single message, however no logging mechanism is supported. We assume that an enhancement of logging positioning data can be further considered.

- RAN2 to consider the enhancement of the logged scheme for positioning meas data in LPP.

# Conclusion

This contribution concludes with the following:

- For the enhancement of immediate MDT to support data logging, the following aspects should be taken into consideration:

- UE memory constraint: i.e., logging of data should be minimized to avoid excessive memory utilization.

- UE power consumption and signaling overhead: frequent real-time and/or long-duration data logging should be avoided. Example for UE is allowed to indicate its preference on enhanced MDT data logging/reporting.

- Signaling complexity, i.e., for both gNB-centric and OAM-centric, like current MDT procedure, UE is not expected to be configured with different data collection processes for gNB and OAM.

- There is no need to support event-based reporting of enhanced MDT report.

- To facilitate data filtering and reduce data logging, event-triggered data logging should be supported. And the event-triggered data logging can be sent by periodical reporting.

- In addition to periodical reporting, the enhanced MDT report with logging data can be reported based on NW request.

- Baseline procedure of enhanced immediate MDT for NW-sided data collection:

- Step 1: gNB configures enhanced immediate MDT (instances of logged L1 measurement) to a UE in RRC_CONNECTED state via a RRC message for data logging and reporting:

- Step 2: Upon reception of enhanced immediate MDT configuration, the UE stores the configuration, starts performing instances of logged L1 measurements.

- Step 3: The UE determines that reporting of measurements is triggered, including periodically, or requested by gNB, i.e.:

  - Periodic reporting (including of event triggered logged data)

  - Requested by NW based on UE indication of the stored enhanced immediate MDT data availability.

- Step 3: The UE reports the stored instances of logged L1 meas to NW via RRC message.

- When a single RRC message is not sufficient to report logged data, RRC message segmentation is used to report logged data.

- For NW-side model training of positioning accuracy enhancement, RAN2 assumes the data can be collected via LPP from UE/PRU to LMF.

- RAN2 to consider the enhancement of the logged scheme for positioning meas data in LPP.

# Reference

[1] R2-2306906	LS out on Data Collection Requirements and Assumptions

[2] R2-2309435	Reply LS on Data Collection Requirements and Assumptions

[3] R1-2310681	Reply LS on Data Collection Requirements and Assumptions