3GPP TSG-RAN WG2 Meeting #127	R2-2406389

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Source:	vivo

Title:	Discussion on UE side data collection

Agenda Item:	8.1.4

Document for:	Discussion and Decision

- Introduction

During the RAN2#126 meeting, RAN2 further discussed the data collection options for UE-side model training, and the characteristics of the options were summarized in the post-meeting email discussion [POST126][034] [AIML PHY] TP for data collection (Ericsson) [1] as following table.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

In this contribution, we will further discuss UE-side data collection to address the FFSs highlighted in the above table. In addition, we will provide potential recommendations regarding UE-side data collection for the RANP#105 meeting.

# Discussion

### 2.1 Privacy concerns

In the email discussion [1], the privacy concerns are captured in the TR 38.843. The following privacy concerns are from OEM vendors’ perspective:

OEM: - User Information Disclosure: OEMs handle a vast amount of user data, including personal information, usage habits, and location data. There is a significant privacy concern if this information is disclosed to external entities without user consent, potentially violating privacy laws and damaging the OEM's reputation. OEMs are adamant that user data should not be shared with third-party entities without explicit and informed user consent, since a UE vendor might be legally bound by a data protection agreement with the end-user. - Proprietary Technology Exposure: OEM vendors develop specialized hardware and software that may contain trade secrets or patented technologies. There is a risk that this information could be unintentionally disclosed to unauthorized parties, leading to privacy breaches. Another risk is that some sensitive data of an OEM vendor may be exposed to third parties without the knowledge of the OEM vendor.

To protect data privacy across different operators, the current MDT supports UE filtering based on PLMN list. When a management-based MDT activation is sent to a gNB, the gNB shall check the availability of the management-based MDT PLMN List IE before  UE selection. If the Management-based MDT PLMN List IE is not available, the gNB shall not select the UE; if the Management-based MDT PLMN List IE is available, the gNB shall verify if the UE’s RPLMN matches the PLMN where TCE resides, i.e., Trace Reference PLMN (PLMN portion of the Trace Reference). The gNB shall not select the UE as well for the condition of mismatching.

To address the privacy concerns among different OEM vendors, a finer granularity of UE filtering can be considered. Specifically, the server for data collection for UE-side model training should only initiate data collection target for the UE manufactured by a trustworthy UE vendor. Meanwhile, the mechanism should allow UE to decide whether to grant consent to trustworthy server(s).

- It is essential to address the privacy concerns for UE-side data collection at the server, the following requirements should be considered:

  - The server for data collection for UE-side model training is expected to perform data collection only from specific UE, e.g., UE manufactured by a trustworthy vendor;

  - The UE is allowed to accept or reject server request for data collection.

### 2.2 FFSs of UE-side data collection

#### 2.2.1 UP/CP tunnel

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |

To address the FFSs related to UP/CP tunnel of Option 2/3, it is essential to first clarify the applicable use cases of Option 2/3.

In the current table, the first termination entities of Option 2 and Option 3 are CN and OAM, respectively. If the NW intends to collect data to train network-sided models, the data could also be shared with the server for UE-side data collection. In other words, the NW-side data collection frameworks can also be utilized for UE-side data collection.

For positioning, LPP is reused for data collection from the UE to LMF. For beam management use cases, the MDT framework is considered for OAM-centric data collection and model training. In this context, Option 2 is applicable for positioning sub-use cases 1a and 2a, i.e., LMF receives the training data from UE/PRU via LPP and transfers the data to the server for data collection for UE-side model training. Option 3 is applicable for beam management, i.e., OAM receives the training data from UE via MDT and transfers the data to the server for data collection for UE-side model training.

- Option 2 is applicable for positioning sub-use cases 1a and 2a, i.e., LMF receives the training data from UE/PRU via LPP and transfers the data to the server for data collection for UE-side model training.

- Option 3 is applicable for beam management, i.e., OAM receives the training data from UE via MDT and transfers the data to the server for data collection for UE-side model training.

In Rel-18, UP-based positioning mechanism was introduced by SA2. LMF and UE may utilize a UP-based connection to transfer supplementary services messages and LPP messages. If LMF decides to employ UP-based positioning, it should indicate the UE with the information to establish a secure UP-based connection; and via this UP-based connection, LPP messages can be transferred between UE and LMF. As UP-based positioning is in SA2 scope, whether it can be reused for AI-based positioning can be determined by SA2.

As to the MDT, no UP-based solution is currently supported. We tend to focus on reusing CP-based solution and not introducing UP-based MDT for AI.

Based on the analysis above, we propose:

- For the UP/CP tunnel of Option 2, further clarify that LPP is one of the typical NAS signalling for CP tunnel. Whether UP tunnel is feasible is left to SA2 decision.

- For the UP/CP tunnel of Option 3, remove the FFSs related to UP tunnel.

#### 2.2.2 Controllability of MNO on data transfer

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |

For the controllability of MNO on data transfer, only “full controllability” was defined in [1] as:

- Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed.

For Option 2 and Option 3, the MNO has full controllability on the data transfer as the first termination entities, i.e., CN and OAM, are within MNO’s network. In Option 1b, the first termination entity is the server for data collection, which is also inside the MNO’s network. From our understanding, the MNO should be able to permit or reject the data exchange between UE and the server, e.g., by permitting or rejecting the setup of the PDU session. In addition, the MNO is aware of the initiating and terminating of the data transfer, e.g., via setup and release of the PDU session. The difference between Option 1b and Option 2/3 is that the MNO can not control when to initiate and terminate the data transfer. To differentiate the controllability of Option 1b from Option 1a/2/3, another level of controllability should be introduced, i.e., partial controllability, with the following definition:

- Partial controllability: The MNO has the capability to permit or reject data exchange between UE and server. In addition, the MNO is aware of the initiating and terminating of data transfer.

- Introduce another level of MNO controllability on data transfer, i.e., partial controllability. The definition can be: The MNO has the capability to permit or reject data transfer between UE and the server. In addition, the MNO is aware of the initiating and terminating of data transfer.

- For the controllability of MNO on data transfer, Option 1b should be classified as having partial controllability.

#### 2.2.3 Options for Visibility

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |

For the visibility of data content in MNO, the following options are identified to realize the different levels of data content visibility to the MNO:

- Full visibility for standardized data content.

- Partial visibility for partially standardized data content.

- No standardized visibility.

For Option 2/3, the MNO could have full visibility of the standardized data content. One remaining FFS is whether Option 2/3 can support partial visibility for partially standardized data content. From our understanding, the NW collects essential data from UE and then transfers the data to the server for data collection for UE-side model training/OTT server. For the first step, the NW should conduct a privacy check to obtain user consent, and the permission granted or denied should be based on standardized data content. In other words, the user is not expected to permit the data collection if the data is unspecified. Therefore, Options 2/3 should only have full visibility and the FFSs related to partial visibility can be removed.

- For the visibility of data content in MNO, Options 2 and 3 are only with Full visibility. FFSs related to other levels of visibility should be removed.

For Option 1b, the visibility is still FFS. As we analyzed in section 2.2.2, the MNO has the capability to permit or reject data exchange between UE and server for Option 1b. From our understanding, the permission or rejection is also based on standardized data content. That is, the NNO is not expected to permit the data transfer if the data is unspecified. Therefore, Option 1b is also with full visibility for standardized data content.

- For the visibility of data content in MNO, Option 1b is also with Full visibility for standardized data content.

The TP and updated table based on the above analysis of section 2.2 is attached in the Annex.

- Agree to the TP on UE-side data collection in the Annex.

### 2.3 Potential recommendations for RANP#105

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |

The UE-sided data collection is one of the study objectives with checkpoints in RANP#105 (Sept ’24). From RAN2’s perspective, the potential options are analyzed to understand potential specification impact. Except for Option 1a, Options 1b/2/3 may have specification impact on multiple WGs.

It may not be so simple for RAN2 to down-select among the different options, considering that no option is deemed infeasible from RAN2’s perspective. Further cross-WG discussions are needed in RANP#105, based on the analysis of RAN2.

- No down-selection of options for UE-sided data collection is expected in RAN2.

- RANP to further discuss the options based on the analysis of RAN2.

- Conclusion

This contribution concludes with the following:

Privacy concerns

Proposal 1	It is essential to address the privacy concerns for UE-side data collection at the server, the following requirements should be considered:

  - The server for data collection for UE-side model training is expected to perform data collection only from specific UEs, e.g., UEs manufactured by a trustworthy vendor;

  - The UE is allowed to accept or reject server(s) requests for data collection.

FFSs of UE-side data collection

Proposal 2.	Option 2 is applicable for positioning sub-use cases 1a and 2a, i.e., LMF receives the training data from UE/PRU via LPP and transfers the data to the server for data collection for UE-side model training.

Proposal 3.	Option 3 is applicable for beam management, i.e., OAM receives the training data from UE via MDT and transfers the data to the server for data collection for UE-side model training.

Proposal 4.	For the UP/CP tunnel of Option 2, further clarify that LPP is one of the typical NAS signalling for CP tunnel. Whether UP tunnel is feasible is left to SA2 decision.

Proposal 5.	For the UP/CP tunnel of Option 3, remove the FFSs related to UP tunnel.

Proposal 6.	Introduce another level of MNO controllability on data transfer, i.e., partial controllability. The definition can be: The MNO has the capability to permit or reject data transfer between UE and the server. In addition, the MNO is aware of the initiating and terminating of data transfer.

Proposal 7.	For the controllability of MNO on data transfer, Option 1b should be classified as having partial controllability.

Proposal 8.	For the visibility of data content in MNO, Options 2 and 3 are only with Full visibility. FFSs related to other levels of visibility should be removed.

Proposal 9.	For the visibility of data content in MNO, Option 1b is also with Full visibility for standardized data content.

Proposal 10.	Agree to the TP on UE-side data collection in the Annex.

Potential recommendations for RANP#105

Proposal 11.	No down-selection of options for UE-sided data collection is expected in RAN2.

Proposal 12.	RANP to further discuss the options based on the analysis of RAN2.

- Reference

- R2-2407209	RAN2 inputs to TR 38.843

- Annex: TP on UE-side data collection

START OF CHANGE

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that for the different options, the data content visibility was discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) (Note 5) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel (Note 5) | RRC layer for CP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Partial controllability (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Opt A) Full visibility for standardized data content. | Opt A) Full visibility for standardized data content. | Opt A) Full visibility for standardized data content. |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. Partial controllability: The MNO has the capability to permit or reject data exchange between UE and server. In addition, the MNO is aware of the initiating and terminating of data transfer. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: For Option 2, LPP is one of the typical NAS signalling for CP, and whether UP tunnel is feasible is left to SA2 decision. |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.

END OF CHANGE