3GPP TSG-RAN WG2 Meeting #127		R2-2406390

Maastricht, Netherlands, 19th – 23rd Aug 2024

Source:	vivo

Title:	Discussion on functionality aspects for Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion and Decision

# Introduction

At the last RAN2#126 meeting [1], the following agreements have been reached for Stage 2 general aspects and functionality aspects:

Agreements on Stage 2 General aspects

1	As baseline, the “inventory only” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

2	As baseline, the “inventory and command” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

3	From RAN2 point of view we will study “Command only” use case.

FFS the options on how to support it:

Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command)

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed.

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed).

In this contribution, we will further discuss the remaining issues based on the above agreements, and provide our views respectively.

# Discussion

## Assumption on AIoT upper layer

Based on the RAN2-led objective description from the SID [2], some examples of the potential AIoT functions have been given for further study, which are listed as below.

RAN2-led objectives from the SID For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

For the above highlighted part, i.e., Interactions with upper layers, it is observed that company views are divergent on what the upper layers refer to. According to SA2 TR [3], the upper layers for A-IOT device are proposed as the AIoT app layer, AIoT NAS layer, or a new A-IOT layer in different solutions. From RAN2 perspective, we can keep all options on the table open and wait for SA2 final conclusions on the definition of upper layers. To facilitate discussion on data transmission, it is proposed to adopt a more general terminology for “upper layers” to cover any possible option that is under SA2 study.

- RAN2 assumes that AIoT Upper layer is supported, which is a protocol layer on top of AIoT AS layer, wherein the AIoT Upper layer is responsible for transfer of device ID and/or any other upper layer data. Details are up to SA2.

## Use case of “Command”

RAN3#124 meeting agreed that “Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.”.

- It is agreed at RAN3#124 that Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

In line with the above agreement, Command over AIoT radio interface can be used for a single device is also sent in unicast manner accordingly.

- In line with Command over CN-RAN interface for a single device, Command over AIoT radio interface can be sent in unicast manner as baseline.

Moreover, for the support of Command on a group of devices, or all devices, RAN2 may need further study on how the Command over the AIoT radio interface can be sent from AS procedure perspective, e.g.., in unicast, groupcast or broadcast manner.

Firstly, for the detailed use case of “inventory and command”, it is supported by the procedure with step A, step B, step C1 and step C2, as baseline. See below Figure in the latest TP [4]. Command over the AIoT radio interface is sent during Step C. Given that the reader has already uniquely identified a device via random access procedure (via random identifier) or D2R data transmission (via device ID) during Step B, we think the Command over the AIoT radio interface can be scheduled individually and reuse the unicast manner for a single device as baseline. While the Command over the AIoT radio interface sent in groupcast or broadcast manner over the AIoT radio interface is kind of resource and signaling optimization and also dependent on the discussion in other WG, which can be considered later.

Figure 6.2.1-1 Overall AS procedures between A-IoT device and reader

Secondly, for the detailed use case of “command-only”, there are two alternatives described as below according to the latest TP [4].

-	Alt 1: It can be also supported by the baseline procedure with step A, step B, step C1 and step C2.

-	Alt 2: another candidate to support this use case is following, whose feasibility still depends on the conclusion from [SA2 TR 23.700-13] and [SA3 TR 33.713]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g. the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

For Alt 1, since it is supported by the baseline procedure for “inventory and command”. How to send the Command over the AIoT radio interface can follow the same principle as that for “inventory and command”. Therefore, we have the following proposal.

- For the use cases of “inventory and command” and “command only” using the baseline procedure for “inventory and command”, RAN2 assumes that Command is sent over the AIoT radio interface in unicast manner to each device individually to achieve Command over CN-RAN interface on a group of devices, or all devices (if supported).

While for Alt 2, the main advantage lies in the procedure efficiency with reduced steps to support Command on a group of devices, or all devices. Nevertheless, there are also some challenges (except security) from AS procedure perspective, which are listed as following:

- Payload size limitation: Command to be included in the Paging message increases the payload size, the potential size of Command can be very large depending on specific service scenario. Meanwhile, in order to guarantee certain reader coverage, the payload size for the Paging message may be limited.

- Resource efficiency: Resources for D2R response message(s) to be indicated in the Paging message are based on an estimated number of devices. If Paging message fails to reach some devices, indicated resources in the Paging message may be wasted.

Therefore, we suggest RAN2 to deprioritize study on how Command is sent over the AIoT radio interface to achieve Command over CN-RAN interface on a group of devices, or all devices in the use case of “command only” until conclusions are clear in other WGs including SA2, SA3 and RAN3.

- For the use case of “command only” supported by the Initial trigger message to contain Command, RAN2 to deprioritize study on how Command is sent over the AIoT radio interface until conclusions are clear in other WGs including SA2, SA3 and RAN3.

## Device temporary ID assignment

In RFID, the 16-bit random identifier (RN16) self-assigned by Tag is used to uniquely identify a Tag for subsequent data transmission to/from this Tag. Meanwhile, in legacy NR Uu, an AS Temporary ID (e.g., C-RNTI) is configured to a UE for subsequent data transmission to/from this UE. For device temporary ID usage over the AIoT air interface, two options that are on the table either following RFID or legacy NR design.

- Option 1: No need of Device temporary ID assignment by reader.

  - In Option 1, a 16-bit random identifier self-assigned by AIoT device via random access procedure is directly used by reader to uniquely identify a specific AIoT device over the air interface.

- Option 2: Need for Device temporary ID assignment by reader.

  - In Option 2, the reader assigns a device temporary ID in accordance with the number of the AIoT device(s) accessed to this reader.

For Option 1, it has some advantages of specification simplicity since the efforts to introduce a new CP procedure for device temporary ID assignment are saved. While for Option 2, the length of the device temporary ID can be varied and shorter than 16-bit, which adapts to the number of AIoT devices accessed to the reader. For example, when the number of AIoT devices is 64, an 8-bit device temporary ID can be enough. Therefore, Option 2 has some advantage of saving overhead for subsequent data transmission especially when the number of AIoT devices managed within the reader is small. Considering that both options can work but with different advantages, we suggest that RAN2 to further evaluate which option is more suitable for AIoT device.

- RAN2 to study the following options to uniquely identify a specific AIoT device over the AIoT air interface by reader.

  - Option 1: Device temporary ID self-assignment by device, e.g., the 16-bit random identifier self-assigned by AIoT device via random access procedure is directly used by this reader.

  - Option 2: Device temporary ID assignment by reader, e.g., the reader assigns a device temporary ID in accordance with the number of the AIoT device(s) accessed to this reader.

## Device type/capability reporting

According to RAN1#116 agreement, three device type has been agreed as below.

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

At the last RAN2#126 meeting, there was initial discussion on the assumptions on the need of device type and device capability reporting but with no consensus. Although some companies think that it may be too early to conclude the necessity of device type and device capability reporting, we believe that some guideline on these aspects is still beneficial to facilitate RAN2 future study. Moreover, since our study strives for harmonized air interface design for all device types, any functionality if required is better to be designed commonly for all device types as much as possible. Hence, to achieve the harmonized design, it is proposed that RAN2 study should aim not to support device type/capability information reporting procedure as baseline.

- RAN2 study should aim at solutions that do not require device type and device radio capability information reporting from device to reader as baseline.

## Service request visibility to reader

In RAN2#125bis, there were preliminary discussions on what information in the service request from CN should be visible to reader. As far as our understanding, upon reception of a service request from CN, the reader should identify the required AS procedure to complete the service request, e.g. is it for an inventory procedure, a write procedure or a read procedure or any other procedure, wherein a write/read procedure may comprise an inventory procedure for the reader to identify the target AIoT device and to complete the handshake with the target AIoT device for processing the subsequent write/read procedure.

- It is mandatory that based on the received service request from CN the reader can identify the related AS procedure(s) to complete this service request.

In more details, the following awareness in relation to the service request from CN by the reader can be considered:

- If the reader can know the identity of the target AIoT device(s), the reader can identify the target AIoT device(s) to execute further AS procedures, e.g. read/write procedure.

- If the reader can know the rough population from the service request for inventory, the reader can determine proper Q value for slotted ALHOHA based inventory procedure, wherein the Q-value can be used by the reader to determine the amount radio resources (i.e. the number of slots) to complete one round of inventory procedure.

- If the reader can know the D2R data/message size from the service request for reading operation, the reader can determine the amount of radio resources that is required for the D2R data/message transmission.

- If the reader can know the periodicity to periodically execute the received service request (e.g. periodically read the temperature from an AIoT device for monitoring the environment temperature), the reader can periodically execute the read procedure according to the periodicity.

- In addition, the reader should also be able to derive the QoS requirement from the service request from CN so that the reader can execute the related AS procedure according to the QoS requirement. Typically, the e2e latency fulfillment can be considered. Furthermore, for inventory service for certain population of AIoT devices in the reader’s coverage, certain target ratio for successful inventory of the target in-coverage devices can help the reader to determine the R2D/D2R radio parameters e.g. coding rate, transmit power level etc, to fulfill the target inventory ratio of devices in reader’s coverage.

Based on the above discussion, we propose:

- The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

  - the service type information, e.g. inventory, read/write command.

  - the ID of the target device.

  - the population of target devices.

  - D2R data/message size.

  - periodicity to execute the service request.

  - QoS requirement (e.g. e2e latency and target successful inventory ratio)

## MAC PDU structure

In RAN2#125bis, it was agreed that SDAP/PDCP/RLC layer will not be supported in RAN2. MAC PDU should be used to configure the AS actions of the AIoT device, provide the associated parameters to AIoT device (if there is) to take these actions and transmit the related associated data/message (if there is) for the AIoT device to take these actions.

For one instance, during an inventory procedure, when the reader pages/selects the target AIoT device(s) via a R2D MAC PDU, the R2D MAC PDU may carry both the paging indication and the associated parameters (e.g. AIoT device ID filtering parameters).

For another instance, during an inventory procedure, when the reader pages the target AIoT devices, the reader can construct an R2D MAC PDU comprising the paging indication and the Q value.

For one more instance, during a write procedure, the reader can send to the target AIoT device a R2D MAC PDU comprising the write indication, the memory indication and the associated data.

To be more generic, a message type field can be defined in R2D MAC PDU, wherein the message type field indicates the AIoT device action (e.g. paging/query/write) and the MAC PDU content/format. For each message type, there could be associated parameters to configure the AIoT device to execute the corresponding AS action. Some of parameters could be conditionally present parameters and/or there could be associated upper layer message/data, complementary fields to indicate the presence of these fields could be necessary.

Based on the above discussion, the following R2D MAC PDU structure is proposed:

- Design an R2D MAC PDU format comprising:

  - message type, which indicates AIoT device action (e.g. paging/read/write) and the MAC PDU format/content,

  - associated parameters (e.g. device ID/filter, Q-value) to facilitate AIoT device action if any, and

  - associated upper layer signalling message/service data if any.

When a R2D MAC PDU is received by an A-IoT device and the AIoT device determines that it is the target AIoT device for receiving the MAC PDU and taking the related actions, the AIoT device may generate a D2R MAC PDU if it determines a D2R MAC PDU in response is required.

For one instance, if an AIoT device has received a R2D MAC PDU for paging the AIoT device, the AIoT device may generate a D2R MAC PDU comprising of certain ID (e.g. RN16) and sends the D2R MAC PDU to the reader. As the D2R MAC PDU is in response to the R2D MAC PDU, the format/content of the D2R MAC PDU can be fully determined based on the knowledge of the corresponding R2D MAC PDU content. In such sense, the message type field is not mandatory for D2R MAC PDU when the D2R MAC PDU transmission is always triggered by the received R2D MAC PDU.

- When a D2R MAC PDU is in response to an R2D MAC PDU, the content/format of the D2R MAC PDU can be identified by the reader based on the R2D MAC PDU content.

- Design D2R MAC PDU format according to the corresponding R2D MAC PDU, wherein the D2R MAC PDU’s format/content is determined based on the corresponding R2D MAC PDU.

In radio transmission protocol, there are the following purposes for sequence number in a service packet:

- identify the packet loss so that related upper layer retransmission in the transmitter can be triggered.

- identify the duplicated packets and remove the duplicated ones.

- to enable in-order-delivery to upper layer in the receiver side.

According to the previous discussion, as PDCP/RLC is not to be supported for AIoT, there is no upper layer (MAC and above) retransmission which requires duplicated packet removal. Furthermore, in case of multiple repetitions for one MAC PDU in physical layer, the physical layer always stops decoding once the TB is correctly decoded and no duplicated TB delivery from physical layer to MAC layer. With these conclusions, it can be derived that if there are multiple service data packets for one service, the service data packets will be sequentially received and delivered to upper layer, i.e. in-order-delivery to upper layer can be automatically ensured. This means that none of the above purposes for sequence number is still valid for AIoT service.  Hence, we propose:

- No MAC layer sequence number is needed for service data transmitted in MAC PDU.

## Multiplexing

It was agreed that multiple AIoT logical channel will not be supported. This means that there will no multiple con-current services for one AIoT services. However, it is not determined whether there is only single or multiple service data packet(s) per service for one AIoT device. As far as our understanding, even if there can be multiple service data packets per service for one AIoT device, the reader can schedule multiple individual transmissions, wherein each transmission can transmit one individual service packet. In this way, the fixed overhead of the MAC sub-header to indicate the start/length of a sub-MAC PDU can be saved. This can be very essential from the spectrum efficiency perspective considering that the data rate of the AIoT air interface is even clearly lower than that of NB-IoT.

- If there are multiple data packets for one AIoT service, the packets can be respectively transmitted in separate transmissions.

Hence, we would kindly request RAN2 to agree the following proposal:

- RAN2 assume no multiplexing of multiple service data packets in one MAC PDU.

## BSR

In RAN#126, there were the following conclusions regarding the SR/BSR procedure for AIoT:

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

For D2R transmission of upper layer data/msg of variable size for some service, there can be no exact data/msg size information in the service request from the CN. However, to allocate the proper amount of radio resources for data transmission from the AIoT device, the reader still needs to know the exact buffer size of the AIoT device. From this perspective, certain BSR mechanisms are still necessary for AIoT devices.

- In case that CN cannot give the exact D2R message size, BSR is required to facilitate the D2R radio resource allocation by the reader.

- BSR should be supported for AIoT device.

If certain BSR procedure is supported, certain BSR triggering procedure should be considered. Basically, it could be preferred that at most one BSR is triggered for one D2R message. With the BSR, the reader can determine the amount of D2R resource to be allocated for the corresponding D2R transmissions. Typically, the BSR should be triggered by an AIoT device based on whether there are remaining bits that have not been reported in a previous BSR.

- If BSR is supported, BSR is triggered when the following two conditions are fulfilled:

  - the first transmission of the upper layer message.

  - there is remaining data after the current transmission.

In previous meetings, single bit SR/BSR was proposed to indicate there were data left in the AIoT device after the current D2R transmission. However, upon reception of single-bit SR/BSR, the reader can only know there is data left in the UE side but no information regarding the exact number of remaining bits. The reader cannot determine the amount of resource that is required to transmit the remaining bits. If less radio resource than required is allocated, more iterations of UL transmissions are required, which results large overhead due to more associated scheduling messages. If more radio resources are allocated than the required radio resource for the transmission of the remaining bits, there is radio resource waste.

- With single bit BSR, the reader cannot derive the amount of radio resource required by the remaining bits, which result in low D2R radio resource efficiency.

There will be no concurrent multiple services for an AIoT device as multiple AIoT logical channel will not be supported. A very simplified BSR with single BS field to indicate the total buffer size of the AIoT device could be considered. For instance, the BSR format with only single-byte BS field (i.e. no LCG/LCH indication in BSR) can be adopted as the maximum AIoT message size is around 1000 bits. For simplicity, a Linear BS table can be considered for quantization of the number of remaining bits.

- If BSR is supported, RAN2 to support single-byte BSR using to indicate the total number of the remaining bits.

## AIoT transport channel

In RAN2#126, some companies proposed to introduce the D2R/R2D transport channel to facilitate the specification procedure description for upper layer data to physical channel mapping. However, the introducing of transport channel has relevance with AIoT logical channel. As the AIoT logical channel was considered to depend on the final modelling issue of MAC, the necessity of AIoT transport channel should also depend on the final modelling issue of MAC.

- Whether AIoT logical channel or AIoT transport channel is used depend on final modelling issue of MAC.

# Conclusion

This contribution mainly discussed the remaining issues from functionality aspects for Ambient IoT. The contribution concludes with:

- It is agreed at RAN3#124 that Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

- In line with Command over CN-RAN interface for a single device, Command over AIoT radio interface can be sent in unicast manner as baseline.

- It is mandatory that based on the received service request from CN the reader can identify the related AS procedure(s) to complete this service request.

- When a D2R MAC PDU is in response to an R2D MAC PDU, the content/format of the D2R MAC PDU can be identified by the reader based on the R2D MAC PDU content.

- If there are multiple data packets for one AIoT service, the packets can be respectively transmitted in separate transmissions.

- In case that CN cannot give the exact D2R message size, BSR is required to facilitate the D2R radio resource allocation by the reader.

- With single bit BSR, the reader cannot derive the amount of radio resource required by the remaining bits, which result in low D2R radio resource efficiency.

Assumption on AIoT upper layer

- RAN2 assumes that AIoT Upper layer is supported, which is a protocol layer on top of AIoT AS layer, wherein the AIoT Upper layer is responsible for transfer of device ID and/or any other upper layer data. Details are up to SA2.

Use case of “Command”

- For the use cases of “inventory and command” and “command only” using the baseline procedure for “inventory and command”, RAN2 assumes that Command is sent over the AIoT radio interface in unicast manner to each device individually to achieve Command over CN-RAN interface on a group of devices, or all devices (if supported).

- For the use case of “command only” supported by the Initial trigger message to contain Command, RAN2 to deprioritize study on how Command is sent over the AIoT radio interface until conclusions are clear in other WGs including SA2, SA3 and RAN3.

Device temporary ID assignment

- RAN2 to study the following options to uniquely identify a specific AIoT device over the AIoT air interface by reader.

  - Option 1: Device temporary ID self-assignment by device, e.g., the 16-bit random identifier self-assigned by AIoT device via random access procedure is directly used by this reader.

  - Option 2: Device temporary ID assignment by reader, e.g., the reader assigns a device temporary ID in accordance with the number of the AIoT device(s) accessed to this reader.

Device type/capability reporting

- RAN2 study should aim at solutions that do not require device type and device radio capability information reporting from device to reader as baseline.

Service request visibility to reader

- The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

  - the service type information, e.g. inventory, read/write command.

  - the ID of the target device.

  - the population of target devices.

  - D2R data/message size.

  - periodicity to execute the service request.

  - QoS requirement (e.g. e2e latency and target successful inventory ratio)

MAC PDU structure

- Design an R2D MAC PDU format comprising:

  - message type, which indicates AIoT device action (e.g. paging/read/write) and the MAC PDU format/content,

  - associated parameters (e.g. device ID/filter, Q-value) to facilitate AIoT device action if any, and

  - associated upper layer signalling message/service data if any.

- Design D2R MAC PDU format according to the corresponding R2D MAC PDU, wherein the D2R MAC PDU’s format/content is determined based on the corresponding R2D MAC PDU.

- No MAC layer sequence number is needed for service data transmitted in MAC PDU.

Multiplexing

- RAN2 assume no multiplexing of multiple service data packets in one MAC PDU.

BSR

- BSR should be supported for AIoT device.

- If BSR is supported, BSR is triggered when the following two conditions are fulfilled:

  - the first transmission of the upper layer message.

  - there is remaining data after the current transmission.

- If BSR is supported, RAN2 to support single-byte BSR using to indicate the total number of the remaining bits.

AIoT transport channel

- Whether AIoT logical channel or AIoT transport channel is used depend on final modelling issue of MAC.

# References

- RAN2#126 Meeting Chair Notes.

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR.

- TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19).

- R2-2406271, TP for TR 38.769 update, Huawei.