3GPP TSG-RAN WG2 Meeting #127		R2-2406392

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	Random Access Procedure for A-IoT Device

Agenda Item:	8.2.4

Document for:	Discussion

# Introduction

In last RAN2 meeting, random access procedure of A-IoT device had been discussed and some initial agreements were achieved as followings [1]:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we continue to discuss details for A-IoT random access procedure, which include determining CBRA or CFRA, determining 2-step RACH or 4-step RACH, access failure handling, Power ramping or repetition for RACH and Multiple access for RACH.

# Discussion

## 2.1 Determining CBRA or CFRA

As the above RAN2 agreements, both CBRA and CFRA will be supported in A-IoT interface. In CBRA procedure，an A-IoT device uses contention based resource and RACH Occasion to access with the reader. However, in CFRA procedure, an A-IoT device uses dedicated resource and RACH Occasion, which are totally different behaviors with each other.

About which entity and how to determine CBRA or CFRA, as the controller of RACH resource and paging procedure, it is up to the reader to determine, e.g. selecting CBRA for a paging procedure with an uncertain number of devices or selecting CFRA for a paging procedure with a known number of devices. In SA2, the details of paging information are still under discussion.  There are two ways to inform the reader about the number of devices related to a paging procedure: the number of paged device IDs that the can be visible by reader (e.g. for a device ID based paging) or explicit number indicator in message from CN to reader (e.g. for a mask or filter based paging). Furthermore, for a CFRA procedure, not only the number of paged devices but also the order of paged devices shall be exactly known by both reader and A-IoT device(s), where each paged device can find its own contention-free resources in an accurate order. It seems that only device ID based paging or a mask/filter based paging for one device can meet the conditions of CFRA.

Observation 1: 	Whether there is a certain number and order of devices for a paging procedure is one of the key factors in reader’s determination of CBRA or CFRA, which is informed from CN to reader.

When the reader obtains a certain number and order of paged devices from CN, it may select a CFRA procedure. Otherwise, a CBRA procedure is selected. On the other hand, whether a CFRA is always selected upon a paging procedure with the certain number and the order of devices, we think it can be left to reader implementation, e.g. trying CBRA in small quantities may be more efficient than trying CFRA directly when a large number of paged devices are not covered by the reader.

After reader selection of CBRA or CFRA, it should be indicated by reader in Msg0/paging to A-IoT device(s), e.g. providing contention-free access resources or explicit indicator. For example, Q value = 0 is a typical CFRA procedure, where only one specific device is permitted to access. Or Q value =2 with a CFRA indicator means that 4 specific devices are permitted to access orderly with contention-free resources. Detailed design can wait for RAN1 progress and RAN2 further discussion.

- CBRA or CFRA should be indicated by reader in Msg0/paging to A-IoT device(s), e.g. providing contention-free access resources.

## 2.2  Determining 2-step RACH or 4-step RACH

In legacy Uu interface, 2-step and 4-step RACH configuration are provided by gNB and UE can choose one of them based on some pre-defined criteria, e.g. resources and RSRP thresholds. However, in A-IoT interface, there is no SIB or pre-configured solutions. It is a feasible way that 2-step RACH or 4-step RACH is indicated by reader in Msg0/paging, e.g. providing different Msg1 resources size or explicit indicator. For example, the reader can select 2-step RACH, e.g. when the number of paged devices is small (i.e. with low collision probability) and the requirement of service delay is higher. Details can be left to reader implementation.

- 2-step RACH or 4-step RACH is indicated by reader in Msg0/paging to A-IoT device(s), e.g. providing different Msg1 resources size or explicit indicator.

On the other side, from the device perspective, 2-step RACH should not be an optional capability since differentiated  capabilities will cause A-IoT reporting procedure and device itself more complex and furthermore 2-step RACH, i.e. just combine Msg3 with Msg1, is not difficult for a basic device. Or it needs to be discussed whether a device is permitted to perform a 4-step RACH when the reader informs that a 2-step RACH is triggered. In our understanding, even with running out of energy device(s), an autonomous fallback scheme from 2-step RACH to 4-step RACH seems useless since a 4-step RACH procedure needs longer latency and detection; and there is no clear evidence that 4-step RACH would be easier than 2-step one.

- RAN2 to discuss whether device(s) autonomous fallbacks from 2-step RACH to 4-step RACH is needed.

## 2.3 Access Failure Handling

There are two types of failure:  contention resolution failure and access failure. Contention resolution failure means that a device can not receive successfully Msg2/MsgB including the same random ID in Msg1/MsgA within a certain time frame, which may mean that collision occurs or link quality is poor. For 2-step RACH, we propose device also carries a random ID in MsgA to achieve a unified architecture between 2-step and 4-step RACH. In legacy Uu, a RAR window is pre-defined for Msg2 reception. However, it is difficult for A-IoT interface to maintain accurate time synchronization and multi-process RACH (except for further TDM or FDM solution). Thus, a device can determine contention resolution failure immediately if next Msg2/MsgB with other random ID than itself or next Msg0 is received.

- For 2-step RACH, device also carries a random ID in MsgA and the reader may echo random ID from MsgA to device in MsgB like that in 4-step RACH for contention resolution.

- A device can determine contention resolution failure immediately if next Msg2/MsgB with other random ID than itself or next Msg0 is received (without further TDM or FDM solution).

Access failure may include Msg3 failure or no Msg3/MsgA upper layer part confirmation. After the reader identified a device with random ID (e.g. RN16), there are three alternatives to determine the access success:

Alt 1: After the device sends the Msg3;

Alt 2: After the device receives the Msg3 confirmation from reader;

Alt 3: After the device receives the upper layer (above AS) confirmation of Msg3;

Alt 3 means that access procedure resembles more like a upper-layer shakehand procedure, e.g. EPC ID in Msg3 and confirmation from upper layer after CN authentication and so on. It seems that only confirmation from upper layer can clearly validate an inventory success. However, Alt 3 is up to SA2 and it is not clear whether a upper layer confirmation of Msg3 is always needed. Furthermore, the upper layer confirmation of Msg3 may need more latency and occupation in A-IoT interface. Hence, it is better to determine the access success in RAN scope, e.g. to avoid interleaving cross layers.

The actual transmission of A-IoT interface may suffer from block errors, especially the HARQ feedback and retransmission mechanism is not supported. Even if there is no collision, the access procedure still has the probability of failure, e.g. Msg2 failure or Msg3 failure. Alt 2 seems to be useful for detection of these failures and the state consistency between reader and device. However, according to some preliminary evaluation, the performance of D2R transmission is usually better than that of R2D, which means the transmission of Msg3 is successful with a high probability if device can receive Msg2 successfully. Hence, a Msg3 confirmation from reader is not very necessary. Alt 1 is the simplest and feasiable. After the device sends the Msg3, it can consider the access success and no re-access is needed any more.

- For 4-step RACH, after the device sends the Msg3, it can consider the access success and no re-access is needed any more.

When a device detects contention resolution failure or access failure, it has two options to re-access:

- Option 1: a device which detects contention resolution failure or access failure, re-accesses in the same “RACH round”.

- Option 2: a device which detects contention resolution failure or access failure, re-accesses in the next “RACH round”.

A “RACH round” can be defined as a time period over which device can randomly select resources for its UL transmissions based on a Msg0 with new access parameters, e.g. randomly (re)select a RACH occasion upon new Q value .

In general, A-IoT device can not maintain steady downlink synchronization. Each RACH occasion may be explicitly indicated by reader, e.g. via Msg0. Msg0 can be divided into two types:  one for each RACH occasion and another for each RACH round. The difference between these two Msg0 is whether new access parameters can be set and device can (re)select randomly resources, e.g. where device only can (re)select randomly RACH occasion upon a new round starting and the RACH occasion of initial selection is maintained throughout the round cycle.

Observation 2: 	A RACH round can be defined as a time period over which devices can randomly select resources for its UL transmissions based on an initial Msg0 with new access parameters, e.g. randomly (re)select a RACH occasion upon new Q value.

In option 1, if a device which detects contention resolution failure or access failure, can re-access in the same RACH round, the reader needs to add new RACH occasion to these failure device(s) or otherwise these failure device(s) will join to the next normal RACH occasion to increase the probability of collision. Addtionnal new RACH occasion in the current round may come with new requirements for both device side and reader side.

In option 2, the current RACH round can be maintained and the probability of collision is not increased. It is easier for both reader and device. Furthermore, the RACH round length is adaptive. One round can be terminated earlier by the reader, e.g. upon detection of too high/low collision. A new round, e.g. with more/less RACH occasions, is initiated.

- A device which detects contention-failure or access failure, re-accesses in the next RACH round.

- The RACH round length is adaptive. One round can be terminated earlier by the reader, e.g. upon detection of too high collision/blank. A new round, e.g. with more/less RACH occasions, is initiated.

## 2.4 Paging/Msg0 and RACH occasion

In the above sub-section, the definition and relationship of RACH occasion and RACH round are mentioned briefly. In this section, we give the whole framework of A-IoT RACH as the following Figure 1.

Figure 1 The framework of A-IoT RACH

The initial triggering of each A-IoT interface procedure, e.g. service request for inventory or command, comes from CN to reader. The first message in A-IoT interface may be paging from reader. One feasible way is that the paging message piggybacked with RACH resources and parameters in MAC layer can be the Msg0 to trigger RACH and as starting point of RACH occasion. This combination method is more efficient and delay-friendly than separate Paging message and Msg0 with RACH configuration. Furthermore, the first Msg0 with initial paging message can be called a Master Msg0 since it acts as the starting point of the first RACH round.

All devices that match the paging condition will randomly select a RACH occasion to perform RACH procedure, e.g. select a random number based on Q value (in the interval [0, 2Q-1]). For each RACH occasion, a success access procedure, collision/failure or blank may occur. The reader will send Secondary Msg0 to indicate next RACH occasion starting point in the same RACH round, which may omit the initial paging message and RACH configuration to save signalling overhead. Upon a Secondary Msg0, device can not change its initial selection of RACH occasion, which can greatly reduce collision.

When the reader detects too high collision or blank, it can terminate current RACH round earlier via a new Master Msg0 instead of planned Secondary Msg0 to initiate a new RACH round with more/less RACH occasions. After the previous round normally ends, the reader can also initiate a new RACH round with new RACH configuration if there may be some failure devices, moving over or energy restored new devices. Hence, the Master Msg0 other than the initial one can also carry the initial paging message for the new devices. All rest devices that match the paging condition and are not accessed successfully will randomly (re)select a RACH occasion to perform RACH procedure, e.g. select a random number based on new Q value (in interval [0, 2Q-1]).

- The paging message piggybacked with RACH resources and parameters can be as the Master Msg0 to initiate the first RACH round.

- There should be a Secondary Msg0 to indicate next RACH occasion starting point in the same RACH round, which may omit the initial paging message and RACH configuration.

- There should be a Master Msg0 to indicate next RACH round starting point, which may carry the initial paging message and/or new RACH configuration.

## 2.5 Multiple access for RACH

In the last RAN1 meeting, RAN1 had the following agreements about RA [2]:

Agreement

Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS.

Agreement

Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

Agreement

Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

Agreement

Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

Hence, TDM and FDM based Msg1 transmission are possible, which means in one RACH occasion there may be multiple devices to send Msg1 in different time-domain or frequency-domain resources. Later, based on further progress of TDM and FDM study, there will be new changes to our A-IoT RA model.

Observation 3:	 RAN1 agreed to study time-domain and frequency-domain multiple access of D2R transmissions and FFS for code-domain.

From RAN2 perspective, it is benefitial if both TDM and FDM can apply to A-IoT Msg1 or MsgA, which will greatly reduce the access delay and improve the QoS experience.

- Both TDM and FDM can apply to A-IoT Msg1 and details up to RAN1 progress.

## 2.6 Power ramping or repetition for RACH

Legacy Uu RACH procedure also supports power ramping for RACH preambles repetition. If UE fails to receive a Msg2 in RAR window, UE will increase TX power and transmits a RACH preamble again. For A-IoT devices, the feasible of power ramping is questionable since the limited energy may not be enough to support this feature except that RAN1 can provide evaluation results to confirm that.  From RAN2 point of view, re-access of failure device should be better to start in the next RACH round. Whether to support power ramping for Msg1 transmission does not affect the RAN2 RACH scheme design.

- Whether to support power ramping for Msg1 transmission is up to RAN1 conclusion and does not affect the RAN2 RACH scheme design.

# Conclusion

Based on the discussion, we have the following proposals and observation:

Observation 1: 	Whether there is a certain number and order of devices for a paging procedure is one of the key factors in reader’s determination of CBRA or CFRA, which is informed from CN to reader.

Observation 2: 	A RACH round can be defined as a time period over which devices can randomly select resources for its UL transmissions based on an initial Msg0 with new access parameters, e.g. randomly (re)select a RACH occasion upon new Q value.

Observation 3:	 RAN1 agreed to study time-domain and frequency-domain multiple access of D2R transmissions and FFS for code-domain.

And we propose:

- CBRA or CFRA should be indicated by reader in Msg0/paging to A-IoT device(s), e.g. providing contention-free access resources.

- 2-step RACH or 4-step RACH is indicated by reader in Msg0/paging to A-IoT device(s), e.g. providing different Msg1 resources size or explicit indicator.

- RAN2 to discuss whether device(s) autonomous fallbacks from 2-step RACH to 4-step RACH is needed.

- For 2-step RACH, device also carries a random ID in MsgA and the reader may echo random ID from MsgA to device in MsgB like that in 4-step RACH for contention resolution.

- A device can determine contention resolution failure immediately if next Msg2/MsgB with other random ID than itself or next Msg0 is received (without further TDM or FDM solution).

- For 4-step RACH, after the device sends the Msg3, it can consider the access success and no re-access is needed any more.

- A device which detects contention-failure or access failure, re-accesses in the next RACH round.

- The RACH round length is adaptive. One round can be terminated earlier by the reader, e.g. upon detection of too high collision/blank. A new round, e.g. with more/less RACH occasions, is initiated.

- The paging message piggybacked with RACH resources and parameters can be as the Master Msg0 to initiate the first RACH round.

- There should be a Secondary Msg0 to indicate next RACH occasion starting point in the same RACH round, which may omit the initial paging message and RACH configuration.

- There should be a Master Msg0 to indicate next RACH round starting point, which may carry the initial paging message and/or new RACH configuration.

- Both TDM and FDM can apply to A-IoT Msg1 and details up to RAN1 progress.

- Whether to support power ramping for Msg1 transmission is up to RAN1 conclusion and does not affect the RAN2 RACH scheme design.

# References

- RAN2#126, Chair’s meeting report;

- RAN1#116b，RAN1#117, Chair’s meeting report;