3GPP TSG-RAN WG2 Meeting #127		R2-2406393

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Source:	vivo

Title:	Discussion on Topology 2 related aspects

Agenda Item:	8.2.5

Document for:	Discussion

# Introduction

Topology 2 with UE as intermediate node under NW control is in the scope of the ambient IoT SID [1]. In previous meetings we achieved below topology 2 related agreements [2]:

RAN2#125bis Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

In this contribution, we will discuss topology 2 related aspects, listed as follows:

- Overall AS procedures

- UE Reader selection

- AIoT radio resource allocation

- AIoT data transport

- QoS handling

- Other functionalities

# Discussion

## 2.1 Overall AS procedures in topology 2

Based on the basic AS procedures between AIoT device and reader, as Figure 1 shows, we may consider the potential impact on the interface between UE Reader and the network (BS/CN):

- Before UE reader initiates the AIoT procedure towards the AIoT device, a UE should be authorized to act the role as UE Reader and selected by the network. The UE reader is paged for AIoT service when the UE reader is in RRC IDLE or INACTIVE state. AIoT radio resources are allocated either before Uu paging or after Uu paging, see details in Section 2.3.

- UE Reader transports the AIoT data between the network and the AIoT device.

- Overall AS procedures in topology 2

- RAN2 to consider additional AS procedures in topology 2 on top of the overall AS procedures in topology 1 (see in Figure 1), which include at least the followings:

            - UE Reader selection

          - AIoT radio resource allocation

          - UE Paging

          - AIoT data transport

## 2.2 UE Reader Selection and Uu Paging

In Topology 2, UE will be paged if IDLE/INACTIVE UE is selected as UE reader. RAN3#124 meeting raised an open issue “In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection”. In our understanding, how the network selects the UE reader is up to SA2/RAN3 discussion. However, we may need to consider how to page the selected UE reader if the UE reader is in IDLE or INACTIVE state, this question is related to the result of UE reader selection, which may be characterized in two cases as:

- Case 1: exact UE reader(s) are selected by AIoT CN and/or AIoT enabled gNB

For example, AIoT CN selects exact UE reader(s) and provides one or multiple UE reader ID(s) to gNB to further initiate the paging, the Uu Paging message can include the exact UE reader ID(s).

- Case 2: a range of UE readers are selected but not specific

For example, if the network wants to inventory all AIoT devices, it may need help from all the UE readers within the range of cells or tracking areas.  In this case, a range of UE readers are selected but without any specific UE reader ID(s). We need to consider how to trigger all UE readers to perform AIoT service efficiently, e.g., as the Uu Paging message for AIoT service may not contain exact UE ID(s), then all paged UE reader responds the Uu paging for AIoT service.

Therefore, we propose to study the Uu Paging for AIoT service considering the selected UE reader(s) is/are explicitly or ambiguously indicated.

- RAN2 to study the Uu Paging for AIoT service considering the UE reader(s) is/are selected explicitly or ambiguously.

## 2.3 AIoT radio resource allocation for UE reader

The UE reader should be allocated with AIoT radio resource before it initiates AIoT radio procedure. Uu resource should be configured as well for AIoT data transport. For UE in RRC_CONNECTED state, it is possible to provide AIoT radio resource and Uu resource to UE reader via reconfiguration procedure. For UE in IDLE or INACTIVE state, it’s an open issue about when to perform AIoT radio resource allocation.

There are some basic options regarding the AIoT radio resource allocation for UE reader in IDLE or INACTIVE state:

- Option 1: AIoT radio resource allocation before Uu paging.

In this option, gNB allocates AIoT radio resources by default or upon receiving the AIoT service request from CN. The UE reader may initiate the AIoT radio procedure directly after receiving the Uu paging for AIoT service. In this way, the UE reader can perform AIoT procedure, e.g., inventory procedure in IDLE/INACTIVE state. Once AIoT data is ready, the UE reader enters RRC_CONNECTED state and is configured with Uu resource for AIoT data transport.

The AIoT radio resource could be pre-configured by gNB, e.g., via system information.

- Option 2: AIoT radio resource allocation after Uu paging.

In this option, gNB allocates AIoT radio resources after receiving the response from UE, or say, UE reader requests AIoT radio resources towards gNB if it decides to perform AIoT radio procedure after receiving the Uu paging for AIoT service. In this way, AIoT radio resource and Uu resource can be provided at the same time after UE enters RRC CONNECTED state. Option 2 has better resource utilization; however, the UE reader is restricted to performing AIoT radio procedure in RRC CONNECTED state.

Hence, we propose to study the above options for AIoT radio resource allocation for UE reader.

- RAN2 to study the initiation and signaling of AIoT radio resource allocation:

            - Option 1: AIoT radio resource allocation before Uu paging, e.g., pre-configured via system information.

            - Option 2: AIoT radio resource allocation after Uu paging, e.g., configured via RRC Reconfiguration

## 2.4 AIoT data Transport

Another key issue in topology 2 is how UE reader transports AIoT data towards gNB/CN. The AIoT data includes AIoT messages and service data for inventory and Command etc. in RAN3, it is still FFS whether control plane transport or user plane transport is used for T2.  We may discuss below potential solutions for this issue.

Option 1 (NAS-based solution). AIoT data is transported via UE reader NAS container. CN handles the received AIoT data from UE reader NAS. In this way, the data of all AIoT devices are transferred through the UE NAS NG without distinction. And, gNB needs not to understand the details of AIoT data. CN has different handling for AIoT data transport in topology 1 and topology 2, this is acceptable because CN has the AIoT service and security related information etc.

Option 2 (RRC-based solution). AIoT data is transported via UE reader RRC message. This is a kind of control plane transport, i.e. SRB is used to transport AIoT data. In this way, it needs gNB to decode RRC message and further transport AIoT data to AIoT CN. From gNB perspective, gNB has different handling for AIoT data transport in topology 1 and topology 2. It seems not advisable to expose too much AIoT data details to gNB.

Option 3 (Dedicated RLC bearer solution or user-plane based solution). AIoT data is transported via dedicated RLC bearer or user plane signaling. gNB can configure a special DRB to UE reader for AIoT data transport. Furthermore, it needs to introduce AIoT data routing, tunnel establishment, etc, to transfer the AIoT data to AIoT CN finally. It brings more complex handling in the network side than option 1.

Comparing the above options, option 1(NAS-based solution) is simple and has less impact on RAN. Therefore, we propose RAN2 to assume AIoT data is transported over UE reader NAS container.

- RAN2 assumes AIoT data transport over UE reader NAS container.

## 2.5 QoS handling

According to SA1 TS 22.369[3], the max allowed end-to-end latency requirement has been defined for performance service requirements of Ambient IoT, details see in Clause 6. Generally, the end-to-end latency requirement needs to be met no matter which topology is deployed. Moreover, it is assumed that there is a radio latency requirement corresponding to the max allowed end-to-end latency requirement, and RAN2 can further study how to guarantee the radio latency requirement from AS perspective. Considering Topology 2 includes the AIoT radio interface and Uu radio interface, the end-to-end latency requirement needs to be split into Uu radio latency and AIoT radio latency requirements. Some detailed issues need further investigation, e.g.:

- Which entity is responsible for splitting the E2E latency requirement, e.g., AIoT CN, gNB, or UE Reader.

- How to apply the AIoT radio latency requirement in the subsequent inventory/command procedures e.g., in terms of resource allocation/scheduling, procedure success/failure, etc.

Hence, we suggest to study how to guarantee the end-to-end latency requirement for topology 2, and propose that:

- RAN2 to study how to guarantee the end-to-end radio latency requirement, which corresponds to the max allowed end-to-end latency requirement as specified in SA1 TS 22.369.

- QoS split functionality is required to define Uu radio latency and AIoT radio latency requirements for AIoT data transmission performance.

## 2.6 Other Functionalities for UE Reader

In topology 2, UE reader reuses the Uu interface to transport AIoT data. According to the analysis of the above sections, we assume that:

- there is no or less impact on the following procedures:

  - RRC connection management (establishment/resume/re-establishment)

  - Access control.

  - Tracking/ RAN Notification Area Update.

- Potential impacts to the following procedures are foreseen:

  - System Information/RRC Reconfiguration. E.g., AIoT radio resources are pre-configured via System Information, or configured via RRC Reconfiguration.

- RAN2 to assume No/less impact to the following procedures:

            - RRC connection management (establishment/resume/re-establishment)

            - Access control

            - Tracking/ RAN Notification Area Update

A UE reader may meet Uu failure while performing AIoT procedure. How to handle the ongoing AIoT procedure in case of Uu failure scenario e.g. radio link failure, should be considered. In general, we think it is better to keep the ongoing AIoT procedure even when the Uu link is temporally suspended, then:

- AIoT specific configuration is maintained to continue AIoT procedure during Uu recovery. UE reader can continue using the allocated AIoT radio resource to perform AIoT procedures.

- AIoT data transport is pending during Uu recovery and only continues after successful Uu recovery. AIoT device data is kept in UE reader, and can be transported later.

- AIoT specific configuration is maintained to continue AIoT procedure during Uu recovery. AIoT data transport is pending during Uu recovery.

# Conclusion

Based on the discussion, we have the following proposals:

Proposal 1.	RAN2 to consider additional AS procedures in topology 2 on top of the overall AS procedures in topology 1 (see in Figure 1), which include at least the followings:

            - UE Reader selection

          - AIoT radio resource allocation

          - UE Paging

          - AIoT data transport

Proposal 2.	RAN2 to study the Uu Paging for AIoT service considering the UE reader(s) is/are selected explicitly or ambiguously.

Proposal 3.	RAN2 to study the initiation and signaling of AIoT radio resource allocation:

            - Option 1: AIoT radio resource allocation before Uu paging, e.g., pre-configured via system information.

            - Option 2: AIoT radio resource allocation after Uu paging, e.g., configured via RRC Reconfiguration

Proposal 4.	RAN2 assumes AIoT data transport over UE reader NAS container.

Proposal 5.	RAN2 to study how to guarantee the end-to-end radio latency requirement, which corresponds to the max allowed end-to-end latency requirement as specified in SA1 TS 22.369.

Proposal 6.	QoS split functionality is required to define Uu radio latency and AIoT radio latency requirements for AIoT data transmission performance.

Proposal 7.	RAN2 to assume No/less impact to the following procedures:

            - RRC connection management (establishment/resume/re-establishment)

            - Access control

            - Tracking/ RAN Notification Area Update

Proposal 8.	AIoT specific configuration is maintained to continue AIoT procedure during Uu recovery. AIoT data transport is pending during Uu recovery.

# References

- RP-240826 Study on solutions for Ambient IoT (Internet of Things) in NR.

- RAN2#125bis Chair Notes.

- 3GPP TS 22.369, Service requirements for ambient power-enabled IoT; Stage 1 (Release 19).