3GPP TSG-RAN WG2 Meeting #127	R2-2406394

Maastricht, Netherlands, 19 – 23 August 2024	R2-2404287

Agenda item:	7.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	PDCP SN gap report and updating RX_NEXT

WID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

Based on the predecessor of this contribution, RAN2#126 discussed how the state variable RX_NEXT should be updated after the introduction of the PDCP SN gap report. This contribution discusses the topic further.

The main changes compared to our earlier contribution [R2-2404287]:

-	In the problem example: changed Count values to PDCP SNs to emphasize that the problem is not limited to the early stages of a DRB and removed the unnecessary restriction that the high-importance SDUs form a single PDU set;

-	Added new section 2.1 with further explanation on why the problem can happen;

-	Corrected the wording of Proposal 1.

# 2	Discussion

In the current specification text for receiving PDCP SN gap report, RX_NEXT is updated as follows:

- else if RX_NEXT <= COUNT value associated with the last PDCP SDU indicated in the PDCP SN gap report: - update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU plus 1.

When pdu-SetDiscard is configured and PSI-based SDU discard is activated, the above way to update RX_NEXT makes the following sequence of events possible.

1.	A transmitting PDCP entity receives from upper layers high- and a low-importance PDU sets such that their PDCP SDUs are interlaced: e.g.

-	the SDUs of the high-importance set(s) are associated with the even-numbered PDCP SNs 10, 12, …, 18; and

-	the SDUs of the low-importance set are assigned the odd-numbered PDCP SNs 11, 13, …, 19.

2.	discardTimerForLowImportance expires for SDU#11 before it is transmitted, leading to discard all the SDUs 11, 13, …, 19. (The high-importance SDUs are not discarded given that the longer discardTimer applies.) A PDCP SN gap report is triggered and sent, indicating these SDUs as discarded. The gap report is submitted to lower layers immediately as in the current specification, and transmitted in a MAC PDU e.g. before SDU#10:

- In both uplink and downlink, this can happen because not even SDU#10 has been submitted to RLC before the gap report,

- Alternatively, in the downlink the gap report may be submitted to RLC even after all the SDUs #10..19, but at RLC, as special treatment of the SN gap report,  it is placed at head of queue, i.e. before SDU#10.

3.	At the receiving entity, before receiving the gap report, using the SN(COUNT) notation of the specification, SN(RX_DELIV) = SN(RX_NEXT) = 10 (i.e. SDU#10 is expected next, no missing SDUs observed), and after receiving the gap report, by the current specification, SN(RX_DELIV) = 10 and SN(RX_NEXT) = 20, leading to starting t-Reordering and setting RX_REORD = RX_NEXT.

4.	t-Reordering expires, meaning that any unreceived SDU with PDCP SN less than SN(RX_REORD) = 20 will never be delivered to upper layer even if still received after this point.

In terms of the previous standard (without the gap report), starting t-Reordering with SN(RX_REORD) = 20 would have required that at least the initial transmission of all SDUs up to #19 has taken place, so leaving behind all unreceived SDUs below RX_REORD has been justified when t-Reordering expires.

With the current specification, this is no longer necessarily the case: in the above example, the gap report may update SN(RX_NEXT) to 20 at a time when the highest-numbered SDU transmitted may still be, say, 12. It follows that the current text proposal can lead the PDCP receiver to abandon unreceived SDUs prematurely – and to make things worse, those SDUs are likely to be of high importance.

Observation 1: If RX_NEXT is updated past all SDUs indicated as discarded, the receiving PDCP entity may prematurely update RX_DELIV past unreceived high-importance SDUs when pdu-SetDiscard is configured and PSI-based SDU discard is activated.

To prevent this problem, SDUs considered as discarded should lead to incrementing RX_NEXT (upon reception of SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded (i.e. indicated as discarded in some received SN gap report). This way, in the above problem example the status SN(RX_DELIV) = SN(RX_NEXT) = 10 remains even after receiving the gap report, and the future behaviour of these variables and t-Reordering is effectively as if the high-importance SDUs 10, 12, …, 18 had consecutive COUNT values, which is the intended operation.

Proposal 1: SDUs considered as discarded (i.e. indicated as discarded in some received SN gap report) lead to incrementing RX_NEXT (upon reception of a SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded.

We provide a corresponding text proposal in Annex.

Proposal 2: RAN2 agree the text proposal in Annex for Proposal 1.

## 2.1	Why the problem can happen

In RAN2#126 there were arguments that, in the above example, the gap report cannot be transmitted in a MAC PDU before any of the high-importance SDUs, because by the time of the reporting, all those SDUs will be at RLC and cannot be by-passed by the gap report.

On prioritizing the gap report at RLC, as discussed in the above sub-bullets of step 2, the network is free to place the gap report at head of queue at RLC, even though no such behaviour is allowed to the UE. In RLC UM, RLC SDUs cannot even be assigned RLC SNs while queued, meaning that there is no SN re-assignment caused by prioritizing the gap report.

Observation 2: In the downlink, nothing prohibits the network from placing a PDCP SN gap report at the head of RLC queue, past PDCP Data PDUs submitted to RLC before the report. In RLC UM, there is even no SN re-assignment caused by prioritizing the gap report.

On submission of the PDCP Data PDUs to RLC prior to the gap report, the following factors should be considered:

-	The case in question is PSI-based discarding, meaning there is uplink congestion and thus possibly already a considerable queue of PDCP PDUs at RLC;

-	On a split bearer, neither one of the RLC queues can be allowed to grow without limit, because otherwise PDCP PDUs could get stuck in an RLC whose throughput suddenly decreases;

-	On any bearer, especially in the uplink the PDCP SN gap report itself is a good reason not to allow the RLC queue to grow without limit (to reduce the transmission delay of a possible report);

-	discardTimerForLowImportance can be configured with a very short duration. As part of PDCP data volume for DSR, the current specification already acknowledges that there can be “delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed”.

Observation 3.1: The PDCP SN gap report is not necessarily generated after all the data containing the indicated gaps is submitted to RLC: the case in question is PSI-based discarding meaning uplink congestion, and the RLC queue(s) should not be allowed to grow without limit to prevent possible issues from delays, e.g. on split bearers, and delay to the gap report itself.

Observation 3.2: The PDCP SN gap report is not necessarily generated after all the data containing the indicated gaps is submitted to RLC: discardTimerForLowImportance can be configured with a very short duration, and the current specification already acknowledges that there can be “delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed”.

# 3	Conclusion

This document has made the following observations:

Observation 1: If RX_NEXT is updated past all SDUs indicated as discarded, the receiving PDCP entity may prematurely update RX_DELIV past unreceived high-importance SDUs when pdu-SetDiscard is configured and PSI-based SDU discard is activated.

Observation 2: In the downlink, nothing prohibits the network from placing a PDCP SN gap report at the head of RLC queue, past PDCP Data PDUs submitted to RLC before the report. In RLC UM, there is even no SN re-assignment caused by prioritizing the gap report.

Observation 3.1: The PDCP SN gap report is not necessarily generated after all the data containing the indicated gaps is submitted to RLC: the case in question is PSI-based discarding meaning uplink congestion, and the RLC queue should not be allowed to grow without limit to prevent possible issues from delays, e.g. on split bearers, and delay to the gap report itself.

Observation 3.2: The PDCP SN gap report is not necessarily generated after all the data containing the indicated gaps is submitted to RLC: discardTimerForLowImportance can be configured with a very short duration, and the current specification already acknowledges that there can be “delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed”.

And proposed the following:

Proposal 1: SDUs considered as discarded (i.e. indicated as discarded in some received SN gap report) lead to incrementing RX_NEXT (upon reception of a SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded.

Proposal 2: RAN2 agree the text proposal in Annex for Proposal 1.

# Annex: Text proposal to TS 38.323

First Modified Subclause

### 5.2.2	Receive operation

#### 5.2.2.1	Actions when a PDCP Data PDU is received from lower layers

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded as specified in clause 5.16.2, with COUNT value > RCVD_COUNT.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

Next Modified Subclause

### 5.16.2	Receive operation

At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall:

-	consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded;

-	if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs:

-	if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs:

-	update RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded, with COUNT value > RX_NEXT;

-	if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s):

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.