3GPP TSG-RAN WG2 Meeting #127	R2-2406397

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	XR SA2 Overview

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Study Item

As a reminder, the Rel-19 SA2 Study on Extended Reality and Media service (XRM) Phase 2 is described in SP-231671 and contains the following 4 objectives:

1.	Enhancement for PDU Set based QoS handling

a.	Study whether and how to enhance PDU Set related (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC) and PDU Set information (including Control Plane and/or User plane information provided by the AF/AS) and the corresponding PDU Set QoS handling enhancement.

NOTE:	This will require close coordination between SA4 and SA2.

b.	Support QoS control and PDU Set identification for XR stream with e2e encryption (e.g. fully encrypted header, partially encrypted header). This is applicable for PDUs received at N6 for DL.

c.	Study whether and how to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

2.	QoS handling enhancement for XRM services

a.	Study whether and what enhancements are needed for traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

b.	Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

NOTE:	This will require close coordination between SA4 and SA2.

c.	Study whether and how to identify  traffic flows and study whether and how QoS handling enhancement may be needed for the UE with the tethered devices for the uplink traffic (for example, traffic from tethered devices mapped to different QoS Flows enabling QoS differentiation such as PDU Set based QoS flows for XR traffic for device 1 vs PDU based QoS flows for eMBB traffic for device 2).

NOTE:	The interface between 3GPP UE and tethered devices behind the UE is outside of scope.

3.	Further enhancement to support XR based on non-3GPP access

4.	Network exposure: Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

NOTE:	Alignment and coordination with RAN work will be needed for the study.

The corresponding TR is 23.700-70. The latest version of this TR is 1.0.0, which was sent for information to the latest SA meeting (SA#104). When compared to the last RAN2 meeting, some key issues were concluded and some interim agreements were reached. This is summarised in the table below:

| KI | Description | Conclusions | RAN Impacts |
| --- | --- | --- | --- |
| 1 | Support of PDU set based QoS handling enhancement: study PDU set based QoS handling enhancements considering both control plane and user plane perspectives. In particular, this KI addresses whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced | Interim | Expected |
| 2 | Support PDU Set information identification for end-to-end encrypted XRM traffic: study the enhancement of PDU Set information Identification for encrypted XRM in 5G networks | None | None Expected |
| 3 | Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network: study whether, how, and what PDU Set QoS information can be used for DSCP marking on the outer header of downlink packets of the PDU Set over N3/N9 in the transport network | Yes | No |
| 4 | Traffic detection and QoS flow mapping for multiplexed data flows: study whether, how, and what PDU Set QoS information can be used for DSCP marking on the outer header of downlink packets of the PDU Set over N3/N9 in the transport network | Interim | None Expected |
| 5 | QoS Handling when Traffic Characteristics Change Dynamically: study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN | Yes | Yes |
| 6 | L4S for non-3GPP access networks and intermediate 5GS nodes: extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes | Yes | No |
| 7 | Support for PDU Set in non-3GPP access: support PDU set based QoS Handling to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes | Yes | No |
| 8 | Enhancement for UE with the tethered devices: study whether and how to identify traffic flows from the tethered devices behind the UE from the uplink traffic (e.g. traffic from different tethered devices may be mapped to different QoS Flows to enable QoS differentiation) | Yes (nothing) | No |
| 9 | Enhancement for XR related network information exposure: to study how to enhance network exposure mechanism to better support the network information/capability exposed to the application layer | Interim | Expected |

For KI#5, the conclusions were captured as followed [23.700-70]:

The following aspects are concluded as principles for the normative work: - Support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling. 1. The AF may indicate to the NEF/PCF to mark the Burst size. 2. Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size. 3. Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header. NOTE: This conclusion assumes that SA4 will specify Burst size indication in the header extension (i.e. RTP HE). 4. The NG-RAN may use the received Burst size to assist radio resource management.

As a result, the study was declared 90% complete [S2-2407294] and one more TU was requested at the next SA2 meeting (in August) [SP-240914]. In other words, the SA2 study should be 100% complete before the next RAN meeting takes place.

# 3	Work Item

Based on the partial outcome of the above study, SA#104 approved a new Rel-19 work item on Extended Reality and Media service (XRM) Phase 2 [SP-240990]. It contains the following objectives:

-	KI#3 Enhancement for PDU Set based DSCP marking:

-	Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

-	KI#5 Dynamic traffic characteristics update:

-	Support dynamic change (via user plane) in traffic characteristics (i.e., burst size) provided by the application in the DN.

-	KI#6 Further enhancement to support XR based on non-3GPP access:

-	Support L4S in untrusted/trusted access (N3IWF, TNGF).

-	Support L4S in wireline access (W-AGF, 5G-RG).

-	KI#7 Extend PDU Set QoS Control mechanisms to non-3GPP access networks.

-	Support PDU Set QoS in untrusted/trusted access (N3IWF, TNGF).

-	Support PDU Set QoS in wireline access (W-AGF, 5G-RG).

-	Provide Protocol Description to the UE for trusted/untrusted access.

-	Enhancements for DSCP marking by N3IWF/TNGF based on PDU Set Importance.

# 4	Conclusion

The Rel-19 SA2 Study on Extended Reality and Media service (XRM) Phase 2 is 90% complete. A work item has been approved in SA2 [SP-240990], which contains one objective impacting RAN (support dynamic change in traffic characteristics). Two of the remaining key issues to be concluded by SA2 in August may also impact RAN.