3GPP TSG-RAN WG2 Meeting #127	R2-2406398

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	RAN3 Overview

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Overview

The following topics have been discussed in RAN3:

-	Support of non-homogeneous NR-DC: RAN3 agreed non-homogenous deployment is possible for XR (as for other features in DC), but could not reach agreement on whether to turn the following WA into an agreement:

WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

-	ECN Marking and Congestion Information exposure: for the ECN Marking and Congestion Information Reporting Status report from SN, RAN3 agreed that it should be done per DRB, but with QoS Flow Info for SN-terminated SCG bearer, and per DRB otherwise.

-	PSI based discard: the need for coordination between MN and SN was discussed and an LS was sent to RAN2 in R3-243957.

-	Burst Arrival Time Reporting: RAN3 discussed whether the SN can receive the BAT from the MN, or directly from the UE and agreed that similarly to handover, SN uses the BAT by considering the SFN offset of MN.

-	End of Data Burst Indication: RAN3 discussed how to handle the End of Data Burst Indication which is only sent to the PDCP-hosting gNB, and agreed that the PDCP-hosting gNB should also forward the End of Data Burst indication to the corresponding gNB when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.

The corresponding endorsed CRs for TS 37.340 and TS 38.423 can be found in R3-243973 and R3-243974 respectively. A summary of the off-line discussions can also be found in R3-243820.

# 2	Conclusion

For the Rel-19 work on XR, RAN3 made some good progress and from RAN2 viewpoint, the point worth noting is the one related to the question RAN3 is asking RAN2 in their LS on PSI based discard R3-243957.