3GPP TSG-RAN WG2 Meeting #127	R2-2406399

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.1.1

Source:	Nokia

Title:	Periodicity and Time to Next Burst

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2 is getting one LS from SA2 on Periodicity and Time to Next Burst (TTNB) [S2-2407351]:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

The purpose of this contribution is to explore possible answers to these two questions.

# 2	Background

The dynamic provision of periodicity and TTNB in GTP-U is related to key issue #5 for QoS Handling when Traffic Characteristics Change Dynamically [23.700-70]:

Certain Rel-18 XRM features work best when the traffic pattern is mostly static. However, the traffic pattern of an XR session change (i.e. it can be dynamic) based on the usage/applications. One example case where the traffic pattern can change dynamically is the size of media frames vary due to scene changes that may occur at times such as initial start-up or due to user drags on a progress bar. Another example is related to file download (e.g. AI model update) before the session can continue. This key issue proposes to study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN. The key issue includes the following aspects: - Identify what traffic characteristics are dynamically changed. - Whether and how the 5G network can be enhanced to know about dynamic changes in traffic characteristics of GBR and non-GBR flows: - What information from the Application in the DN, if any, is needed by the 5G network to be able to know the dynamic changes in traffic characteristics and how this information is provided by the Application in the DN. - At what granularity does the 5GS need to know of changes in traffic characteristics (e.g. QoS Flow granularity). - What handling may be needed when the 5G network knows about changes in the traffic characteristics. NOTE: This Key Issue may require coordination with RAN WGs and/or SA WG4.

# 3	Discussion

The EoDB indication signals an opportunity to sleep until the beginning of the next period as determined from the periodicity. To reflect dynamic changes in traffic characteristics (e.g. variations in capture and/or transmission pace), the idle time until the next burst would also need to be known, and that is the purpose of TTNB. With TTNB, the duration of the opportunity to sleep can always be optimal.

Observation 1:  TTNB helps optimising power consumption when the traffic characteristics change dynamically.

With respect to periodicity, our understanding is that the periodicity is transmitted once and remains valid until a new value is signalled. It is bound to isochronous media capture and transmission and does not capture the flexible transmission nature of video streams, nor rate adaptation mechanisms. If periodicity was updated at every burst to capture such flexibility, it would then become equivalent to TTNB. Thus, the provision of periodicity in addition to TTNB can only be justified if GTP-U overhead is a concern and the changes in traffic characteristics are not very dynamic. In all other scenarios, TTNB alone would be enough.

Observation 2: the usefulness of periodicity in addition to TTNB in GTP-U is limited.

We therefore suggest the following:

Proposal 1: only TTNB is carried over GTP-U.

# 4	Conclusion

This document has made the following observations:

Observation 1:  TTNB helps optimising power consumption when the traffic characteristics change dynamically .

Observation 2: the usefulness of periodicity in addition to TTNB in GTP-U is limited.

And proposed the following:

Proposal 1: only TTNB is carried over GTP-U.