3GPP TSG-RAN WG2 Meeting #127	R2-2406400

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.5

Source:	Nokia

Title:	RLC Enhancements

WID/SID:	NR_XR_Ph3-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

Regarding the RLC enhancements for the Rel-19 work on XR, the agreements of the last meeting were:

Avoiding unnecessary retransmissions - For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned Autonomous retransmissions - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

This contribution discusses further details of these two aspects, namely avoiding unnecessary retransmissions and ensuring timely retransmissions.

# 2	Avoiding unnecessary retransmissions

## 2.1	Window handling

The receiving window in RLC AM is defined, maintained, and applied as follows.

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows: - a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size; - a SN falls outside of the receiving window otherwise. … When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall: - if x falls outside of the receiving window; or - if byte segment numbers y to z of the RLC SDU with SN = x have been received before: - discard the received AMD PDU. … a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

The transmitting window is defined and maintained as follows.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows: - a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size; - a SN falls outside of the transmitting window otherwise. The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window. … a) TX_Next_Ack – Acknowledgement state variable This state variable holds the value of the SN of the next RLC SDU for which a positive acknowledgment is to be received in-sequence, and it serves as the lower edge of the transmitting window. It is initially set to 0, and is updated whenever the AM RLC entity receives a positive acknowledgment for an RLC SDU with SN = TX_Next_Ack.

Conversely, the transmitter is free to submit to lower layer any AMD PDU whose SN falls within the transmitting window, e.g. one with SN = TX_Next_Ack + AM_Window_Size – 1.

These two windows are maintained hand in hand:

1.	The receiving window is progressed (whenever possible);

2.	The progressed receiving-window status is informed to the transmitter using RLC ACKs;

3.	The transmitting window is updated to match the receiving window progressed in step 1.

This is to make sure that the transmitting window never spans beyond the receiving window: if it did, the transmitter would be allowed to transmit PDUs that end up discarded as being outside the receiving window.

Observation 1: in RLC AM, updating the transmitting window always requires knowledge of the receiving window having been updated accordingly, to prevent transmission of AMD PDUs that end up discarded as being out of the receiving window.

This same principle needs to be adhered to also in the avoidance of unnecessary retransmissions. In the TX-initiated approach, the transmitter cannot progress its transmitting window simply after sending a discard indication to the receiver. Instead, it needs to wait for some indication of the receiving window having been updated accordingly. RAN2#126 agreed the principle of Observation 1 for the RX-initiated approach, but for some reason not for the TX-initiated approach.

Proposal 1: the RAN2#126 agreement on proper advancing of the window for RX-initiated approach for avoiding unnecessary retransmissions also applies to the TX-initiated approach i.e. “RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter” applies to both RX- and TX-initiated approaches.

Another consequence of this principle to the TX-initiated approach is that the transmitting window cannot be progressed past an SDU discarded by the transmitter before either the SDU or its discard indication is successfully delivered to the receiver. This is why in [R2-2404293] we said about the TX-initiated approach: “the mandatory delivery of a Data PDU is only exchanged for a mandatory delivery of the discard/SN-gap indication”.

Proposal 2: RAN2 acknowledge that in the TX-initiated approach for avoiding unnecessary retransmissions, the mandatory delivery of an SDU is only exchanged for a mandatory delivery of its discard indication.

## 2.2	How to indicate abandoned SDUs to transmitter

Previous contributions have proposed that also RLC SDUs not received and abandoned by the receiver are indicated to the transmitter using RLC ACK. While this is straightforward from RLC point of view, received RLC ACKs are also used locally by PDCP, for instance to determine how high-numbered PDCP SDUs can be transmitted by PDCP:

NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

If a PDCP PDU is RLC ACKed, but the transmitting RLC and PDCP do not know whether the PDU is successfully delivered to the receiving RLC and PDCP, or whether the receiving RLC has abandoned the PDU while the receiving PDCP has not yet progressed its receiving window (defined by its RX_DELIV variable) past that PDU, the RLC ACKs will no longer serve their current purpose at PDCP. As a replacement, regular Status reporting would seem to be needed at PDCP.

Observation 2:	If a received RLC ACK passed to PDCP no longer reliably indicates successful delivery to the peer PDCP entity, regular PDCP status reporting is needed for the transmitting PDCP to know how high-numbered PDCP SDUs it can proceed to transmit.

Proposal 3: For indicating abandoned RLC SDUs from RLC receiver to transmitter, in order that the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, RAN2 select between:

A)	RLC ACK, combined with regular PDCP status reporting to keep the PDCP transmitter reliably informed of successful delivery; or

B)	A new explicit RLC indication separate from ACK, of SDUs abandoned by the receiver.

## 2.3	How Rx side determines that an SDU should be abandoned

This aspect was left FFS in RAN2#126. It seems that this mechanism needs to be no different from the one already used by PDCP for the same purpose: PDCP’s t-Reordering and related state variables could be replicated at RLC AM, even with the same timer duration – although it is probably simplest to have the PDCP and RLC timers independently configurable.

Proposal 4: in the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer like t-Reordering at PDCP or t-Reassembly in RLC UM.

# 3	Timely retransmissions

## 3.1	Autonomous retransmissions

This approach is a part of transmitter-initiated alternatives, where the retransmissions are triggered without explicit knowledge of failed RLC SDUs (but are not triggered in case of timely RLC ACKs). Different triggers have been proposed during RAN2#126 and are listed below.

1.	Using a threshold for remaining time: the solution is based on monitoring the remaining time of a PDU and if it falls below a preconfigured threshold, a retransmission is triggered. Such a solution might need to factor in the shorter PDCP discard timer applied to low importance PDU in case of congestion (so that they do not get prioritized wrongly). The same effect without the PDCP interaction could be achieved with a simple retransmission timer at RLC.

2.	Indication from HARQ regarding several failed retransmissions: while this solution provides an indication of a failed transmission it faces some challenges. Firstly, it requires the knowledge of mapping between TBs and RLC SDUs as a TB can contain several RLC SDUs in which some of them may not belong to XR traffic and not be delay-critical. Secondly, for the UL,  the UE only knows about a failed HARQ transmission once it gets a scheduling grant for a retransmission, at which point an ARQ retransmission would come on top of an upcoming HARQ retransmission. Therefore, the lack of explicit HARQ feedback in UL  results in added delays  for triggering RLC retransmissions which hinders the timeliness on them.

3.	A new TB is scheduled after transmitting all RLC PDUs once: this assumes that once new TB are scheduled after sending an empty BSR, retransmissions of the previous burst are required.

Observation 3: autonomous retransmissions provide the fastest RLC retransmission response among all the alternatives but more studies are required to determine which solution is best suited to XR traffic.

## 3.2	Retransmission based on enhanced status report

This approach is a part of receiver-initiated alternatives that relies on the indicated missing RLC SDUs within the RLC STATUS PDU. Such reporting is normally triggered by expiry of t-Reassembly timer, which starts after detecting an out of order reception of an RLC SDU. One solution that came from some of the companies was reducing the t-Reassembly timer values to values as short as 1ms to speed up the status PDU triggering at the transmitter. However, this can cause sending STATUS PDUs with pre-mature NACKs for some of the SDUs that are still being transmitted (or retransmitted by lower layers). Such premature NACKs can be repeated in several successive STATUS PDUs, each triggering a separate retransmission of the same SDU. The status-prohibit timer can be used to prevent such repetition, but that can easily defeat the purpose of the short t-Reassembly.

Observation 4: Reducing t-Reassembly timer does not properly solve the issue of slow STATUS PDU triggering.

Other trigger enhancements for STATUS PDU were also proposed (e.g. R2-2404332), but they introduce other problems:

-	As mentioned above, very frequent STATUS PDUs are not reliable and will show a lot of premature NACKs and thus cause many more unnecessary RLC retransmissions, worsening the condition of the link load.

-	On the other hand, more frequent STATUS PDUs causes added overhead in the reverse link and will impact the capacity.

Observation 5: Retransmissions based on RLC NACKs is not efficient enough to fit for short PDB of XR services.

## 3.3	Retransmission based on enhanced polling

This approach is a part of transmitter-initiated alternatives, where a STATUS PDU will be triggered in response to the polling from the receiver side. Even if we assume ideal conditions to enhance the polling procedures, we still need to deal with the issues caused by the STATUS reporting as discussed in the previous. For example, regardless of how frequent the polling, the triggering of the STATUS PDU and the SN range that it can report will still be governed by the state variable RX_Highest_Status whose updating is controlled by t-Reassembly. Therefore, in a realistic scenario, where polling will add additional delays, the solutions will not be suitable for XR.

Observation 6: Enhancing polling does not solve the issues associated with the STATUS reporting.

Based on the three observations above, we propose the following:

Proposal 5: for timely retransmissions, RAN2 to deprioritize the solutions based on STATUS reporting and polling enhancements and focus on using autonomous alternatives for timely RLC retransmissions by comparing which of the suggested solutions (or combinations) is best suited to XR services.

# 4	Conclusion

This document has made the following observations:

Observation 1:	In RLC AM, updating the transmitting window always requires knowledge of the receiving window having been updated accordingly, to prevent transmission of AMD PDUs that end up discarded as being out of the receiving window.

Observation 2:	If a received RLC ACK passed to PDCP no longer reliably indicates successful delivery to the peer PDCP entity, regular PDCP status reporting is needed for the transmitting PDCP to know how high-numbered PDCP SDUs it can proceed to transmit.

Observation 3: autonomous retransmissions provide the fastest RLC retransmission response among all the alternatives but more studies are required to determine which solution is best suited to XR traffic.

Observation 4: Reducing t-Reassembly timer does not properly solve the issue of slow STATUS PDU triggering.

Observation 5: Retransmissions based on RLC NACKs is not efficient enough to fit for short PDB of XR services.

Observation 6: Enhancing polling does not solve the issues associated with the STATUS reporting.

And proposed the following:

Proposal 1: the RAN2#126 agreement on proper advancing of the window for RX-initiated approach for avoiding unnecessary retransmissions also applies to the TX-initiated approach i.e. “RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter” applies to both RX- and TX-initiated approaches.

Proposal 2: RAN2 acknowledge that in the TX-initiated approach for avoiding unnecessary retransmissions, the mandatory delivery of an SDU is only exchanged for a mandatory delivery of its discard indication.

Proposal 3: For indicating abandoned RLC SDUs from RLC receiver to transmitter, in order that the transmitting PDCP reliably knows how high-numbered PDCP SDUs it can proceed to transmit, RAN2 select between:

A)	RLC ACK, combined with regular PDCP status reporting to keep the PDCP transmitter reliably informed of successful delivery; or

B)	A new explicit RLC indication separate from ACK, of SDUs abandoned by the receiver.

Proposal 4: in the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer like t-Reordering at PDCP or t-Reassembly in RLC UM.

Proposal 5: for timely retransmissions, RAN2 to deprioritize the solutions based on STATUS reporting and polling enhancements and focus on using autonomous alternatives for timely RLC retransmissions by comparing which of the suggested solutions (or combinations) is best suited to XR services.