3GPP TSG-RAN WG2 Meeting #127	R2-2406401

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	Simulation results for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, RAN2 agreed on the baseline of simulation assumptions and methodology for the RRM measurement prediction.

As to the potential scenarios, cases 2,3,4 in the following table are prioritized.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we will provide initial simulation results on the agreed prioritized scenarios and agreed assumptions.

- Discussion

  - Methodology of simulation

    - Clarification of Case B

Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub-set of measurement instances in the temporal domain of the same cell. The description of case B in the beam management is as follows [1]:

- Case B: based on a periodicity T of the required reference signals for measurements to achieve a certain beam prediction accuracy. An example is shown in Figure 6.3.1-3. - For non-AI baseline (Option 2), every T=X ms reference signals for measurements are needed - For AI, every T=Y ms, reference signals for measurements are needed - In this case, - For Set B = Set A, the RS overhead reduction is 1-X/Y. - For Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is [1-XN/(YM)]. Figure 6.3.1-3: Example for Case B

For Case B, the UE can perform measurements with a larger interval and utilize predictions to replace the measurement at the original time instance. As to the timing relationship of the measurement and prediction, there can be two Options:

- Option 1:. The input are historical measurement values and the output are the values at the subsequent time instances where measurement is skipped. That is, the prediction is always after the measurement and is at future time instance(s).

- Option 2: The input are historical measurement values and the output are the values between the historical measurement time instances where measurement is skipped. That is, the prediction is at historical time instance(s).

Figure 2.1-1 Case B prediction (MRRT=1/2, 3/4)

For example, with a measurement reduction rate of 1/2 and 3/4:

- Reduction rate 1/2:

- Option 1: The RRM measurement prediction at time instance 22 can be inferred from the RRM measurement at time instances {3, 5, ..., 18, 21}.

- Option 2: The RRM measurement prediction at time instance {2,4} can be inferred from the RRM measurement at time instances {1, 3, 5}.

- Reduction rate 3/4:

- Option 1: The RRM measurement prediction at time instances {42, 43, 44} can be inferred from the RRM measurement at time instances {5, 9, ..., 37, 41}.

- Option 2: The RRM measurement prediction at time instance {2,3,4} can be inferred from the RRM measurement at time instances {1, 5}.

For the above two options, Option 2 may have better performance. However, Option 2 is not applicable for some scenarios. For instance, if the prediction is sub-use case 2 (i.e., Cell -> Cell) and is used for measurement event triggering, extra triggering latency will be introduced as the UE needs the next measurement to obtain the prediction at the previous time instance(s). As a consequence, too-late handover may occur.

Proposal 1: For temporal domain case B prediction for measurement reduction, further clarify the timing relationship between the measurement and prediction. The following two options can be considered:

- Option 1: the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

- Option 2: The input is historical measurement values and the output is the values between the historical measurement time instances where measurement is skipped, i.e., the prediction is at historical time instance(s).

Option 1 is selected in the following simulation.

    - Selection of Cells as test set

For mobility management, the UE mainly focuses on neighboring cells with relatively high RSRP, e.g., as candidate cells for HO. To minimize unnecessary inference, the test set should select neighboring cells with high RSRP. Furthermore, inference based on neighboring cells with lower RSRP may affect the final prediction accuracy. The selection of cells as output can be based on the company's implementation and report.

In our simulation, we select the cells with RSRP larger than -110dBm as the validity dataset for inference.

Proposal 2: For the Cell-level measurement prediction, select the neighbor cells with high RSRP as the test set. How to select the cells is up to company’s implementation and report.

  - FR1 to FR1 intra-frequency (temporal domain)

Based on the simulation results in section 2.3, sub-use case 2 has the minimum model complexity and relatively high performance of prediction accuracy, therefore the following simulation on measurement reduction will also adopt sub-use case 2, i.e., cell-level measurement result(s) is predicted based on actual cell-level measurement result(s).

    - MRRT = 1/2

During the simulation process based on the agreed simulation assumptions, one issue arose: how to evaluate the gain of AI-based prediction. The current definition of prediction accuracy is as follows.

- Measurement prediction accuracy for cell-level RRM measurement prediction is defined as the average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

Further discussion is needed on how to evaluate the gain of AI-based prediction. The "sample and hold" mechanism is the most basic non-AI mechanism as a benchmark, where the predicted value equals the previous actual measured value. The simulation results with MRTT=1/2 for non-AI and AI mechanisms are as follows:

Table 2.2-1 Simulation results (MRRT=1/2)

| Speed | Prediction accuracy of Non-AI mechanism (sample and hold) | Prediction accuracy of AI-based mechanism |
| --- | --- | --- |
| 30km/h | 0.17 | 0.07 |
| 60km/h | 0.34 | 0.13 |
| 90km/h | 0.51 | 0.20 |

From the simulation results, it can be observed that compared to the non-AI "sample and hold" approach, the AI-based mechanism can improve prediction accuracy, with the improvement increasing as the speed increases. Other non-AI mechanisms are up to company’s implementation and company can report the mechanism with the simulation results.

Proposal 3: To evaluate the gain of AI-based prediction, introduce a non-AI mechanism as the benchmark, e.g., sample and hold (the predicted value equals the previous actual measured value). Other non-AI mechanisms are up to company’s implementation and report.

    - MRRT = 3/4

The simulation results with MRTT=3/4 for non-AI and AI mechanisms are as follows Table.

Table 2.2-2 Simulation results (MRRT=3/4)

| Speed | Prediction accuracy of Non-AI mechanism (sample and hold) | Prediction accuracy of AI-based mechanism |
| --- | --- | --- |
| 30km/h | 0.35 | 0.13 |
| 60km/h | 0.55 | 0.21 |
| 90km/h | 0.74 | 0.32 |

According to the result in Table 2.2-1, MRRT = 1/2 is relatively easy to predict under the current simulation assumption. From our simulation result in Table 2.2-2, the prediction of MRRT = 3/4 can also achieve good performance. Therefore, we propose:

Proposal 4: For temporal domain prediction with sample period of 40ms, consider 3/4 as another alternative value of measurement reduction rate.

    - Simulation results (template table)

The simulation results are illustrated in the following template table:

Table 2.2-3 Simulation results in template table

| Report parameters |  | vivo | vivo | vivo | vivo | vivo | vivo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | option 1 | option 1 | option 1 | option 1 | option 1 | option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | option 2 | option 2 | option 2 | option 2 | option 2 | option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30km/h | 60km/h | 90km/h | 30km/h | 60km/h | 90km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Whether LOSsoft is modeled or not | No | No | No | No | No | No |
|  | spatial consistency option (A or B) | A | A | A | A | A | A |
|  | Number of TX beams | 8 | 8 | 8 | 8 | 8 | 8 |
|  | Number of RX beams | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50% | 50% | 50% | 75% | 75% | 75% |
|  | Observation window(Note3) | 800ms | 800ms | 800ms | 1600ms | 1600ms | 1600ms |
|  | Prediction window (Note3) | 40ms | 40ms | 40ms | 120ms | 120ms | 120ms |
|  | Any other parameters (Note 4) |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 200000 | 200000 | 200000 | 200000 | 200000 | 200000 |
|  | Testing | 40000 | 40000 | 40000 | 40000 | 40000 | 40000 |
| AI/ML model input/output | Model input (Note 5) | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP |
|  | Model output(Note 6) | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM | LSTM | LSTM | LSTM | LSTM | LSTM |
|  | Model complexity in a number of parameters(M) | 0.016 | 0.016 | 0.016 | 0.017 | 0.017 | 0.017 |
|  | Model complexity in model size (e.g. Mbyte) | 0.069 | 0.069 | 0.069 | 0.069 | 0.069 | 0.069 |
|  | Computational complexity [FLOPs] | 0.169 | 0.169 | 0.169 | 0.169 | 0.169 | 0.169 |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.07 | 0.13 | 0.20 | 0.13 | 0.21 | 0.32 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | N/A | N/A | N/A | N/A | N/A | N/A |

  - FR1 to FR1 inter-frequency (frequency domain)

For the inter-frequency simulation, we use RSRP measurement results at 10 historical time instances (including the current time instance) of 4GHz to predict the current RSRP of 2GHz. For example, the RRM measurement prediction at time instance {10} of 2GHz can be inferred from the RRM measurement at time instances {1, 2, ..., 10} of 4GHz.

Figure 2.3-1 Inter-frequency prediction

    - Simulation results (template table)

The simulation results of inter-frequency are illustrated in the following template table:

Table 2.3-1 Simulation results in template table

| Report parameters |  | vivo | vivo | vivo |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | option 1 | option 1 | option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | option 2 | option 2 | option 2 |
|  | UE speed (30,60,90,120 Km/h) | 30km/h | 60km/h | 90km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes | Yes | Yes |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full | Full | Full |
|  | Whether LOSsoft is modeled or not | No | No | No |
|  | spatial consistency option (A or B) | A | A | A |
|  | Number of TX beams | 8 | 8 | 8 |
|  | Number of RX beams | 1 | 1 | 1 |
|  | Measurement reduction rate(50%,…Note2) | 100% | 100% | 100% |
|  | Observation window(Note3) | 400ms | 400ms | 400ms |
|  | Prediction window (Note3) | 0ms | 0ms | 0ms |
|  | Any other parameters (Note 4) | N/A | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 200000 | 200000 | 200000 |
|  | Testing | 40000 | 40000 | 40000 |
| AI/ML model input/output | Model input (Note 5) | History L3 cell level RSRPof 4GHz | History L3 cell level RSRPof 4GHz | History L3 cell level RSRP of 4GHz |
|  | Model output(Note 6) | Predicted L3 cell level RSRP of 2GHz | Predicted L3 cell level RSRP of 2GHz | Predicted L3 cell level RSRP of 2GHz |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM | LSTM | LSTM |
|  | Model complexity in a number of parameters(M) | 0.016 | 0.016 | 0.016 |
|  | Model complexity in model size (e.g. Mbyte) | 0.069 | 0.069 | 0.069 |
|  | Computational complexity [FLOPs] | 0.169 | 0.169 | 0.169 |
| Metrics | Average L3 cell level RSRP difference (dB) | 2.33 | 2.35 | 2.32 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | N/A | N/A | N/A |

From our simulation results above, we can see that inter-frequency RRM prediction accuracy is almost the same for different UE speeds, which means the prediction is independent of the UE speed.

Observation 1: For Inter_F_C (inter-frequency inter-cell), the RRM prediction accuracy is almost the same for different UE speeds.

    - Consideration of non-colocated

With the UE moves, the typical RSRP values of the co-located inter-frequency cells are shown in the figure below. The sudden changes in RSRP are due to the transition from Line-of-Sight (LOS) to Non-Line-of-Sight (NLOS). During LOS or NLOS periods, the RSRP difference between the two cells is relatively stable. Therefore, it is easy to achieve good performanc of prediction accuracy for the co-located scenario. In order to expand the application scope of inter-frequency prediction, we think that non-colocated inter-frequency prediction should also be considered.

Figure 2.3-2 L3 cell-level RSRP difference(dB) between 4GHz and 2GHz

Observation 2: For Inter_F_C (inter-frequency inter-cell), the RSRP difference of the co-located cells is relatively stable.

Proposal 5: For Inter_F_C RRM prediction, non-colocated neighbouring cell should also be considered.

  - FR2 to FR2 intra-frequency (temporal domain)

    - Sub-use case 1/2/3

3 sub-use cases are considered for cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The simulation results of FR2 intra-frequency for the 3 sub-use cases are illustrated in the following tables:

Table 2.4-1 Simulation results of measurement prediction accuracy

| Speed | Prediction accuracy of Non-AI mechanism (sample and hold) | Prediction accuracy of AI-based mechanism (L3 cell-level) |  |  |
| --- | --- | --- | --- | --- |
|  |  | sub-use case 1 | sub-use case 2 | sub-use case 3 |
| 60km/h | 2.4 | 1.8 | 1.3 | 1.2 |
| 90km/h | 3.1 | 2.2 | 1.9 | 1.7 |
| 120km/h | 3.6 | 2.5 | 2.2 | 1.9 |

From the simulation results, it can be observed that sub-use case 2 and sub-use case 3 show better performance of measurement prediction accuracy than sub-use case 1.

Observation 3: For FR2 to FR2 intra-frequency temporal domain prediction, sub-use cases 2 and 3 show better performance of measurement prediction accuracy than sub-use case 1.

Proposal 6: RAN2 to discuss whether to down-select the sub-use cases based on simulation results.

    - Simulation results (template table)

The simulation results of intra-frequency are illustrated in the following template table:

Table 2.4-2 Simulation results in template table (Sub-use case 1)

| Report parameters |  | vivo | vivo | vivo |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | option 1 | option 1 | option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | option 2 | option 2 | option 2 |
|  | UE speed (30,60,90,120 Km/h) | 60km/h | 90km/h | 120km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A | N/A | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A | N/A | N/A |
|  | Whether LOSsoft is modeled or not | No | No | No |
|  | spatial consistency option (A or B) | A | A | A |
|  | Number of TX beams | 32 | 32 | 32 |
|  | Number of RX beams | 8 | 8 | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A | N/A | N/A |
|  | Observation window(Note3) | 400ms | 400ms | 400ms |
|  | Prediction window (Note3) | 400ms | 400ms | 400ms |
|  | Any other parameters (Note 4) | N/A | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 200000 | 200000 | 200000 |
|  | Testing | 40000 | 40000 | 40000 |
| AI/ML model input/output | Model input (Note 5) | History L1 beam level RSRP | History L1 beam level RSRP | History L1 beam level RSRP |
|  | Model output(Note 6) | Predicted L1 beam level RSRP | Predicted L1 beam level RSRP | Predicted L1 beam level RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | CNN | CNN | CNN |
|  | Model complexity in a number of parameters(M) | 4.51 | 4.51 | 4.51 |
|  | Model complexity in model size (e.g. Mbyte) | 18.07 | 18.07 | 18.07 |
|  | Computational complexity [FLOPs] | 7.37 | 7.37 | 7.37 |
| Metrics | Average L3 cell level RSRP difference (dB) | 1.8 | 2.2 | 2.5 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | L1 beam level RSRP difference = 2.3 L3 beam level RSRP difference = 1.5 | L1 beam level RSRP difference = 2.8 L3 beam level RSRP difference = 1.9 | L1 beam level RSRP difference = 3.3 L3 beam level RSRP difference = 2.2 |

Table 2.4-3 Simulation results in template table (Sub-use case 2)

| Report parameters |  | vivo | vivo | vivo |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | option 1 | option 1 | option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | option 2 | option 2 | option 2 |
|  | UE speed (30,60,90,120 Km/h) | 60km/h | 90km/h | 120km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP |
|  | Whether LOSsoft is modeled or not | No | No | No |
|  | spatial consistency option (A or B) | A | A | A |
|  | Number of TX beams | 32 | 32 | 32 |
|  | Number of RX beams | 8 | 8 | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A | N/A | N/A |
|  | Observation window(Note3) | 400ms | 400ms | 400ms |
|  | Prediction window (Note3) | 400ms | 400ms | 400ms |
|  | Any other parameters (Note 4) | N/A | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 200000 | 200000 | 200000 |
|  | Testing | 40000 | 40000 | 40000 |
| AI/ML model input/output | Model input (Note 5) | N/A | N/A | N/A |
|  | Model output(Note 6) | N/A | N/A | N/A |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM | LSTM | LSTM |
|  | Model complexity in a number of parameters(M) | 0.76 | 0.76 | 0.76 |
|  | Model complexity in model size (e.g. Mbyte) | 3.04 | 3.04 | 3.04 |
|  | Computational complexity [FLOPs] | 15.18 | 15.18 | 15.18 |
| Metrics | Average L3 cell level RSRP difference (dB) | 1.3 | 1.9 | 2.2 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | N/A | N/A | N/A |

Table 2.4-4 Simulation results in template table (Sub-use case 3)

| Report parameters |  | vivo | vivo | vivo |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | option 1 | option 1 | option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | option 2 | option 2 | option 2 |
|  | UE speed (30,60,90,120 Km/h) | 60km/h | 90km/h | 120km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | History L3 cell level RSRP | History L3 cell level RSRP | History L3 cell level RSRP |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP | Predicted L3 cell level RSRP |
|  | Whether LOSsoft is modeled or not | No | No | No |
|  | spatial consistency option (A or B) | A | A | A |
|  | Number of TX beams | 32 | 32 | 32 |
|  | Number of RX beams | 8 | 8 | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A | N/A | N/A |
|  | Observation window(Note3) | 400ms | 400ms | 400ms |
|  | Prediction window (Note3) | 400ms | 400ms | 400ms |
|  | Any other parameters (Note 4) | N/A | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 200000 | 200000 | 200000 |
|  | Testing | 40000 | 40000 | 40000 |
| AI/ML model input/output | Model input (Note 5) | N/A | N/A | N/A |
|  | Model output(Note 6) | N/A | N/A | N/A |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | CNN | CNN | CNN |
|  | Model complexity in a number of parameters(M) | 2.021 | 2.021 | 2.021 |
|  | Model complexity in model size (e.g. Mbyte) | 8.091 | 8.091 | 8.091 |
|  | Computational complexity [FLOPs] | 4.885 | 4.885 | 4.885 |
| Metrics | Average L3 cell level RSRP difference (dB) | 1.2 | 1.7 | 1.9 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) | N/A | N/A | N/A |

In general, the simulation results in template tables (in sections 2.2.3, 2.3.1 and 2.4.2) can be captured into the TR.

Proposal 7: Capture the simulation results in template tables (in sections 2.2.3, 2.3.1 and 2.4.2) into the TR.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Methodology of simulation

Proposal 1: For temporal domain case B prediction for measurement reduction, further clarify the timing relationship between the measurement and prediction. The following two options can be considered:

- Option 1: the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

- Option 2: The input is historical measurement values and the output is the values between the historical measurement time instances where measurement is skipped, i.e., the prediction is at historical time instance(s).

Proposal 2: For the Cell-level measurement prediction, select the neighbor cells with high RSRP as the test set. How to select the cells is up to company’s implementation and report.

FR1 to FR1 intra-frequency (temporal domain)

Proposal 3: To evaluate the gain of AI-based prediction, introduce a non-AI mechanism as the benchmark, e.g., sample and hold (the predicted value equals the previous actual measured value). Other non-AI mechanisms are up to company’s implementation and report.

Proposal 4: For temporal domain prediction with sample period of 40ms, consider 3/4 as another alternative value of measurement reduction rate.

FR1 to FR1 inter-frequency (frequency domain)

Observation 1: For Inter_F_C (inter-frequency inter-cell), the RRM prediction accuracy is almost the same for different UE speeds.

Observation 2: For Inter_F_C (inter-frequency inter-cell), the RSRP difference of the co-located cells is relatively stable.

Proposal 5: For Inter_F_C RRM prediction, non-colocated neighbouring cell should also be considered.

FR2 to FR2 intra-frequency (temporal domain)

Observation 3: For FR2 to FR2 intra-frequency temporal domain prediction, sub-use cases 2 and 3 show better performance of measurement prediction accuracy than sub-use case 1.

Proposal 6: RAN2 to discuss whether to down-select the sub-use cases based on simulation results.

Proposal 7: Capture the simulation results in template tables (in sections 2.2.3, 2.3.1 and 2.4.2) into the TR.

- Reference

- TR 38.843		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface