3GPP TSG-RAN WG2 Meeting #127	R2-2406402

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	Other aspects related to RRM measurement prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, RAN2 discussed the system-level performance and agreed on the following way forward.

=>	We will start the evaluation with measurements prediction accuracy and model complexity.  We can discuss system performance after we see which scenarios have good measurements prediction accuracy.

Based on the simulation results in [1], we will further discuss whether and what system-level performance evaluation is needed for RRM measurement prediction.

In addition, we will further analyze the remaining issues related to RRM measurement prediction.

- Discussion

  - System-level performance KPI

For the use case of beam management in NR_AI/ML_Air, the performance KPI includes the following two types:

- Type 1: Beam prediction accuracy-related KPIs, e.g.,

- Average L1-RSRP difference of Top-1 predicted beam

- CDF of L1-RSRP difference for Top-1 predicted beam

- Beam prediction accuracy

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam

- Type 2: System performance-related KPIs, e.g.,

- UE throughput: CDF of UE throughput, average and 5%-ile UE throughput

- RS overhead reduction

Clauses 6.3.2 of TR 38.843 [2] captured the simulation results of AI/ML-based beam management. Among them, the prediction accuracy-related KPIs (Type 1) can provide a general understanding of the performance of AI/ML model, while the system performance-related KPIs (Type 2) can directly reflect the impact of introducing AI/ML, e.g., how much the throughput decreases when beam prediction is introduced and the corresponding RS overhead is reduced.

Observation 1: For the use case of beam management in NR_AI/ML_Air, both prediction accuracy KPI and system performance KPI (e.g., UE throughput) are considered.

Observation 2: The KPI of UE throughput can reflect how much the system performance decreases when beam prediction is introduced for RS overhead reduction.

The simulation results of measurement prediction accuracy in [1] are copied below:

Table 2.1-1 Simulation results of measurement prediction accuracy

| Speed | Prediction accuracy of Non-AI mechanism (sample and hold) | Prediction accuracy of AI-based mechanism (L3 cell-level) |  |  |
| --- | --- | --- | --- | --- |
|  |  | sub-use case 1 | sub-use case 2 | sub-use case 3 |
| 60km/h | 2.4 | 1.8 | 1.3 | 1.2 |
| 90km/h | 3.1 | 2.2 | 1.9 | 1.7 |
| 120km/h | 3.6 | 2.5 | 2.2 | 1.9 |

For the RRM measurement, RAN4 has specified the measurement accuracy requirement as follows, e.g.,±6 dB for FR2 [4]. Similar requirements for prediction accuracy can also be evaluated by RAN4.

##### 10.1.3.1.1 Absolute SS-RSRP Accuracy Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2 Accuracy; Conditions. Normal condition; Extreme condition; SSB Ês/Iot; Io Note 2 range. Minimum Io; Maximum Io. dB; dB; dB; dBm / SCSSSB Note 1; dBm/BWChannel; dBm/BWChannel. SCSSSB = 120kHz; SCSSSB = 240kHz. 6; 9; ≥-6; Same value as SSB_RP in Table B.2.2-2, according to UE Power class, operating band and angle of arrival; N/A; -70. 8; 11; N/A; -70; -50. Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table..

Considering RAN4 scope/work will be defined and confirmed by RAN#105 after some RAN2 discussions, we propose:

Proposal 1: RAN2 foresees that the requirements for RRM measurement prediction accuracy need RAN4 input. Detailed work can be confirmed at the RANP#105 meeting.

From our understanding, the measurement prediction accuracy is an intermediate KPI. It can be utilized to compare the performance of sub-use cases. However, if the system-level performance is not considered, the impact of introducing AI/ML for mobility cannot be fully assessed.

At the RAN2#126 meeting, two study goals of RRM prediction were agreed:

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

For 1st study goal, similar to AI/ML for beam management, system performance is necessary to reflect the impact on mobility performance when RRM measurement prediction is used instead of actual measurements. For 2nd study goal, system performance is also necessary to evaluate the optimization of mobility robustness after the introduction of RRM measurement prediction.

Therefore, we propose:

Proposal 2: System-level performance evaluation is needed to reflect the impact on mobility performance after RRM measurement prediction is introduced for both study goals.

For the specific system-level performance KPI, we suppose all the mobility-related performance KPIs listed in the SID can be considered, such as Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, measurement reduction, etc.

As to the definition, the following KPIs are captured in TR 36.839 and can be reused:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- A Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- The RLF performance metric is defined as the average number of RLF occurrences per UE per second.

- The HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

For the Handover interruption, modeling the detailed RACH procedure and the corresponding interruption during handover presents a complex challenge for simulation. Assuming that each handover procedure may introduce close interruption, one simple approach is to introduce an equivalent KPI, such as handover frequency, which can be defined as the number of HO occurrences per UE per second.

Based on the above, we propose:

Proposal 3: The system-level performance KPIs for RRM measurement prediction include Ping-pong HO rate, short ToS rate, HOF rate, RLF frequency, handover interruption.

Proposal 4a: Reuse the definition of the following system-level performance KPIs in 36.839:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- RLF frequency is defined as the average number of RLF occurrences per UE per second.

- HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

Proposal 4b: The KPI of handover interruption can be reflected as HO occurrences per UE per second.

  - Other parameters for simulation

At the RAN2#126 meeting, the general parameters to acquire the dataset for RRM prediction are discussed.

To assess the system-level performance after introducing RRM measurement prediction, the RLF and mobility-related parameters also needed to be discussed and aligned for simulation.

2.2.1 RLF-related parameters

To get the system-level performance KPI like RLF frequency (as outlined in section 2.1), it is essential to first align the Radio Link Failure (RLF) modeling. The RLF modeling-related parameters are detailed in the table below [3]. Additionally, Qout is monitored using a 200ms window, and Qin is monitored using a 100ms window.

Table 2.2-1: The parameters for determining the RLFs and the PDCCH failures.

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 36.331) |
| N310 | 1 |
| N311 | 1 |

We believe that the parameters can be reused. Therefore, we propose:

Proposal 5: Reuse the RLF modeling-related parameters in TR 36.839 for system-level performance evaluation:

| Parameter | Value |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| Qin sliding window length | 100ms |
| Qout sliding window length | 200ms |
| T310 | 1s |
| N310 | 1 |
| N311 | 1 |

2.2.2 Mobility-specific parameters

The following tables capture the additional recommended HetNet mobility-specific parameters in [3]:

Table 2.2-2: HetNet mobility specific parameters

| Items | Description |
| --- | --- |
| Pico cell placement | At fixed location(s) e.g., at 0.5 ISD, 0.3 ISD on the boresight direction. Or randomly placed. |
| Cell loading (NOTE 1) | 100%, 50% |
| UE speed | 3 km/h, 120km/h, 30km/h, 60km/h |
| Channel model | Either one of the models, TU or ITU, could be used. (fast fading included) |
| TimeToTrigger [ms] | 40, 80, 160, 480 |
| a3-offset [dB] | -1, 0, 1, 2, 3 |
| TMeasurement_Period, Intra, L1 filtering time in TS36.133 [2] | 200ms (other values could be added later) |
| Layer3 Filter Parameter K | 4, 1, 0 |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

Table 2.2-3: Summary of Mobility related simulation parameters for the MSE

| HO Parameter | Value |
| --- | --- |
| Time To Trigger (TTT) | Dynamic, 480 ms in normal Mobility |
| A3 Offset | 3 dB Macro and Pico |
| Ping-Pong-Time | 1 s |
| Measurements Rate | 0.2 s |
| HO Execution Time (including Preparation) | 0.15 s |
| RSRP error – zero mean Gaussian | 1 dB std |
| Filtering Factor K | 4 |
| RLF: Qout Threshold | - 8 dB |
| RLF: Qin Threshold | - 6 dB |

We think that most of the parameters can be reused in AI-based mobility simulation. For the baseline value of Time to Trigger, we think that both a long TTT such as 480ms, and a short TTT such as 80ms should be considered to compare with the AI-based mobility performance. Because long TTT aims to avoid ping-pong and short ToS, but may result in more handover failure problems. While short TTT aims to avoid handover failure problems, but may result in more ping-pong and short ToS. The potential gain for AI-based mobility is to reduce HOF, ping-pong and short ToS at the same time or to achieve better balance for these metrics, therefore, only when AI-based mobility shows gains compared to both long TTT and short TTT configurations can it be considered to have gains.

Proposal 6: Take the mobility-specific simulation parameters in TR 36.839 as the baseline for system-level performance evaluation:

| Parameter | Value |
| --- | --- |
| A3 Offset | 3 dB |
| TimeToTrigger | 480 ms (baseline), 80 (baseline) |
| Ping-Pong-Time/short time of stay | 1 s |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

  - Simulation result of system-level KPI

In this section, three types of HO mechanism are simulated:

- Set 1: legacy HO with TTT=480 ms;

- Set 2: legacy HO with TTT=80 ms;

- Set 3: AI-based HO with measurement duration=80ms and prediction window=400ms. The prediction window is set to 400ms to reuse the result of [1].

For Set 3, the detailed procedures of the AI-based HO mechanism are as follows:

- Step 1: NW may configure the UE with a prediction window and the time instance within the window;

- Step 2: UE performs the measurement and inference to obtain the RRM prediction at the time instance within the window.

- Step 3: UE determines whether to send a report to NW based on the RRM prediction. The report may include the current RRM measurement and the RRM prediction.

- Step 4: NW determines whether to HO the UE to another cell based on the report.

The simulation results of each set are summarized in the following Table 2.3-1. The HOF rate and Short-ToS rate are further illustrated in Figure 2.3-1 and Figure 2.3-2.

Table 2.3-1: Simulation results of system-level KPI for each parameter set

| Parameters | KPI | 60km/h | 90km/h | 120km/h |
| --- | --- | --- | --- | --- |
| Set 1: Legacy HO TTT = 480ms | HOF rate | 11.3% | 13.0% | 15.2% |
|  | Ping-pong rate | 3.2% | 3.7% | 4.2% |
|  | Short ToS rate | 26.1% | 28.0% | 30.1% |
| Set 2: Legacy HO TTT = 80ms | HOF rate | 6.3% | 6.9% | 8.1% |
|  | Ping-pong rate | 9.7% | 12.5% | 13.4% |
|  | Short ToS rate | 35.3% | 42.4% | 46.9% |
| Set 3: AI-based HO Measurement= 80ms Prediction= 400ms | HOF rate | 7.8% | 8.5% | 9.4% |
|  | Ping-pong rate | 5.5% | 6.2% | 6.9% |
|  | Short ToS rate | 30.0% | 33.3% | 38.2% |

Figure 2.3-1: HOF rate of legacy HO and AI-based HO

Figure 2.3-2: Short-ToS rate of legacy HO and AI-based HO

With system-level performance, it can be observed that the AI-based HO can balance the HOF rate and Short-ToS rate compared with legacy HO.

Observation 3: Compared with legacy HO, The AI-based HO relies on RRM prediction can balance the HOF rate and Short-ToS rate.

Therefore, we propose:

Proposal 7: System-level performance evaluation results can be provided by companies and captured in the TR.

  - Remaining issues related to RRM measurement prediction

2.4.1 Sub-use cases

In NR, beamforming technology is used as the underlying technology. The UE measures multiple beams of a cell and the measurement results are averaged to derive the cell quality. The NR measurement model is as below (extracted from [2]):

The corresponding high-level measurement model is described below: Figure 9.2.4-1: Measurement Model

In general, L3-based handover decision is based on cell-level measurements derived from beam-level measurements in Layer 1. In addition, RACH resources for contention-free RACH are bounded with specific SSBs(beams). If target gNB wants UE to perform a contention-free RACH procedure, target gNB first needs to select an SSB with good quality, and then allocate the dedicated (contention-free) RACH resource associated with the selected SSB. Therefore, beam-level measurements are useful for handover and L3 beam-level RRM prediction of target cell, e.g., the best K (K>= 1) beams, can be considered for the dedicated RACH resource configuration.

Observation 4: L3 beam-level measurements can be utilized for dedicated RACH resource configuration.

Proposal 8: In addition to Cell-level RRM measurement prediction, L3 beam-level prediction should be considered for the dedicated RACH resource configuration.

If the above proposal is agreed, for beam-level prediction (i.e., Case1), L3 beam-level qualities can be derived based on the predicted L1 beam qualities. For cell-level prediction, AI model should also consider beam-level output. If so, we think that a new sub-use case can be considered to directly predict cell-level results and beam-level results. For the new sub-use case, beam-level results are necessary as input since the output includes beam-level results. Cell-level results may be optional for input.

Proposal 9: For RRM measurement prediction, besides Case1-3, a new sub-use case (Case 4) is considered: To directly predict cell-level results and beam-level results based on beam-level results and optional cell-level results.

In the legacy measurement model, there are the following multiple measurements (extracted from [2]):

-	A: measurements (beam specific samples) internal to the physical layer.

-	A1: measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

-	E: a measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

-	B: a measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

-	C: a measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at point B. This measurement is used as input for one or more evaluation of reporting criteria.

The measurements at point A and point A1 of the measurement model are L1 beam measurements. The measurements at point E are L3-filtered beam measurements. The measurements at point B are cell measurements without L3 filtering. The measurements at point C are L3-filtered cell measurements. We think that the 3 types of measurements can be considered as input and output of RRM measurement prediction for further discussion. As known, L3-filtered qualities are the further filtered ones based on L1 qualities. According to TS38.331 [3], the Layer 3 filtering formula is as below:

Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

According to the Layer 3 filtering formula, the filtering configuration, i.e., filterCoefficient, will greatly impact the filtered measurement value.

Observation 5: The Layer 3 filtering configuration, i.e., filterCoefficient, will greatly impact the filtered measurement value.

We think that input and output should be quantities with the same level of filtering to avoid adverse effects caused by Layer 3 filtering configurations.

Proposal 10: For RRM measurement prediction, RAN2 to discuss whether input and output should be with the same level of L3 filtering, i.e., L3-filtered or non-L3-filtered:

- If the input are non-L3-filtered measurements (i.e., L1 beam measurements or Cell measurements without L3 filtering), the output should be non-L3-filtered;

- If the input are L3-filtered measurements (i.e., L3-filtered beam measurements or L3-filtered cell measurements), the output should be L3-filtered.

2.4.2 Other aspects

In the latest RAN2 meeting, an issue was raised in [4] [5] that it is unclear how to derive cell quality based on both predicted beam qualities for some beams and measured beams qualities for the other beams for the case of spatial RRM prediction. As known, predicted beam qualities are different from measured beams qualities since predicted beam qualities may be inaccurate compared to measured beam qualities. It is unclear whether predicted beam qualities can directly be used together with measured beam qualities for beam consolidation/selection. We think that RAN2 may need to discuss the issue.

Proposal 11: For spatial domain prediction, RAN2 to consider how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

For handover decision and target cell selection, we think that generally the network does not care about the exact predicted measurement results (e.g., predicted cell RSRP value) for the triggered (candidate target) cells for an event since the triggered cells fulfill the event threshold and are enough good, but really the signal change trend of the triggered cells. For example, Event A3 is triggered and the triggered cells include cell 1 and cell 2. Cell 1 is becoming better and cell 2 is becoming worse. Cell 1 may be the better choice as target cell than cell 2 even if the RSRP of cell 2 is higher than the one of cell 1. Therefore we think that the signal change trend of the triggered cells may be more useful than the exact predicted value in some scenarios.

Proposal 12: RAN2 to consider that the output of RRM measurement prediction can include the signal change trend of cells or beams.

At the latest RAN2 meeting, RAN2 had the conclusion of “We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements”. However, the concept of intra-frequency prediction is unclear so far and there may be multiple different understandings.

The definition of inter-frequency and intra-frequency measurements are as follows:

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

It can be observed that the existing definitions of intra-frequency and inter-frequency measurements are differentiated based on whether the center frequency and subcarrier spacing of the measured neighbor cell and serving cell are the same. Therefore, one understanding can be intra-frequency and inter-frequency predictions are also differentiated based on the frequency of the predicted cell and serving cell.

From our understanding, intra-frequency and inter-frequency predictions are differentiated based on whether the center frequencies of input and output are the same.

For example, there are the following assumptions:

- cell 1: serving cell, frequency: f1

- cell 2: neighbor cell, frequency: f1

- cell 3: neighbor cell, frequency: f2

There may be the following different understandings of intra-frequency prediction:

- Understanding 1: the frequency of the predicted cell and the serving cell are the same, e.g., the predicted cell is cell 1 or cell 2. The case of input being cell 3 and output being cell 3 can be categorized as inter-frequency prediction.

- Understanding 2: the frequency of the predicted cell and input cell are the same, e.g., input is cell 3 and output is cell 3.

RAN2 should clarify the concept to align companies’s understanding.

Proposal 13: RAN2 to clarify the concept of intra-frequency prediction:

- Option 1: the frequency of the predicted cell and the serving cell are the same;

- Option 2: the frequency of the predicted cell and the input cell are the same.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

System-level performance KPI

Observation 1: For the use case of beam management in NR_AI/ML_Air, both prediction accuracy KPI and system performance KPI (e.g., UE throughput) are considered.

Observation 2: The KPI of UE throughput can reflect how much the system performance decreases when beam prediction is introduced for RS overhead reduction.

Proposal 1: RAN2 foresees that the requirements for RRM measurement prediction accuracy need RAN4 input. Detailed work can be confirmed at the RANP#105 meeting.

Proposal 2: System-level performance evaluation is needed to reflect the impact on mobility performance after RRM measurement prediction is introduced for both study goals.

Proposal 3: The system-level performance KPIs for RRM measurement prediction include Ping-pong HO rate, short ToS rate, HOF rate, RLF frequency, handover interruption.

Proposal 4a: Reuse the definition of the following system-level performance KPIs in 36.839:

- Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

- Short ToS rate is defined as the number of Short ToS occurrences divided by the number of successful handovers. I.e., Short ToS rate = (number of Short ToS occurrences)/(total number of successful handovers).

- RLF frequency is defined as the average number of RLF occurrences per UE per second.

- HOF rate is defined as Handover failure rate = (number of handover failures) / (Total number of handover attempts).

Proposal 4b: The KPI of handover interruption can be reflected as HO occurrences per UE per second.

Other parameters for simulation

Proposal 5: Reuse the RLF modeling-related parameters in TR 36.839 for system-level performance evaluation:

| Parameter | Value |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| Qin sliding window length | 100ms |
| Qout sliding window length | 200ms |
| T310 | 1s |
| N310 | 1 |
| N311 | 1 |

Proposal 6: Take the mobility-specific simulation parameters in TR 36.839 as the baseline for system-level performance evaluation:

| Parameter | Value |
| --- | --- |
| A3 Offset | 3 dB |
| TimeToTrigger | 480 ms (baseline), 80 (baseline) |
| Ping-Pong-Time/short time of stay | 1 s |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

Simulation result of system-level KPI

Observation 3: Compared with legacy HO, The AI-based HO relies on RRM prediction can balance the HOF rate and Short-ToS rate.

Proposal 7: System-level performance evaluation results can be provided by companies and captured in the TR.

Remaining issues related to RRM measurement prediction

Observation 4: L3 beam-level measurements can be utilized for dedicated RACH resource configuration.

Proposal 8: In addition to Cell-level RRM measurement prediction, L3 beam-level prediction should be considered for the dedicated RACH resource configuration.

Proposal 9: For RRM measurement prediction, besides Case1-3, a new sub-use case (Case 4) is considered: To directly predict cell-level results and beam-level results based on beam-level results and optional cell-level results.

Observation 5: The Layer 3 filtering configuration, i.e., filterCoefficient, will greatly impact the filtered measurement value.

Proposal 10: For RRM measurement prediction, RAN2 to discuss whether input and output should be with the same level of L3 filtering, i.e., L3-filtered or non-L3-filtered:

- If the input are non-L3-filtered measurements (i.e., L1 beam measurements or Cell measurements without L3 filtering), the output should be non-L3-filtered;

- If the input are L3-filtered measurements (i.e., L3-filtered beam measurements or L3-filtered cell measurements), the output should be L3-filtered.

Proposal 11: For spatial domain prediction, RAN2 to consider how to derive cell-level quality based on predicted beam qualities and measured beam qualities.

Proposal 12: RAN2 to consider that the output of RRM measurement prediction can include the signal change trend of cells or beams.

Proposal 13: RAN2 to clarify the concept of intra-frequency prediction:

- Option 1: the frequency of the predicted cell and the serving cell are the same;

- Option 2: the frequency of the predicted cell and the input cell are the same.

- Reference

- R2-2406401	Simulation results for RRM measurement prediction

- TR 38.843		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TR 36.839		Mobility enhancements in heterogeneous networks

- TS 38.133		Requirements for support of radio resource management