3GPP TSG-RAN WG2 Meeting #127	R2-2406403

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	vivo

Title:	Simulation assumptions and evaluation methodology for RLF prediction

Agenda Item:	8.3.4.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#126 meeting, RAN2 reached the following agreements regarding the simulation assumptions and evaluation methodology for RLF prediction.

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this contribution, we will provide further analysis of the simulation assumptions and evaluation methodology specific to RLF.

- Discussion

  - FFS issues on the definition of RLF prediction

At the RAN2#126 meeting, the definition of RLF prediction result for the direct case was agreed and there are two remaining FFS issues to address:

- Whether the definition is applicable to the indirect case for RLF prediction.

- Whether an expected RLF time is needed as another output/result.

From our perspective, indirect RLF prediction is based on the temporal domain serving cell measurement predictions. specifically, the result is derived by post-processing the prediction of SINR within a time window or at a specific time instance, which provides an exact prediction of whether RLF would occur or not, i.e., without probability. The time window or time instance for SINR prediction should be specified in the specification or configured by the network.

Therefore, the terminology of RLF prediction result in the current agreement is not applicable for indirect case, and the terminology can be refined as the ‘RLF occurrence’ instead of ‘RLF probability’ within a time window or at a time instance. Furthermore, for the time window case, an expected RLF time associated with the RLF prediction can also be derived during the post-processing.

Regarding the direct case, RLF prediction is the output of the AI model directly. With prior knowledge, RLF probability can be predicted as a classification problem using input of the current and historical information, e.g., UE mobility, RRM measurement. As for the time information associated with the prediction of RLFprobability, it should be a certain time window/ time instance specified in the specification or configured by the network, i.e., expected RLF time is not needed as another prediction output/result for direction RLF prediction.

Based on the above analysis, we propose:

Proposal 1: For indirect RLF prediction, the result is whether RLF will occur within a time window or at a time instance. If the result is related to the time window, an expected RLF time can be derived as a part of the result.

Proposal 2: For direct RLF prediction, expected RLF time is not needed as another prediction output/result.

Proposal 3: For both direct and indirect RLF prediction, the time window/instance is configured by the network or specified fixed parameter, i.e., not the output of the AI model/functionality.

  - Simulation assumptions of RLF prediction

During the RAN2#126 meeting, it was agreed that the study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. Therefore, RAN2 should further align on the parameters related to RLF due to T310 expiry.

The RLF modeling-related parameters were captured in TR 36.839 as shown in the following table [1]. Additionally, Qout is monitored using a 200ms window, and Qin is monitored using a 100ms window.

Table 2.2-1: The parameters for determining the RLFs and the PDCCH failures.

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 36.331) |
| N310 | 1 |
| N311 | 1 |

We suggest using the parameters as a starting point for further discussion, and they can be refined when needed.

Proposal 4: Take the RLF-related parameters in TR 36.839 as a starting point for further discussion, and the parameters can be refined when needed.

| Parameter | Value |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| Qin sliding window length | 100ms |
| Qout sliding window length | 200ms |
| T310 | 1s |
| N310 | 1 |
| N311 | 1 |

The values of T310 and N311 from the above table will have an impact on the occurrence of RLF. Specifically, a shorter T310 duration or a larger N311 number will lead to a higher RLF occurrence rate. The following three sets of RLF-related parameters can serve as examples:

- Set 1: T310=1s, N311=1 (Same as TR 36.839);

- Set 2: T310=0.2s, N311=4;

- Set 3: T310=0.1s, N311=4.

At the RAN2#126 meeting, it was also agreed that the simulation assumptions of RLF should reuse that of RRM prediction as much as possible. Combining the three sets of RLF-related parameters, the initial simulation results of RLF rate are illustrated in Table 2.2-2.

Table 2.2-2 Simulation results of RLF rate

(TTT = 480ms, N310 = 1, ue_speed = 120km/h)

| Profile | T310 | N311 | Average RLF Occurrences/UE/s |
| --- | --- | --- | --- |
| Set 1 | 1s | 1 | 2.5397e-4 |
| Set 2 | 0.2s | 4 | 5.0794 e-4 |
| Set 3 | 0.1s | 4 | 9.5238e-4 |

Based on the current simulation assumptions for RRM prediction, it can be observed that very few RLF incidents may occur. Below, we display the CDF of SINR at the initial position of each drop.

Figure 2.2-1: CDF of SINR at the initial position

There is no UE with SINR lower than the Qout value, e.g., -8dB after initial access, which means that the coverage is very good in this scenario. With a good coverage range and normal handover strategy, very few RLF incidents are expected to be triggered.

Observation 1: With the simulation assumptions for RRM measurement prediction, very few RLF incidents may occur.

To obtain the dataset for RLF prediction, RAN2 should further discuss the simulation assumptions. From our understanding, there are two options:

- Option 1: Adjust the parameters for RRM prediction that impact coverage, or modify the RLM and handover-related parameters to make RLF occur more frequently.

- Option 2: Exclude handover modeling in the simulation. Specifically, the UE will not handover to neighbor cells, and UE’s trajectory will be terminated upon RLF occurrence.

For Option 1, It can be seen that before and after modifying the RLM parameters, the order of magnitudes of the RLF occurrence probability has not changed, i.e., 10-4. In other words, Modifying the RLM parameters has a limited impact on increasing the probability of RLF occurrences. If we further discuss adjusting other parameters, e.g., parameters for RRM prediction that impact coverage, massive simulation tasks are foreseen.

To simplify the simulation, Option 2 is preferred.

Proposal 5: To obtain the dataset of RLF prediction, UE is not expected to execute handover and UE’s trajectory terminates upon RLF occurrence in simulation.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

FFS issues on definition of RLF prediction

Proposal 1: For indirect RLF prediction, the result is whether RLF will occur within a time window or at a time instance. If the result is related to the time window, an expected RLF time can be derived as a part of the result.

Proposal 2: For direct RLF prediction, expected RLF time is not needed as another prediction output/result.

Proposal 3: For both direct and indirect RLF prediction, the time window/instance is configured by the network or specified fixed parameter, i.e., not the output of the AI model/functionality.

Simulation assumptions of RLF prediction

Proposal 4: Take the RLF-related parameters in TR 36.839 as a starting point for further discussion, and the parameters can be refined when needed.

| Parameter | Value |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| Qin sliding window length | 100ms |
| Qout sliding window length | 200ms |
| T310 | 1s |
| N310 | 1 |
| N311 | 1 |

Observation 1: With the simulation assumptions for RRM measurement prediction, very few RLF incidents may occur.

Proposal 5: To obtain the dataset of RLF prediction, UE is not expected to execute handover and UE’s trajectory terminates upon RLF occurrence in simulation.

- Reference

- TR 36.839		Mobility enhancements in heterogeneous networks