3GPP TSG-RAN WG2 Meeting#127	R2-2406404

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | - | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Tx carrier (re-)selection for SL-PRS transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the agreement in RAN1#126 meeting, SL-PRS transmission/reception is supported to be compatible with SL CA in a Shared SL-PRS resource pool on a single SL carrier, or a Dedicated SL-PRS resource pool in a single SL carrier when the slots configured for such resource pool does not collide with the slots configured for any other resource pool or S-SSB resource(s) in other carriers. In NR Rel-18, in a band (pre)configured with SL CA, SL PRS transmission /reception can be supported: - In a shared SL PRS resource pool in a single SL carrier. - Tx power control follows the rule defined for SL CA in NR Rel-18 - In a dedicated SL PRS resource pool in a single SL carrier when the slots (pre)configured for the dedicated SL PRS resource pool do not collide with the slots (pre)configured for any other resource pool or S-SSB resource(s) in other carriers. - FFS: new UE capability(ies) are defined for this combination of features Note: whether this combination of features is supported in Rel-18 requires a conclusion on whether to introduce new UE capability(ies). No specification work until the FFS is resolved.. However, this agreement is not implied in any specification in any form. It is thus necessary to make it clear, and MAC Spec is an appropriate place to capture since the detailed procedure of SL-PRS transmission is illustrated in MAC spec. To be specific, when the condition goes into that “if multiple SL carriers are configured”, a sidelink process triggered for SL-PRS transmission goes into Tx carrier selection part if such UE supports SL CA. Following the CBR-priority criterion as SL data, the only difference for SL-PRS is that UE should only decide on one SL carrier among all candidate SL carriers to proceed the following procedure. Another editorial change is due to the duplicated note number. |
| Summary of change: |  | In carrier selection triggered for sidelink process related to SL-PRS transmission, add the restriction on UE behaviour to select only one SL carrier among all the candidate carriers. Change the serial number of the note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may transmit SL-PRS on multiple selected carriers which may functionalize erroneously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.1, 5.22.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 5.22.1.1	SL Grant reception and SCI transmission

<omitted>

NOTE 3Al:	MAC entity, based on UE implementation, decides how to determine COT sharing cast type, COT sharing additional ID and remaining COT duration specified in TS 37.213 [18].

NOTE 3Am:	If sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and if multiple preferred resource sets are received from the same UE, it is up to UE implementation to use one or multiple of them in its resource (re)selection.

<omitted>

End of change

Start of change

#### 5.22.1.11	TX carrier (re-)selection

The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.215 [24] if CBR measurement results are available, or the corresponding sl-DefaultTxConfigIndex configured by upper layers if CBR measurement results are not available.

If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, 5.22.1.2 or 5.22.1.3.3, the MAC entity shall:

1>	if SL-PRS is pending for transmission:

2>	if there is no selected sidelink grant on any carrier allowed for SL-PRS transmission as indicated by upper layers (TS 38.331 [5]):

3>	for each carrier configured by upper layers allowed for SL-PRS transmission:

4>	if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the pending SL-PRS:

NOTE X:	In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation.

5>	consider the carrier as a candidate carrier for TX carrier (re-)selection for the pending SL-PRS.

2>	else:

3>	for each pending SL-PRS, if any, that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1, if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the SL-PRS:

4>	select the carrier and the associated pool of resources.

3>	else:

4>	for each carrier configured by upper layers on which the pending SL-PRS is allowed, if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the SL-PRS:

5>	consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the pending SL-PRS is allowed.

- else:

2>	if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]):

3>	for each carrier configured by upper layers associated with the concerned sidelink logical channel:

4>	if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

NOTE 1:	In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation, taking into account of sl-HARQ-FeedbackEnabled for the sidelink logical channel.

5>	consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel.

2>	else:

3>	for each sidelink logical channel, if any, where data is available and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1, if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel:

4>	select the carrier and the associated pool of resources.

3>	else:

4>	for each carrier configured by upper layers on which the sidelink logical channel is allowed, if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

5>	consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed.

The MAC entity shall select a carrier on which the SL-CSI Request was received as a carrier for transmission of a Sidelink CSI Reporting MAC CE. The MAC entity shall for the Sidelink CSI Reporting MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall select a carrier on which a Sidelink Inter-UE Coordination Request was received as a carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE. The MAC entity shall for the Sidelink Inter-UE Coordination Information MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall:

1>	if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection:

2>	if Tx carrier (re-)selection is triggered, for each pending SL-PRS allowed on the carrier:

3>	select one carrier among the candidate carriers with the lowest CBR, and select the associated pool(s) of resources.

2>	else if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available:

3>	select one or more carrier(s) among the candidate carriers with increasing order of CBR from the lowest CBR, and select the associated pool(s) of resources:

4>	if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel:

5>	select one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

4>	else:

5>	select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

NOTE 2:	It is left to UE implementation how many carriers to select based on UE capability.

NOTE 3:	It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.

NOTE 4:	It is left to UE implementation to determine whether the resource pool for CBR measurement is reused as the resource pool for SL grant creation.

NOTE 5:	For the transmission of Sidelink Inter-UE Coordination Request MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is required. For the transmission of Sidelink Inter-UE Coordination Information MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is located.

End of change