3GPP TSG-RAN WG2 Meeting #127	R2-2406406

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:   8.2.5

Source:        Xiaomi

Title:           Considerations on TP2 related aspects between BS and UE reader

Document for:  Discussion and Decision

Introduction

In the Rel-19 Ambient IoT system, topologies 1 & 2 (UE as an intermediate node under NW control) as per TR 38.848 are to be supported, as shown in the pictures below.[1]

Fig1. Topology 1: BS ↔ Ambient IoT device

Fig2: Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

During the previous discussion, RAN2 focused on the study of the interface between the reader and the device in TP1 and TP2, aiming to achieve a harmonized design across both topologies. [2]

In this contribution, we provide our considerations on the delta part in the TP2, i.e., the Uu interface between UE reader and BS.

Discussion

  - General principle

As outlined in the SID's general scope, the communication spectrum of the new air interface is assumed to be FR1 licensed spectrum in FDD. Consequently, the radio resource for the Ambient IoT interface should be under the control of the BS, i.e., the radio resource for the A-IoT interface should be managed by the BS and allocated to the UE for A-IoT operations.

Proposal 1: In Topology 2, the radio resource for the A-IoT interface should be controlled by the BS and allocated to the UE reader.

As for how to perform the UE reader selection/reselection in TP2, to ensure better network control, it should be performed under the control of NW node (either NG-RAN or CN node). Since this issue is more relevant to the network side and is being discussed in RAN3 as quoted below, we can just leave it to RAN3 and SA2, and wait for the progress on the conclusion or any other impacts on RAN2.

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

Observation 1: The UE reader should be selected by the network node, either gNB or CN node, which is still under other WGs discussion.

Proposal 2: RAN2 waits for the RAN3 and SA2 progress on the UE reader selection.

  - A-IoT related Uu protocol stack

In TP2, for the Uu protocol stack for communication between the BS and UE reader, there are many options proposed during the discussions in RAN3 [3] and SA2 [4], as summarized below:

Option 1: UE reader’s PDU-based solution

Option 2: UE reader’s NAS-based solution

Option 3: UE reader’s RRC-based solution

From the RAN2 perspective, we can consider option 3 for further study, while the options 1 and 2 are more relative to the other WGs and have a minor impact on RAN2, we can just leave them to RAN3 and SA2.

As for the AS-aware solution, in addition to the RRC-based solution, given that the device data size is small due to device capability limitations and forwarding to the RRC layer may cause data handling delays on the UE reader side, we can also consider the MAC-based solution. Upon receiving the data from the device, the UE reader can put the data into the MAC CE and transfer it to the BS to reduce the latency.

Proposal 3: Upper layer based solution (e.g., PDU based solution, NAS based solution) can be left to RAN3 and SA2.

Proposal 4: For the Uu protocol stack for the device data transfer, the following AS based solutions can be considered:

- Option1: Protocol stack for device data transfer via MAC CE

- Option2: Protocol stack for device data transfer via RRC signalling

  - TP2 specific issues on the UE reader side

    - Device data aggregated on the UE reader side

During the previous discussion, RAN2 agreed to support paging multiple devices for response in one A-IoT procedure. For the procedure involving multiple devices, upon received the response from the device, there are two options for the reader to forward it to the network:

Option 1: Forward the device response one by one immediately upon received it. Take inventory as an example, as shown below:

Fig.1 Non-aggregated data forwarding on reader side

Option 2: Aggregate multiple responses and forward them together in one message. Take inventory as an example, as shown below:

Fig.2 Aggregated data forwarding on reader side

For the options above, we can note that option1 could result in a signalling storm between the reader and the A-IoT CN node, while option2 would have a significantly smaller signalling overhead at the expense of response latency.

RAN3 discussed this issue for TP1 last meeting and achieved the following agreement.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

As the options above have impacts on different interfaces in TP1 and TP2, we should discuss in RAN3 and RAN2 separately. Also, for the signaling storm introduced in Option 1, it has much more serious impact in the TP2 compared to TP1, as usually we care more about the radio resource consumption in the Uu interface.

From this view, Option 2 is a more suitable and efficient choice for TP2. Furtherly, considering the increased response latency, Option 2 can be applied to the A-IoT procedure, which is not sensitive to latency, such as the inventory use case. As for the command use case, we can analyze it further or leave it to the UE implementation.

Observation 2: UE forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 5: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

    - RRC state of the UE reader

Once a UE is selected as an intermediate node and has received the service request from the network while in RRC_CONNECTED state, whether the UE need to remain in the RRC_CONNECTED state to serve as an intermediate node needs to be discussed.

As a baseline, data collection from devices is most efficient when the UE is in the RRC_CONNECTED state, in order to ensure the real-time communication with the network. Additional, in some certain scenarios, the UE can collect and aggregate device data for later transmission, allowing data/signaling exchanges on the IoT interface to occur independently of the RRC connection. In this case, to reduce the UE power consumption, it is unnecessary to require the UE to remain in the RRC_CONNECTED state. The UE can collect the device data in RRC_IDLE or RRC_INACTIVE state and switch to RRC_CONNECTED state for data reporting.

Observation 3: If device data can be aggregated in the UE, the UE can serve as an intermediate node in the RRC_IDLE or RRC_INACTIVE state to save the power consumption.

Proposal 6: As baseline, the UE reader can keep in RRC_CONNECTED state during the A-IoT procedure.

Proposal 7: RAN2 to further consider the UE reader to be in RRC_INACTIVE or RRC_IDLE state, at least for the inventory use case.

    - Service continuity during the UE reader mobility

According to the SID, the Ambient IoT device does not support mobility. However, the UE reader in the TP2 still needs to support the mobility, similar to the NR system. Considering that the UE also acts as an intermediate node which is responsible for data/signaling exchange at the Ambient IoT interface. The UE mobility in the Uu interface may affect the Ambient IoT procedure. For example, during the device data collection, if the UE moves to the edge of the BS coverage or to a BS that does not support Ambient IoT functionalities at all, the device data forwarding to the network may be interrupted and lead to the service data loss.

In such case, the impact of UE reader mobility on the Ambient IoT procedure at the A-IoT interface needs to be considered and the continuity of Ambient IoT services needs to be supported to prevent data loss and service interruptions.

Observation 4: Service interruption or data loss may occur if the UE moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 8: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

    - UE BSR for the device data forwarding

In the existing mechanism, there is pre-emptive Buffer Status Report (BSR) mechanism which is introduced for the IAB-node to reduce the UL scheduling latency through signalling of a Pre-emptive BSR to its parent node.

Similarly, in the A-IoT system, the UE reader could also consider a BSR mechanism similar to the pre-emptive BSR. Specifically, the UE reader may be able to predict the volume of data it will receive from devices. This prediction could be based on factors such as the estimated number of device responses during an inventory procedure or the expected size of the data to be read from the device. With this predicted data volume, the UE reader could initiate a pre-emptive BSR to reduce data transmission delays. This is particularly beneficial for delay-sensitive processes, such as command use case.

Observation 5: The pre-emptive BSR is introduced and used by the IAB-MT to provide the amount of the data expected to receive from its child node(s) and/or UE(s) connected to it.

Proposal 9: RAN2 to discuss whether/how to support the BSR enhancement, similar to the IAB Pre-emptive BSR.

    - Active time control of PDRCH reception

In the current spec, if the UE is configured with DRX, it does not need to continuously monitor PDCCH for power saving. For a UE serving as an intermediate node in the A-IoT system, it also needs to monitor the PDRCH in the A-IoT interface as well as the PDCCH, and the impacts of the UE DRX on the PDRCH need to be considered.

If there are separate reception module for the PDCCH and PDRCH, the control of the PDCCH and PDRCH monitoring is separate, there is no need for the coordination of the both. However, the additional reception module for PDRCH monitoring on the UE side will increase costs to some extent.

If there is a shared reception module for PDCCH and PDRCH, the DRX configured for the UE applies to both PDCCH and PDRCH monitoring. During the inactivity period for PDCCH monitoring, the UE will also not monitor the PDRCH. To avoid missing PDRCH, the UE reader should remain active for PDRCH reception during A-IoT operation, and the impacts on the UE DRX need to be considered, such as not to configure UE reader with the DRX to always enable PDRCH monitoring, or to configure the UE DRX on-duration time with PDRCH reception taken into account, etc.

Observation 6: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 10: In case the reception module for PDCCH and PDRCH is shared on the UE reader side, RAN2 to discuss the impacts of the PDRCH reception on the UE DRX.

    - Co-existence of the A-IoT function and CA

Carrier aggregation (CA) is used to increase data rates and improve network capacity by utilizing multiple frequency bands simultaneously. However, due to the limited capabilities of A-IoT devices, such as the small size of data packets they handle, there is no need for UE reader to support CA, as it would only add unnecessary complexity to both the UE and the network. Therefore, we propose that a UE serving as an intermediate node in the A-IoT system should not be required to operate in CA mode. Even if the UE can be configured with CA, to reduce the complexity and the impact on the spec, we propose that only the radio resource of the Pcell can be configured to UE for the A-IoT operation towards the device.

Proposal 11: For the co-existence of the A-IoT function and CA, following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

Conclusions

According to the analysis given above, we have the following observations and proposals:

- General principle

Proposal 1: In Topology 2, the radio resource for the A-IoT interface should be controlled by the BS and allocated to the UE reader.

Observation 1: The UE reader should be selected by the network node, either gNB or CN node, which is still under other WGs discussion.

Proposal 2: RAN2 waits for the RAN3 and SA2 progress on the UE reader selection.

- A-IoT related Uu protocol stack

Proposal 3: Upper layer based solution (e.g., PDU based solution, NAS based solution) can be left to RAN3 and SA2.

Proposal 4: For the Uu protocol stack for the device data transfer, the following AS based solutions can be considered:

- Option1: Protocol stack for device data transfer via MAC CE

- Option2: Protocol stack for device data transfer via RRC signalling

- TP2 specific issues on the UE reader side

Device data aggregated on the UE reader side

Observation 2: UE forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 5: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

RRC state of the UE reader

Observation 3: If device data can be aggregated in the UE, the UE can serve as an intermediate node in the RRC_IDLE or RRC_INACTIVE state to save the power consumption.

Proposal 6: As baseline, the UE reader can keep in RRC_CONNECTED state during the A-IoT procedure.

Proposal 7: RAN2 to further consider the UE reader to be in RRC_INACTIVE or RRC_IDLE state, at least for the inventory use case.

Service continuity during the UE reader mobility

Observation 4: Service interruption or data loss may occur if the UE moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 8: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

UE BSR for the device data forwarding

Observation 5: The pre-emptive BSR is introduced and used by the IAB-MT to provide the amount of the data expected to receive from its child node(s) and/or UE(s) connected to it.

Proposal 9: RAN2 to discuss whether/how to support the BSR enhancement, similar to the IAB Pre-emptive BSR.

Active time control of PDRCH reception

Observation 6: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 10: In case the reception module for PDCCH and PDRCH is shared on the UE reader side, RAN2 to discuss the impacts of the PDRCH reception on the UE DRX.

Co-existence of the AIOT function and CA

Proposal 11: For the co-existence of the A-IoT function and CA, following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

References

- TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, 3GPP, September 2023.

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, 3GPP, March 2024.

- RAN3 #124 Meeting Report Fukuoka, Japan, 20/05/2024 - 24/05/2024

- 3GPP TR 23.700-13 V0.3.0 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)