3GPP TSG-RAN WG2 Meeting #127	R2-2406407

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.1.4

Source:	Xiaomi

Title:	Data collection for UE side model training

Document for:		Discussion and Decision

Introduction

In RAN2#126 meeting, following was agreed regarding UE side data collection:

Solution 1a has no specification impact.

For the options identified to realize the different levels of data content visibility

1.	Full visibility for standardized data content.

2.	Partial visibility for partially standardized data content.

3.	No standardized visibility

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

Capture table below in TR (Rapporteur will clean up and refine and review over email)

Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

Email discussion “[POST126][034][AIML PHY] TP for data collection (Ericsson)” [1] was held to discuss how to capture the table and meeting agreements on data collection. Comparison table is put in Annex for convenience.

In this contribution, we discuss data collection for UE side model training.

Discussion

Visibility of data content in MNO

In RAN2#126 meeting, following three levels of data visibility were agreed:

- Full visibility for standardized data content.

- Partial visibility for partially standardized data content.

- No standardized visibility

For Option 2 and 3, possible Options for Visibility of data content in MNO and Data format are as follows:

- Opt A) Full visibility for standardized data content.

- FFS: Opt B) Partial visibility for partially standardized data content.

- FFS: Opt C) No standardized visibility.

- FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility.

UE side data collection is used for the training of AI model, with the aim of model interference at UE side. The collected data for training is directly related to the data for interference at UE side. It is expected that UE/chipset vendor can have different implementations / optimizations regarding types of data for collection / training and interference, and some information can be UE / chipset proprietary. Therefore standardizing all types of data not only increases standardization efforts, but also prohibits product differentiation / optimization / evolution. Hence it is proposed to support partial visibility for partially standardized data content, which includes standardized fields for data visibility and container for UE/chipset proprietary information.

Proposal 1: In Option 2 and 3, partial visibility for partially standardized data content is supported, with standardized fields for data visibility and container for UE/chipset proprietary information.

UP tunnel

There are FFSs for Option 2 and 3 regarding UP tunnel if data volume exceeds the NAS / RRC signalling capacity. This is related to the evaluation of area A1 (Large, no upper limit model/model parameter size) of model transfer in TR 38.843, with the difference that data collection is for UL while model transfer is for DL. Related part from TR 38.843 is summarized in Table 1 below.

Table 1: Comparison of model transfer solutions for large size support (from TR 38.843)

| Model transfer solutions | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| Solution 1a (RRC) | Model size >45kBytes is not supported based on existing number of RRC segments | Extension of the number of RRC segments is required to support models larger than 45kBytes |
| Solution 2a (CN CP) and 3a (LMF CP) | - Model size >45kBytes is not supported based on existing number of RRC segments - Core Network supports NAS signalling segmentation - LMF supports LPP signalling segmentation | If NAS/LMF does not do segmentation for model transfer/delivery, it may need RRC segmentation, and extension of the number of RRC segments is required to support models larger than 45kBytes |
| Solution 1b (UP) | - No model size limitation - PDU session termination at gNB is not supported | Requires PDU session termination at gNB if needed |

One note about above analysis is that from NAS specifications (24.501 and 24.007), it seems that NAS does not support segmentation.

Similar analysis to Table 1 is applicable for UE side data collection. The main difference is that UL RRC signalling capacity is different from that of DL. In UL, there can be maximum 16 segments, as from ASN.1 of ULDedicatedMessageSegment message, copied below:

ULDedicatedMessageSegment-r16-IEs ::=     SEQUENCE {

segmentNumber-r16                         INTEGER (0..15),

...

}

Since the maximum size of each segment is 9 kBytes, UL RRC message can have maximum 16 * 9 kBytes = 144 kBytes.

A comparison table of option 1a/1b/2/3 for large size support is shown in Table 2 below.

Table 2: Comparison of UE side data collection options for large size support

| Model transfer solutions | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| Option 1a / 1b (UP) | - No data collection size limitation |  |
| Option 2 (CP) | - Data collection size >144 kBytes is not supported based on existing number of RRC segments - There is no NAS signalling segmentation - LMF supports LPP signalling segmentation | If LMF does not do segmentation for data collection or NAS signalling is used for data collection, either UP based solution is needed or RRC segmentation with extension of the number of RRC segments is required to support data collection larger than 144 kBytes |
| Option 3 (CP) | Data collection size > 144 kBytes is not supported based on existing number of RRC segments | UP based solution or extension of the number of RRC segments is required to support data collection larger than 144 kBytes |

From RAN1 LS regarding data collection requirements [2], typical data size is under discussion in RAN1 and it varies for different use cases. Maximum size can be e.g. 150 kbits (CSI compression), 1.5 Mbits (CSI prediction), 4096 * N bits (positioning, N is number of PRS/SRS resources and existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix). Both CSI compression and CSI prediction are study objectives with corresponding checkpoints in RAN#105 (September 2024). Therefore necessity of large data size support depends on final conclusions on supported use cases (e.g. whether to support CSI prediction).

Proposal 2: Whether data volume exceeds the NAS / RRC signalling capacity depends on final conclusions on supported used cases (e.g. whether to support CSI prediction).

It is nevertheless helpful to study how option 2 and 3 can realize UP tunnel if NAS / RRC signalling capacity becomes an issue. Currently, LPP protocol can be carried over either CP or UP tunnels. Similar UP tunnel option can be applicable to option 2, and data transfer path becomes UE ⇒ UPF ⇒ CN ⇒ Server for data collection for UE-side model training/OTT server. For Option 3, data transfer path for UP tunnel becomes UE ⇒ UPF ⇒ OAM ⇒ Server for data collection for UE-side model training/OTT server. The main difference between UP tunnels for option 2 and 3 is whether there is interface between UE and CN function or OAM. Given that there is already UP tunnel between UE and CN functions (e.g. LMF), it might be easier to have UP tunnel between UE and CN function for data collection compared with interface / UP tunnel between UE and OAM. Therefore from UP tunnel perspective, Option 2 is preferred over Option 3.

Proposal 3: If UP tunnel is needed (e.g. due to NAS / RRC signalling capacity issue), Option 2 is preferred over Option 3 since there are already UP tunnel between UE and CN functions.

Other FFS

For Option 3, SA2 involvement is marked as FFS. Since SA2 is handling overall architecture, it is helpful to involve SA2 for Option 3.

Proposal 4: List SA2 as one of involved WGs for Option 3.

Recommendation of options

In past RAN2 discussions, company views on the options for data collection are divergent. Taking into account pros and cons of the options as well as the interest of different stakeholders, we prefer Option 2 with partial visibility for partially standardized data content, with standardized fields for data visibility and container for UE/chipset proprietary information.

Proposal 5: For UE side data collection, it is recommended to select Option 2 with partial visibility for partially standardized data content, with standardized fields for data visibility and container for UE/chipset proprietary information.

Conclusion

In this contribution, we discuss data collection for UE side model training, and propose the following:

Proposal 1: In Option 2 and 3, partial visibility for partially standardized data content is supported, with standardized fields for data visibility and container for UE/chipset proprietary information.

Proposal 2: Whether data volume exceeds the NAS / RRC signalling capacity depends on final conclusions on supported used cases (e.g. whether to support CSI prediction).

Proposal 3: If UP tunnel is needed (e.g. due to NAS / RRC signalling capacity issue), Option 2 is preferred over Option 3 since there are already UP tunnel between UE and CN functions.

Proposal 4: List SA2 as one of involved WGs for Option 3.

Proposal 5: For UE side data collection, it is recommended to select Option 2 with partial visibility for partially standardized data content, with standardized fields for data visibility and container for UE/chipset proprietary information.

- References

[1] R2-2407209, Ericsson, "RAN2 inputs to TR 38.843"

[2] R2-2311720, RAN1 LS, "Reply LS on Data Collection Requirements and Assumptions"

- Annex Comparison Table from [1]

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |