3GPP TSG-RAN WG2 Meeting #127	R2-2406408

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.7.1.1

Source:	Xiaomi

Title:	Discussion on SA2 and RAN3 LSs for Rel-19 XR

Document for:		Discussion and Decision

Introduction

There are two LSs related to XR: SA2 LS on FS_XRM Ph2 (S2-2407351) and RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957).

In this contribution, we discus RAN2 replies for SA2 and RAN3 XR LSs.

Discussion

Discussion on SA2 LS on FS_XRM Ph2 (S2-2407351)

Regarding the following SA2 question:

Question1 [for SA4, RAN2 and RAN3]: SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

It is assumed that the question is for DL as SA2 Rel-19 discussion is focused on DL and the question mentions “i.e. in GTP-U”.

In Rel-18, CP based solution providing the periodicity is agreed for XR traffic by reusing TSCAI, as from TS 38.300 clause 16.15.2:

5GC may provide XR traffic assistance information to gNB through NG AP TSC Assistance Information (TSCAI) as specified in clause 5.37.8 of TS 23.501[3] (for both GBR and non-GBR QoS flows):

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

Related information is defined in NG-AP signalling, as in TS 38.413 clause 9.3.1.131.

Compared with CP-signalling, user plane signalling (in-band signalling) is more accurate and faster, and therefore can be more helpful for gNB scheduling e.g. to adjust configuration parameters. It should be noted that typically, periodicity is associated with burst arrival time and jitter information, as in the CP signalling defined in NG-AP as well as RRC signalling (as in UAI) for UL traffic information. Therefore, it might be also helpful to also provide burst arrival time and / or jitter information in-band.

Proposal 1: For Q1 of SA2 LS, RAN2 understands that the question is for DL, and indicating periodicity via in-band signalling is helpful for gNB resource scheduling. RAN2 also understands that typically periodicity is associated with burst arrival time and jitter information.

For the following SA2 question:

Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

Similar to above discussion, it is assumed that the question is for DL as SA2 Rel-19 discussion is focused on DL.

This question is related to previous RAN2 reply LS to SA2 (R2-2405779 / S2-2407415), with the RAN2 reply as below:

Regarding Question 4 on whether the size of incoming burst is useful for RAN resource scheduling, RAN2 understands this question is for DL, so only discussed the DL. RAN2 understands the size of incoming burst is useful for gNB resource scheduling if it can be provided early enough, e.g. in the first packet of the burst.

Along the same line, the time to next burst is also useful for RAN resource scheduling.

This question is also related to Q1 of SA2 LS, as just discussed above. Time to next burst is more dynamic compared with periodicity information since time to next burst needs to be provided burst by burst. It is not clear whether time to next burst can be estimated with very high accuracy or time to next burst is associated with jitter information.

Proposal 2: For Q2 of SA2 LS, RAN2 understands that the question is for DL, and the time to next burst is useful for gNB resource scheduling. It is not clear whether time to next burst can be estimated with very high accuracy or time to next burst is associated with jitter information.

Discussion on RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957)

Regarding the following RAN3 question:

In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured.

Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

In TS 38.321 clause 6.1.3.73 for PSI-Based SDU Discard Activation/Deactivation MAC CE, following is specified:

Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with discardTimerForLowImportance and with RLC entity(ies) associated with this MAC entity.

The above means that for a split bearer, UE follows the PSI-Based SDU Discard Activation/Deactivation MAC CE sent by MN and SN, i.e. UE follows the MAC CE whenever it receives it. For example, if MN sends MAC CE for activation, UE activates the PSI-Based SDU discard for the corresponding split bearer. If later SN sends MAC CE for deactivation, UE deactivates the PSI-Based SDU discard for the corresponding split bearer.

Proposal 3: For RAN3 LS, RAN2 replies that for a split bearer, UE follows the indication from PSI-Based SDU Discard Activation/Deactivation MAC CE whenever it receives the MAC CE from MN or SN.

Conclusion

In this contribution, we discus RAN2 replies for SA2 and RAN3 XR LSs, and propose the following:

Proposal 1: For Q1 of SA2 LS, RAN2 understands that the question is for DL, and indicating periodicity via in-band signalling is helpful for gNB resource scheduling. RAN2 also understands that typically periodicity is associated with burst arrival time and jitter information.

Proposal 2: For Q2 of SA2 LS, RAN2 understands that the question is for DL, and the time to next burst is useful for gNB resource scheduling. It is not clear whether time to next burst can be estimated with very high accuracy or time to next burst is associated with jitter information.

Proposal 3: For RAN3 LS, RAN2 replies that for a split bearer, UE follows the indication from PSI-Based SDU Discard Activation/Deactivation MAC CE whenever it receives the MAC CE from MN or SN.