3GPP TSG-RAN WG2 Meeting #127                              R2-24xxxxx

Maastricht, Netherlands, Aug 19th- 23rd, 2024

Source: 				ZTE Corporation

Title:            Restructure of RACH resource selection procedure in MAC

Agenda item:		7.0.2.8

Document for: 	Discussion and decision

- Introduction

In RAN2#125bis meeting, companies discussed the RACH resource set selection procedure in MAC spec based on following CRs. However, since the text procedure isn’t stable and it involves multiple WIs, so, the CRs were postponed.

R2-2402462	Correction on RACH resource set selection for CFRA	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1793	-	F	NR_redcap_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core	Revised

R2-2403642	Correction on RACH resource set selection for CFRA	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1793	1	F	NR_cov_enh2-Core, NR_MIMO_evo_DL_UL-Core, NR_redcap_enh-Core	R2-2402462

-	Ericsson doesn’t think we need to clarify.  LG is ok with the intention but considering that CFRA discussion is not stable we can postpone.  Vivo agrees with LG.  Huawei thinks that we have already postponed.

-	ZTE is concerned that the logic of the current CR is wrong. Rapporteurs can coordinate offline and find a better structure.

=>	The CR is postponed

Considering the MAC spec (v18.2.0) is stable after June, as the MAC CR rapporteur of Rel-18 Coverage enhancement topic, in this contribution, we analysis the problems in current MAC spec and provide the updated TP structures for further discussion.

- Discussion

In current MAC spec, the RACH resource set selection procedure is described as below, to facilitate the discussion, we marked different cases into different colours:

yellow refers to CFRA that triggered via MAC CE (i.e. LTM switch command);

green refers to CFRA that triggered via RRC reconfiguration with sync;

pink refers to CFRA that triggers via PDCCH order;

blue refers to Msg1 based SI request;

cyan refers to CFRA that triggered via MAC CE or RRC reconfiguration with sync or PDCCH order.

1> else if contention-free Random Access Resources have been provided for this Random Access procedure in the LTM Cell Switch Command MAC CE and a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE, and RedCap is applicable for the current Random Access procedure: 2> select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 1> else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure and Msg1 repetition number is indicated in rach-ConfigDedicated, and RedCap is applicable for the current Random Access procedure: 2> select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 1> else if contention-free Random Access Resources have been provided for this Random Access procedure in LTM Cell Switch Command MAC CE and a non-zero Msg1 repetition number is indicated in LTM Cell Switch Command MAC CE, and eRedCap is applicable for the current Random Access procedure: 2> select the set of Random Access resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 1> else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure and Msg1 repetition number is indicated in rach-ConfigDedicated, and eRedCap is applicable for the current Random Access procedure: 2> select the set of Random Access resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 1> else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication; or 1> if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with eRedCap indication; or 1> if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is no set of Random Access resources available that is only configured with eRedCap indication and there is one set of Random Access resources available that is only configured with RedCap indication: 2> select this set of Random Access resources for this Random Access procedure. 1> else: 2> if the Random Access procedure is initiated by PDCCH order with DCI PRACH association indicator field set to 1 and SSB-MTC-AdditionalPCI is configured by upper layers, as specified in clause 7.3.1.2.1 of TS 38.212 [9]: 3> select the set of Random Access resources corresponding to the additionalPCI associated with active TCI states. 2> else if the Random Access procedure is initiated by PDCCH order for an LTM candidate cell: 3> select the set of Random Access resources corresponding to the field Cell indicator in PDCCH order. 2> else if contention-free Random Access Resources have been provided for this Random Access procedure in the LTM Cell Switch Command MAC CE, and a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE: 3> select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 2> else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure, and Msg1 repetition number is indicated in rach-ConfigDedicated: 3> select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 2> else if the Random Access procedure was initiated for SI request and Random Access Resources associated with Msg1 repetition for SI request and Msg1 repetition number have been provided for this Random Access procedure: 3> if the BWP selected for Random Access procedure is indicated by initialUplinkBWP-RedCap: 4> if RedCap is applicable for the current Random Access procedure: 5> select the set of Random Access Resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 4> else if eRedCap is applicable for the current Random Access procedure: 5> if there is one set of Random Access resources available that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number: 6> select this set of Random Access resources for this Random Access procedure. 5> else: 6> select the set of Random Access Resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number. 3> else: 4> select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 2> else: 3> select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

As observed, the current text mainly has the following problems:

- Issue 1: Confusing structure, some CFRA use cases are described before “1> else:” but some are after “1> else:”. Originally, the text after “1> else:” is supposed to capture the handling of non-(e)RedCap UEs, but, by adding the texts for Rel-18 2TA, LTM and Msg1 based SI request, such splitting rule is gone. This makes the spec less readable and not easy for extension in future.

The structure of current text is illustrated in figure 1:

Figure 1 Structure of current text procedure

- Issue 2: Conflict processing, for (e)RedCap UEs, the cyan text and pink text conflicts. In detailed, the cyan text is supposed to cover all CFRA without Msg1 repetition for RedCap and eRedCap UEs, including CFRA that triggered by PDCCH order. But, for MIMO 2TA and LTM early RACH, the UE is expected to choose the pre-configured dedicated RACH resources that associated with the additional PCI or candidate PCI, according to the pink text. However, for (e)RedCap UE, once the network triggers PDCCH order for secondary TA acquisition (or LTM early RACH), the cyan text will be met first and the pink text will never be executed.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication; or

1>	if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with eRedCap indication; or

1>	if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is no set of Random Access resources available that is only configured with eRedCap indication and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	if the Random Access procedure is initiated by PDCCH order with DCI PRACH association indicator field set to 1 and SSB-MTC-AdditionalPCI is configured by upper layers, as specified in clause 7.3.1.2.1 of TS 38.212 [9]:

3>	select the set of Random Access resources corresponding to the additionalPCI associated with active TCI states.

2>	else if the Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

3>	select the set of Random Access resources corresponding to the field Cell indicator in PDCCH order.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Observation 1: In current TS 38.321, the text structure of RACH resource set selection is confusing and not easy for extension. In addition, for (e)RedCap UE, there is conflict processing for PDCCH order based CFRA.

To solve the issues, we need to reformulate the text, based on analysis, the current processing has the following characteristics:

- For Rel-18 2TA and LTM early RACH, the dedicated RACH resources are configured independently (e.g. via RACH-ConfigTwoTA-r18 or ltm-EarlyUL-SyncConfig-r18). In these two cases, the UE should select the dedicated RACH resources that associated with the indicated PCI, not from the common RACH resource pool;

- The handling of SI-request is also independent, because it is not CFRA and the it also relates to which BWP is selected;

- For all other CFRA cases, special treatment is only needed if the UE is (e)RedCap, or if Msg1 repetition is enabled, or both; otherwise, the UE just select the RACH resource set that not associated with any feature.

Based on above, the handling of 2TA and LTM early RACH should not be mixed up with other CFRA cases, the handling of SI-request can be kept separately. Also considering future extension, two options can be considered.

The text structure of Option 1 is illustrated in figure 2, in addition to SI-request, other cases are categorized into three levels: CFRA type-> UE type-> Use case.

Figure 2 Text structure of Option 1

The advantages of Option 1 are, once new UE type or new use cases are to be introduced in future, the level 2/3 can be extended easily, the structure also supports different handling for different CFRA types. But the disadvantage of Option 1 is that, the same processing text will be described multiple times in different scenarios. For example, for RedCap UE, for both “MAC CE with Msg1 repetition” and “ReconfigurationWithSync with Msg1 repetition”, the UE should select the RACH resource set that associated with both RedCap and Msg1 repetition indications, with Option 1, such text procedure needs to be described twice under different scenarios. So, the TP for Option 1 will be lengthy.

If we want to avoid duplicated texts, a simplified Option 2 is provided and illustrated in figure 3:

Figure 3 Text structure of Option 2

In Option 2, all the cases that have the same processing are categorized together, the handling of 2TA and LTM early RACH are listed separately and before other cases. In future, if new UE type is introduced, it can be added in parallel with RedCap/eRedCap, and new use cases can be considered case by case.

The TP for Option 1 and Option 2 are provided in [1][2]. RAN2 is kindly requested to discuss and select one of them.

Proposal 1: RAN2 to discuss the updated TP structure (Option 1 and Option 2) and select one TP from [1] and [2].

- Conclusion

In this contribution, proposals and observations are:

Observation 1: In current TS 38.321, the text structure of RACH resource set selection is confusing and not easy for extension. In addition, for (e)RedCap UE, there is conflict processing for PDCCH order based CFRA.

Proposal 1: RAN2 to discuss the updated TP structure (Option 1 and Option 2) and select one TP from [1] and [2].

- Reference

[1] R2-2406413 Correction on RACH resource set selection procedure---Option 1  ZTE Corporation 	CR TS 38.321;

[2] R2-2406414	Correction on RACH resource set selection procedure---Option 2  ZTE Corporation	 CR TS 38.321.